3GPP TSG-SA WG2 Meeting #129BIS	S2-1812709

# West Palm Beach, USA, 26-30 November 2018	(merge of S2-1812005 and S2-1811729)

Source:	Orange, Huawei, Convida Wireless, AT&T

Title:	Key Issue 1&2 evaluation

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution proposes an analysis to clarify how the use cases could apply the analytic services provided by the solutions in current TR 23.791. It proposes also the evaluation for Key Issues 1&2.

# Discussion

Obervation#1: For analytic services provided by the solutions in current TR 23.791, there are two types of solutions:

- Generic solutions, which apply to multiple use cases: Solutions 1 and 9.

- Solutions specific to use cases: Solutions #3, #4, #5, #7, #8, #10, #11, #18, #20, #21, #22, #23, #27, #29. These solutions might potentially reuse (refer to) the generic solutions.

Observation#2: the Solution #9 is different from Solution #1, and goes slightly beyond the scope of the TR.

Proposal#1: Among the generic solutions, 3 steps may be considered to clarify the different objectives of analytic services:

- P: Solution #1 with analytics related to the past (statistics)

- F: Solution #1 with analytics related to the future (predictions)

- R: Solution #9 with recommendations for the value of parameters (future)

These three steps appear as progressive levels of intelligence in the NWDAF.

- Statistics in the past (P) may be considered simply as a report, but also as a “vanilla” estimation of the future in general, with the underlying paradigm that “future” shall be very similar to “past” (therefore no need to refine the prediction)

  - Example: statistics in the last 2 months show that this area has average normal traffic on Fridays

- Predictions (F) are a step beyond the previous, because trend analysis may determine evolving drivers for the future. But in this step, the major part of the decision-making process remains in the consumer NF.

  - Example: a prediction shows that this Friday is a special day (social event for certain communities, depending on Moon calendar and therefore difficult to predict by basic statistics), therefore a much greater traffic is expected

- The next step is the “recommendation” (R), where the NF delegates some calculation of optimal parameters to the NWDAF, but still remains responsible for the final choices.

    - Example: fine tuning of QoS parameter of certain DNNs is recommended, because traffic will mainly occur there but not on all DNNs.

Depending on the use cases, step P, or F, or R may be required from start. 
But when the level P (or F) is required, sometimes the next levels F (respectively: or R) could be possible.

Proposal#2: an analysis is proposed to clarify how the use cases could apply the solutions.

# Proposal

It is proposed to modify the following clause in TR 23.791.

**** Start of change (all new text) ****

## 7.X	Evaluation of Solutions for Analytic Information Exposure

For analytic services, two types of solutions are described in the TR 23.791:

-	Generic solutions, which apply to multiple use cases: Solutions #1 and #9.

-	Solutions specific to use cases: Solution #3, #4, #5, #7, #8, #10, #11, #18, #20, #21, #22, #23, #27, #29. These solutions might potentially reuse (refer to) the generic solutions.

The services provided by the generic solutions can be further classified into three levels of analytics:

-	Statistics of the past, which may be considered simply as a report, but also as a "vanilla" estimation of the future in general, with the underlying paradigm that "future" shall be very similar to "past" (therefore no need to refine the prediction).

-	Predictions, which are a step beyond the previous, because trend analysis may determine evolving drivers for the future which cannot be detected by basic statistics. But in this step, a major part of the decision-making process remains in the consumer NF.

-	Recommendations, where the NF delegates some calculation of optimal parameters to the NWDAF, but still remains responsible for the final choices.

Solution #1 provides statistics and predictions, while Solution #9 provides recommendations.

Solution #9 (Recommendations produced by NWDAF) is a solution that has been evaluated to possibly only cover a limited number of key issues/use cases, which are a subset of what can be solved by analytics. It is not recommended for Release 16, because it requires additional study. It is desirable that this Solution be reconsidered following the results obtained on the implementations of Release 16, in order to examine its relevancy on stronger use case basis.

An analysis of how the use cases are covered by the solutions, according to the above classification, is provided in Annex X.

**** Next change (all new text) ****

# Annex X: Analysis of the analytic services provided by the solutions relevant to each use case

The following table shows, for each use case, when applicable, the level of analytics that can be used from the generic solutions, as well as the specific Solution # applicable.

Table X-1: Analytic services provided by the solutions relevant to each use case

| Use Case | Services provided by Solution #1 | Specific solution applicable |
| --- | --- | --- |
| #1: how to get information from AF | N/A | N/A |
| #2: NWDA-Assisted QoS Provisioning | Statistics, predictions | #3: QoS Profile Provisioning |
| #3: NWDA-Assisted Traffic Handling | Statistics | #11: UPF selection based on output from NWDAF, #20: NWDAF analytics usage for UPF selection |
| #4: Using NWDA output to customize mobility management | Predictions | #4: Optimizing registration area management based on NWDAF output, #5: Optimization based on paging failure prediction from NWDAF, #7: Customizing mobility management based on NWDAF output, #10: NWDAF assisted Handover decision, #23: AMF using NWDAF outputs to optimize UE mobility procedures, #29: NWDAF assisted MICO mode for mIoT terminals |
| #5: NWDA-assisted Determination of Policy | Statistics | #18: NWDAF assisting Future Background Data Transfer (BDT), #27: NWDAF assisting Determination of Background Data Transfer Policy #28: Exposure to AF for Background Data Transfer |
| #6: NWDAF-Assisted QoS Adjustment | Statistics |  |
| #7: NWDAF assisting 5G edge computing | Predictions | #11: UPF selection based on output from NWDAF |
| #8: Performance improvement and supervision of mIoT terminals | Statistics | #8: Performance improvement and supervision of mIoT terminals #21: Supervision of IoT devices for risk precaution |
| #9: NWDAF-assisted load balancing/re-balancing of network functions | Predictions |  |
| #10: NWDA-assisted determination of areas with oscillation of network conditions | Statistics |  |
| #11: Prevention of various security attacks | Statistics | #8: Performance improvement and supervision of mIoT terminals |
| #12: NWDA-Assisted predictable network performance | Predictions |  |
| #13: UE driven analytics sharing | Statistics, predictions |  |
| #14: How to ensure that slice SLA is guaranteed | Statistics, predictions |  |

**** End of Changes ****