SA WG2 Meeting 129#BIS	S2-1812906

26 - 30 November 2018, West Palm Beach, Florida	(revision of S2-1812089)

Source:	Huawei, HiSilicon

Title:	Solution for Key Issue #6: Division of E2E PDB

Document for:	Approval

Agenda Item:	6.20

Work Item / Release:	FS_5G_URLLC/Rel-16

Abstract of the contribution: This paper intends to add a new solution for the division of E2E PDB.

# Discussion

This document proposes a new solution to address the Key Issue 6: Division of  E2E PDB. Solutions for the division of E2E PDB during the PDU Session Establishment /Modification procedures and Service Request procedure are proposed for the key issue.

# Proposal

It is proposed to update TR 23.725 as follows:

*******START OF CHANGE (all new)*******

## 6.X	Solution for Key Issue #6: Division of E2E PDB

### 6.X.1	Description

This solution applies to key Issue #6. In the solution, besides of current PDB (i.e. PDB between UE and PSA UPF), the SMF will provide the RAN with the PDB between the PSA UPF and the RAN (i.e. PSA-RAN  PDB), so that the RAN nodes can take it into account when derivingAN PDB.

The SMF is locally configured by the OAM with the packet delay budgets between a specific RAN node and an PSA UPF (i.e. PSA-RAN PSA-RAN  PDB) for different 5QIs. The SMF can provide the RAN with the PSA-RAN PSA-RAN  PDB for a QoS Flow during the PDU Session Establishment procedure, the PDU Session Modification procedure or the Service Request procedure.

### 6.X.2	Procedures

#### 6.X.2.1	PDB provisioning during PDU Session Establishment /Modification or Service Request procedure

Figure 6.X.2.1-1 depicts the enhanced PDB provision for a URLLC QoS Flow during PDU Session Establishment /Modification procedure and Service Request procedure.

Figure 6.X.2.1-1: Enhanced PDB provisioning for a URLLC QoS Flow

During UE triggered Service Request procedure, steps 1-22b from clause 4.2.3.2 in TS 23.502 [3] are performed with the following differences:

- In steps 11-12, the PSA-RAN PSA-RAN  PDB together with PDB for each QoS Flow may be included in the N2 SM information and provided to the RAN node as depicted in step 2a and step 3 in Figure 6.X.2.1-1.

During Network triggered Service Request procedure, steps 1-7 from clause 4.2.3.3 in TS 23.502 [3] are performed with the following differences:

- In step 3a, the PSA-RAN  PDB together with PDB for each QoS Flow may be included in the N2 SM information and sent to the AMF via Namf_Communication_N1N2MessageTransfer request as depicted in step 2b in Figure 6.X.2.1-1.

- In step 4a, when the steps 12 to 22 in UE Triggered Service Request procedure in clause 4.2.3.2 in TS 23.502 [3] are invoked, the PSA-RAN  PDB together with PDB for each QoS Flow may be included in the N2 SM information and sent to the RAN node via N2 Session Request as depicted in step 3 in Figure 6.X.2.1-1.

- In step 6, when the steps in UE Triggered Service Request procedure in clause 4.2.3.2 in TS 23.502 [3] are invoked, the PSA-RAN  PDB together with PDB for each QoS Flow may be included in the N2 SM information and provided to the RAN node as depicted in step 2a and step 3 in Figure 6.X.2.1-1.

During UE requested PDU Session Establishment procedure, steps 1-20 from clause 4.3.2.2.1 in TS 23.502 [3] are performed with the following differences:

- In steps 11-12, the PSA-RAN PSA-RAN  PDB together with PDB for each QoS Flow may be included in the N2 SM information and provided to the RAN node as depicted in step 2b and step 3 in Figure 6.X.2.1-1.

During UE or network requested PDU Session Modification procedure, steps 1-13 from clause 4.3.3.2 in TS 23.502 [3] are performed with the following differences:

- In step 3a, 3b and step 4, the PSA-RAN PSA-RAN  PDB together with PDB for each QoS Flow may be included in the N2 SM information and provided to the RAN node as depicted in step 2a, 2b and step 3 in Figure 6.X.2.1-1.

After RAN receive PSA-RAN PSA-RAN  PDB and PDB for the QoS Flow, RAN can derive PDB for RAN part of this QoS Flow accordingly.

Note 1: If standardized 5QI is used for the QoS Flow, the PDB in the N2 information is omitted, but the PSA-RAN PDB may still be included.

Note 2: The PSA-RAN PSA-RAN  PDB is only apply to QoS Flows of Delay Critical GBR resource type.

### 6.X.3	Impacts on Existing Nodes and Functionality

SMF:

-	The SMF provides PSA-RAN PSA-RAN  PDB for a QoS Flow to the NG-RAN node during PDU session establishment/modification or service request procedures.

RAN: Derive the PDB of RAN part based on the received PDB and PSA-RAN PSA-RAN  PDB.

### 6.X.4	Solution Evaluation

With this solution, RAN node can obtain PSA-RAN  PDB for a specific QoS Flow, so that RAN can derive PDB for RAN part by taking it into account. PSA-RAN  PDB is an optional parameter, so the solution is compatible with existing Rel-15 PDB mechanism.

*******END OF CHANGE*******