SA WG2 Meeting #129	S2-1810167

15 - 19 October 2018, Dongguan, China	(revision of S2-18XXXX)

Source:	Sony, Deutsche Telekom

Title:	General Evaluation

Document for:	Approval

Agenda Item:	6.28

Work Item / Release:	FS_RACS / Rel-16

Abstract of the contribution: This contribution proposes text for the general evaluation section.

# 1 Discussion

This contribution provides some overall evaluation statements for several solutions.

# 2 Proposal

This document proposes the following addition into TR 23.743 (all new).

* * * Start of Change 1 (all new) * * *

# 7	Evaluation

Evaluation criteria X1

Some of the solutions are based on that an UE reports its capability ID and corresponding capabilities and then the network re-uses that capability ID and corresponding capabilities to other UEs in the network that reports the same Capability ID. To allow the network to re-use capabilities uploaded from one UE and re-use it to other UEs the solutions must ensure that the network is protected against that a malicious UE does not update the network with incorrect capabilities. One evaluation criteria should be:

Does the solution support that the network is protected against malicious updates of UE Capability ID’s and corresponding UE Radio Capabilities?

Evaluation criteria X2

Compared to current functionality in LTE and EPC where it is possible to fulfill capability signaling within the operator’s domain so the operator can ensure reliability and to support sufficient short response times to capture the actual capabilities. For example, how does an operator handle an UE from a manufacturer that has gone out of business and does not provide any capability server any more. The evaluation criteria are:

For an UE with a capability ID that is unknown within the network, is the network independent of resources external to the operator or the UE?

| Solution | X1, Does the solution support that the network is protected against malicious updates of UE Capability ID’s and corresponding UE Radio Capabilities? | X2, For an UE with a capability ID that is unknown within the network, is the network independent of resources external to the operator or the UE |
| --- | --- | --- |
| 1 | No, if combined with any solution that allows capture actual capabilities from the UE | No, if combined with any solution that allows capture actual capabilities from external servers (vendors servers) |
| 2 | No, except in combination with solution 3 | Yes |
| 3 | Yes | Yes |
| 4 | N/A, the capabilities does not come from the UE | No |
| 5 | N/A, the capability ID is assigned by the network | Yes |
| 6 | No, except in combination with solution 3 | No, except in combination with solution 3 |
| 7 | No, except in combination with solution 3 | Yes |

* * * Next Change (all new) * * *

# 8	Conclusions

Solutions 1 and 4 can’t fulfil evaluation criteria X1 and X2 and therefore these should not be considered for the normative phase.

Solutions 2, 6 and 7 without solution 3 can’t fulfil evaluation criteria X1 and X2. Unless these solutions are not combined with solution 3 then they should not be considered into normative phase.

* * * End of changes * * *