3GPP TSG-SA WG2 Meeting #129	S2-1810282

Dongguan, China, 15 – 19 October, 2018	(revision of S2-18xxxxx)

Source:	LG Electronics

Title:	Additional IP address allocation for MPTCP

Document for:	Approval

Agenda Item:	6.8 Study on Access Traffic Steering, Switch and Splitting support in the 5G system architecture

Work Item / Release:	FS_ATSSS / Rel-16

Abstract of the contribution: This contribution proposes that additional IP address for MPTCP subflow is allocated to the MPTCP proxy.

Discussion

In the conclusion, there is a following Editor's Note.

Editor's note:	Whether the two IP addresses are allocated to UE or to the MPTCP proxy is FFS.

This contribution tries to resolve above Editor's Note.

According to clause 3.4 of RFC 6824 (TCP Extensions for Multipath Operation with Multiple Addresses), MPTCP supports Address Knowledge Exchange. By using Address Advertisement, each host can exchange additional IP addresses for MPTCP subflows.

This mechanism allows IP addresses to change in flight, and also allows the signalling of addresses belonging to other address families (e.g., both IPv4 and IPv6). In addition, the SMF does not need to provide additional IP address to the UE via NAS signalling or DHCP so there is no impact to the NAS protocol.

Based on above observation, we propose that the IP address for MPTCP is allocated only to the MPTCP proxy. The UE and MPTCP proxy can exchange the IP address information via Address Knowledge Exchange mechanism of MPTCP.

Proposal

* * * * Start of 1st Change * * * *

7	Conclusions

The ATSSS solution in Rel-16 shall be based on the following principles:

Editor's note:	These principles below do not specify a complete ATSSS solution for Rel-16, but they provide the guidelines, which the complete solution should be based on. It is FFS how these principles can be amended or modified.

Support of MA-PDU sessions

1.	The solution shall support ATSSS with a Multi-Access PDU (MA-PDU) session, i.e. ATSSS procedures shall be applied after a MA-PDU session is established.

Editor's note:	If and how ATSSS can be applied without a MA-PDU session is FFS.

2.	A MA-PDU session is established with the Separate Establishment procedure, as specified in clause 6.2.3.1, or, with the Combined Establishment procedure, as specified in clause 6.2.3.2.

Editor's note:	It is FFS whether only one establishment procedure (Separate or Combined) or both will be supported.

3.	When the UE sends a NAS message to request a single-access PDU session and the UE supports MA-PDU sessions, the UE includes an "MA-PDU capability" indication in the NAS message. This indication may be used by the network to establish a multi-access PDU session, instead of the requested single-access PDU session, as specified in clause 6.2.4.

Policy for Multi-Access QoS Control

Editor's note:	The PCC rules created by PCF for QoS control in a MA-PDU session are FFS.

Policy for ATSSS Control

4.	During the establishment of a MA-PDU session, the PCF may create PCC rules for the MA-PDU session. These rules specify how specific service data flows (SDFs) should be routed across the 3GPP and non-3GPP accesses.

Editor's note:	The PCC rules created by PCF for ATSSS control in a MA-PDU session are FFS.

5.	The SMF maps the PCC rules into (a) ATSSS rules which are sent to UE via the AMF, and (b) Packet Detection Rules which are sent to UPF. The ATSSS rules are used by the UE for uplink traffic steering and the Packet Detection Rules are used by the UPF for downlink traffic steering.

6.	The ATSSS rules are sent to UE with a NAS message when the MA-PDU session is created or when they are updated by SMF/PCF. Similarly, the Packet Detection Rules are sent to UPF when the MA-PDU session is created or when they are updated by SMF/PCF.

7.	An ATSSS rule includes the following:

a)	A Precedence value, which identifies the priority of this ATSSS rule with respect to other ATSSS rules.

b)	A Traffic Descriptor, which identifies a service data flow (SDF). It may include e.g. an Application ID, IP descriptors, non-IP descriptors, etc.

c)	A Steering Mode, which identifies how the matching SDF should be steered across 3GPP and non-3GPP accesses. The following Steering Modes will be supported:

-	Active-Standby: It is used to steer a SDF on one access (the Active access), when this access is available, and to switch the SDF to the other access (the Standby access), when Active access becomes unavailable. When the Active access becomes available again, the SDF is switched back to this access. If the Standby access is not defined, then the SDF is only allowed on the Active access and cannot be transferred on another access.

-	Smallest Delay: It is used to steer a SDF to the access that is determined to have the smallest Round-Trip Time (RTT). As defined below, measurements may be conducted to determine the RTT over 3GPP access and over non-3GPP access.

-	Load-Balancing: It is used to split a SDF across both accesses. With a 50/50 load-balancing, the SDF traffic is equally split across the two accesses. With an 80/20 load-balancing, about 80% of the SDF traffic is sent on one access and 20% on the other access.

d)	A Steering Function, which identifies whether the MPTCP or the ATSSS function shown in Fig. 7-1 should be used to steer the traffic of the matching SDF. This is useful in case the UE supports multiple functions for traffic steering.

Editor's note:	It is FFS (a) if additional steering modes are needed, and (b) if the structure of the ATSSS rule needs to be modified. The details of the Steering Function are also FFS.

8.	As an example, the following ATSSS rules could be provided to UE:

a)	"Traffic Descriptor: UDP, DestAddr 1.2.3.4", "Steering Mode: Active-Standby, Active=3GPP, Standby=non-3GPP"

-	This means "steer UDP traffic with destination IP address 1.2.3.4 to the active access (3GPP), if available. If the active access is not available, use the standby access (non-3GPP)".

b)	"Traffic Descriptor: TCP, DestPort 8080", "Steering Mode: Smallest Delay"

-	This means "steer TCP traffic with destination port 8080 to the access with the smallest delay". The UE needs to occasionally measure the RTT over both accesses, in order to determine which access has the smallest delay.

c)	"Traffic Descriptor: Application-1", "Steering Mode: Load-Balancing, 3GPP=20%, non-3GPP=80%", "Steering Function: MPTCP"

-	This means "send 20% of the traffic of Application-1 to 3GPP access and 80% to non-3GPP access by using MPTCP".

Support of Measurements

9.	It shall be possible for the UE and the network to measure the RTT over both accesses. Such measurements may be required only under certain conditions, e.g. only when the UE has a valid ATSSS rule using a "Smallest Delay" steering mode.

10.	Measurements between the UE and the network take place over the user-plane.

Editor's note:	It is FFS if the network can provide measurement policy to UE to assist the UE in taking measurements. It is also FFS if additional measurements (other than RTT) are needed.

Support of MPTCP

11.	The solution will support MPTCP as follows:

a)	During the MA-PDU session establishment, if the UE wants to use MPTCP for traffic steering, the UE provides an "MPTCP Request" indication.

b)	If the network agrees to enable MPTCP for the MA-PDU session then:

i)	The network allocates two IP addresses for the MA-PDU session. This is required, otherwise MPTCP cannot be used. As specified in RFC 6824, "... there must be multiple addresses at least at one endpoint, for MPTCP to be used".

The additional IP address for MPTCP subflow is allocated to the MPTCP proxy. The UE and MPTCP proxy can exchange the additional IP address information via Address Knowledge Exchange mechanism as described in RFC 6824.

ii)	The network may send MPTCP proxy information to UE, e.g. the IP address(es), port and type (SOCKS5 [9] or TCP Convert Protocol, TS 38.215 [11]) of the MPTCP proxy.

iii)	The network may indicate to UE the list of applications for which MPTCP should be applied.

Editor's note:	It is FFS how the above list of applications can be provided to the UE.   It is also FFS whether the MPTCP proxy information must be provided to UE, and/or whether a transparent MPTCP proxy can be used.

12. Within the same MA-PDU session, if MPTCP is enabled, it is possible to steer the MPTCP flows by using the MPTCP protocol (the MPTCP function) and, simultaneously, to steer all other flows by using a lower-layer steering functionality, called the "ATSSS function". This is schematically illustrated in Fig. 7-1, which shows an example UE model with an MPTCP function and an ATSSS function. The MPTCP flows is the traffic of the applications for which MPTCP can be applied (see bullet 11.b.iii). Note that in Fig. 7-1, the MPTCP traffic goes through the MA-PDU session but is not handled by the ATSSS function.

13.	The same set of ATSSS rules is applied for steering decisions by the MPTCP function and by the ATSSS function.

Figure 7-1: An example of a UE supporting the MPTCP function and by the ATSSS function

* * * * End of Changes * * * *