SA WG2 Meeting #129	S2-1811293

15 – 19 October, Dongguan, China	(revision of S2-18xxxx)

Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO, Orange, Verizon, Oracle, Ericsson

Title:	Interim Conclusion proposal for KI#4

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution proposes interim conclusion for key issue 4.

# 1 Discussion

It is proposed to service set for 5GC Reliability as it has the following benefits.

- Set Concept - Allows multiple NF/NF service instance sets per network for deployment flexibility

- State efficiency - Service instances may keep state during a signaling flow (= temporary binding) if required, controlled by producer instance. Fully stateless otherwise. Temporary binding can be setup and released. Thus, the NFs receiving frequent UE transactions are state efficient considering performance efficiency.

- Performance and efficiency - Storage layer data retrieval once per transaction / procedure (signaling flow).

- Storage layer - Segmented storage layer kept near NF/NF service set for better scalability and performance

- Service Instance compatibility in terms of functionality/features (Network Slice) supported - Assumes all service instances in a set to be equivalent and interchangeable

- SMF-UPF relationship - Allows regional or other grouping of SMF and UPF instances

- Race conditions are limited to the instances within the SET, (the given DSF and the given SET).

  - Set can comprise of 1 to N instance, where N can be a huge number (even comprising of all instances deployed in the network as part of the set).

  - It adds additional flexibility and optimizations for better performance and efficiency. Allows support for both state-ful (for signaling and perfomance efficiency in the middle of a procedure) and stateless (for better scalability and improved resiliency).

Otherwise, the consequence will be as follows:

- Set Concept - Equivalent to having a single set per network (deployment option: one set)

- State efficiency - Requires all service instances to be fully stateless beyond a single request/response

- Performance and efficiency - Storage layer data retrieval for each request received/processed.

- Storage layer - Segmented storage layer kept near NF/NF service set for better scalability and performance

- Service Instance compatibility in terms of functionality/features (Network Slice) supported - Assumes all service instances in a network to be equivalent and interchangeable

- SMF-UPF relationship - Requires full mesh of SMF and UPF within a network

- Race conditions are Spread across the network- could result in significant propagation delay when it is across DSFs and NF instances in a huge nationwide network.

# 2 Proposal

It is proposed to introduce the following updates to TR 23.742:

* * * Start of Change 1 (all new text) * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.x	Interim conclusions for 5GC Reliability

Following are the principles for 5GC Reliability:

- Introduce the concept of NF/NF Service/Service instance Set for 5GC and the Set is identified by SET ID.

- The NF/NF Service instances within a given NF/NF Service/Service instance Set are expected to have access to the same data sets in a data storage entity e.g. UDSF. Thus, in principle, any NF/NF Service/Service Instance within a Service Instance set should be able to process UE transactions as it has access to UE context. Following are the key principles for NF/NF Service/Service Instance Sets:

-	A Set of instances of the same service type.

-	A Set of instances supporting same network slices.

-	All NF/NF Service/Service instances in a Set can access the same data storage e.g. UDSF.

Editor’s note: depending on the architecture adopted in rel-16, whether it is NF/NF Service/Service, the Set concept needs to be adapted to the actual choice FFS.

- Multi-vendor interoperability is achieved across SETs. Here is an example:

  - NF/NF Service SET1 of NF instances optimized for IoT is from vendor 1.

  - NF/NF Service SET2 of NF instances optimized for IoT is from vendor 2.

  - NF/NF Service SET1 of NF instances optimized for eMBB is from vendor 1.

  - NF/NF Service SET2 of NF instances optimized for eMBB is from vendor 2.

- When a NF/NF Service/Service Instance Set exposes multiple NF/NF Service/service instances towards a consumer, the consumer is allowed to reselect a different NF/NF Service/Service Instance (within the same set) between transactions. Race conditions with multiple requests for the same UE is up to implementation to resolve, potentially using mechanisms like redirect between NF/NF Service/Service Instances in the Set.

- There may be additional restrictions for creation of assigning NFs/NF Services/Services to a set:

  - Sets of NFs/NF services/Services (i.e. SMF) managing UPFs need to be connected to the same set of UPFs

NOTE 1:	The UPFs are not grouped as SETs.

  - The enhancements related to NF/NF Service/Service Set shall also be supported for the protocol(s) supported over N4.

NOTE 2: The Set within an operator network is identified by SET ID and the actual mapping of instances to a given SET is up to deployment.

* * * End of Change 1 * * *