SA WG2 Meeting #129	S2-1811404

Dongguan, China, 15-19 October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0656 | rev | 2 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network controlled NSSAI for SR-related Access Stratum connection establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, NEC, Telecom Italia, Huawei, InterDigital, AT&T, Verizon, Sprint, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-10-9 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed in tdoc S2-1810390 we need to conclude on the topic of what NSSAI value to include in the RRC connection establishment when this is triggered by a SR (whithout this, the Specification is incomplete). This CR proposes a way forward in alignment with S2-1810390 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aa PLMN can instruct inform the UE on which behaviour to adopt when triggering a Service Request procedure, in a Registration Accept Message by including Access Stratum Connection Establishment NSSAI Inclusion Mode information in a Registration Accept Message that instructs the UE to: 1.- Include in Access Sstratum Connection Establishment a NSSAI = Aallowed NSSAI for the UE, or 2. Include in Access Stratum Connection Establishment only the S-NSSAIs of the Network Slices that trigger the RRC Connection Establishment, or 3. Include no NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undefined UE behaviour |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | If SA3 deems that it is a security issue to send S-NSSAI in cleartext in AS signaling and to send encrypted S-NSSAI in the NAS signaling, then it is an option to remove S-NSSAI from NAS signaling, instead RAN can send S-NSSAI in the N2 signaling towards AMF. |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

### 5.15.x	Operator-controlled inclusion of NSSAI in Access Stratum Connection Establishment

The operator controls the NSSAI the UE includes in the Access Stratum when establishing a connection to send Service Request or a Registration Request not caused by Mobility NAS messages.

During the Registration procedure, the AMF may provide the UE in the Registration Accept message an Access Stratum Connection Establishment NSSAI Inclusion Mode parameter indicating whether and when the UE shall include NSSAI information in the Access Stratum Connection Establishment (e.g. a RRC connection Establishment defined in TS 38.331) according to one of these modes:

a)	The UE includes a NSSAI set to the Allowed NSSAI for the UE for both Registration Request not due to mobility and Service request triggered Connection Establishment

b)	The UE includes a NSSAI with the following content:

-	for the case of Access Stratum Connection Establishment caused by a Service Request: a NSSAI including the S-NSSAI(s) of the Network Slice(s) that trigger the Connection Establishment

-	for the Registration Request not due to mobility, a NSSAI set to the Allowed NSSAI; or

c)	The UE includes no NSSAI for both Registration Request not due to mobility and Service Request triggered Connection Establishment.

The UE shall by default operate in mode c) unless it has been provided with a different indication from the AMF.

A PLMN by default only allows sending NSSAI in the Access stratum with confidentiality protection in the Access Stratum Connection establishment.  To allow sending NSSAI without confidentiality protection in the Access Stratum Connection establishment, the AMF may provide to the UE in the Registration Accept a NSSAI AS Confidentiality Protection Not Required indicator. This indication is provided if both information received from the UDM as part of the UE subscription information and local policy in the Serving PLMN UE to send NSSAI in the access stratum when no AS protection is available. The default setting of the UDM subscription information is to not allow the UE to send NSSAI in the access stratum without confidentiality protection.

When the UE receives for a PLMN the Confidentiality Protection Not Required indicator, the reception of this message may be stored for the PLMN alongside any other slicing configuration information for the PLMN. If there is no storage of reception of such indicator for the PLMN, the UE can send NSSAI only if it is confidentiality protected.

If the UE is not registered with a PLMN, it shall never send NSSAI in the Access Stratum without confidentiality protection when it executes the first Registration with this PLMN, unless for this PLMN it stores it has previously received a AS Confidentiality Protection Not Required Indicator.

NOTE: 	In this release of the specifications there is no support for confidentiality protecting the NSSAI over 3GPP access when it is sent in RRC connection Establishment.

**** END OF CHANGES ****