SA WG2 Meeting #129	S2-1811597

October 15 - 19, 2018, Dongguan, P. R. China	(Revision of S2- 1811512)

Source:	Sprint, AT&T, Deutsche Telekom, Vodafone, China Mobile, Bell, Telstra, SK Telecom, KDDI, Ericsson, Verizon

Title:	Conclusions for key issue 4 – Stateless NF Services

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution proposes a set of conclusions for key issue 4 of the Study on Enhancements to the Service-Based 5G System Architecture, TR 23.742.

# 1 Introduction

This document proposes a set of conclusions for key issue 4.

# 2 Proposal

This document proposes a set of conclusions for key issue 4. It addresses only the aspects rendering NF Services as stateless, i.e., separating functional processing from state repository. The conclusions gather together principles to agree upon that already exist in the TR as a way to construct a solution from drawing upon portions of several proposed solutions.

* * * Start of Changes * * *

# 8	Conclusions

For Key issue#4 solution(s) shall be developed and evaluated based on to the following principles:

1.	Specify service based procedures for service context transfer from one NF/NF service instance to another NF/NF service instance from a different vendor. This is for services in all major NFs (such as SMF, PCF, etc.).

a.	As part of the procedure  the context information to be transferred between instances shall be standardized. E.g. such standard context is already available on Namf and Nsmf.

2.	This procedures shall allow some service procedures to be handled by one service instance and the following procedures of the same service to be handled by the service instance from a different vendor.

Editor’s note:	Whether all service procedures can be seamlessly continued after it is transferred is FFS.

3.	It shall be possible to transfer service context between different vendor’s instances without contacting the UE.

NOTE 1:	After resumption of a new service transaction, it may be necessary to contact the UE using existing procedures.

NOTE 2:	Some features of an NF service instance may not continue after the inter-vendor transfer to a new service instance.

4.	A list of triggers for context transfer shall be defined. This includes triggers from operator’s OA&M system (e.g. load level).

* * * End of Changes * * *