SA WG2 Meeting #130	S2-1900058

21 – 25 January, Kochi, India	(revision of S2-19xxxx)

Source:	Vodafone, Verizon, Motorola Mobility, Lenovo

Title:	FS_UDICOM - Conclusions for KI#1

Document for:	Approval

Agenda Item:	6.29

Work Item / Release:	FS_UDICOM / Rel-16

Abstract of the contribution: This pCR proposes conclusion for FS_UDICOM KI#1

# 1 Proposal

As closure of UDICOM study approaches, evaluations and conclusions are deemed necessary. The proposal is to include conclusions for the study.

******************First change***********************

# 8	Conclusions

## 8.x Conclusion for Key Issue #1: Interaction between UDM and HSS with separate repositories

### 8.x.1 For EPS and 5GS interworking required for mobility cases from EPS to 5GS and from 5GS to EPS, including the impacts on SMS handling

Solution #2 is considered to progress as normative optional feature. A number of open issues listed in clause 7 need to be addressed during normative phase.

For cases in which operator deployments require an alternative from solution #2, e.g. to introduce only a UDM/UDR for 5G, it is recommended to capture indications in the normative specifications to allow implementation options as follows:

-	Solution #1 for scenarios of dual registration mode, when application front ends need to communicate and/or retrieve information from different repositories, e.g. HSS from 5Gs UDR or UDM from EPS UDR. Recommendation is to allow an implementation option via a translation function, between HSS and UDM.

-	Solution #3 can be considered to be an implementation option when 5G subscription data and 4G subscription data must be split in two different repositories. It is recommended to allow combo UDM/HSS with 5G UDR as implementation option.

-	Solution #5 for scenarios of single registration with N26 support. Recommendation is to allow an option for implementation via an intermediate/adaptation layer between front ends and repositories.

These conclusions require a new stage 2 spec to clarify the interactions between HSS and UDM. CT4 is expected to take responsibility over the new spec while SA2 will add relevant references in affected 5G specifications (TS 23.501, TS 23.502).

### 8.x.2 For IMS and 5GS interworking required for T-ADS, P-CSCF restoration and NPLI retrieval

Three different scenarios are addressed by the proposed solutions:

- HSS as single point of contact for IMS (diameter based)

- UDM as single point of contact for IMS (diameter based)

- IMS HSS/SLF as single point of contact for IMS (diameter based)

Any solution for IMS interactions needs to serve the plethora of different deployments and not preclude any conclusion taken by the ongoing study for evolving IMS to SBA (see ref [14]).

To facilitate this, several items are concluded:

1.-	Progress normative work for a new service with new service Request-Response operation(s) produced by UDM: for T-ADS (i.e. IMS over PS support) and NPLI (i.e. time stamp and RAT type), and for P-CSCF restoration. The decision about the number of operations is left for normative work.

The operations can be consumed by HSS (as in solution #2) for scenarios in which HSS is the single point of contact for IMS (diameter based) interactions and adapted to SBA.

The operations can be consumed by an IMS HSS when in stand-alone mode and acting as a single point of contact for a diameter-based IMS. This case would correspond to solution #2 when the HSS referred acts as IMS HSS and is adapted to SBA.

NOTE 1: 	These new operations are considered to be handled as part of UDICOM normative work, but coordination is required to avoid disparate solutions with FS_eIMS5G [14].

2.-	Highlight that combo UDM/HSS, mandatory entity in Rel15, becomes now option for termination of IMS (termination of IMS is in the HSS part of the combo).

3.-	Specify a mechanism to retrieve the UE SUPI, when IMS requests are based on IMPU, prior to triggering any retrieval of T-ADS/NPLI to AMF or carry out any P-CSCF restoration procedure. Where to store the IMPU-SUPI relation needs to be determined during normative phase.

NOTE 2: 	These conclusions require modification of TS 23.228 to clarify the interactions between HSS, UDM and IMS; the responsibility for this modification will be coordinated with any normative work arising from FS_eIMS5G [14].

******************End of changes***********************