3GPP TSG-SA2 Meeting #130	S2-1900115

# Kochi, India, 21 – 25 January, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0754 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Description of solution 2 in 23.725 for redundancy as an informational annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC |  |  |  |  |  | Date: |  |  | 2019-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Impement the conclusion of the FS_URLLC for user plane redundancy |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add multiple UE based solution for user plane redundancy as an informational annex, not requiring normative changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusion of FS_URLLC not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Appendix X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change ( all new text)* * * *

Annex X (informative):
Redundant user plane paths based on multiple UEs per device

Clause X.X. describes the use of redundant user plane path based on a single UE and dual connectivity in RAN. As an alternative, it is also possible to realize multiple user plane paths in the system based on a device having multiple UEs using the existing scpecifications, which is described below. The approach assumes a RAN deployment where redundant coverage by multiple gNBs is generally available. Upper layer protocols, such as the IEEE TSN (Time Sensitive Networking), can make use of the multiple user plane paths.

Multiple PDU Sessions are set up from the UEs, which use independent RAN (gNB) and CN (UPF) entities. The Figure X-1 below illustrates the architecture view of the solution. UE1 and UE2 are connected to gNB1 and gNB2, respectively and UE1 sets up a PDU Session via gNB1 to UPF1, while UE2 sets up a PDU Session via gNB2 to UPF2. UPF1 and UPF2 connect to the same Data Network (DN), but the traffic via UPF1 and UPF2 might be routed via different user plane nodes within the DN. UPF1 and UPF2 are controlled by SMF1 and SMF2, respectively. (Other 3GPP entities not relevant for this discussion are not shown in the figure).

Figure X-1: Architecture with redundancy based on multiple UEs in the device

This approach has a number of preconditions to be applicable.

-	The solution uses separate gNBs to achieve user plane redundancy over the 3GPP system. It is however up to operator deployment and configuration whether separate gNBs are available and used. If separate gNBs are not available for a device, the solution may still be applied to provide user plane redundancy in the rest of the network as well as between the device and the gNB using multiple UEs.

-	Terminal devices integrate multiple UEs which can connect to different gNBs independently.

-	RAN coverage is redundant in the target area: it is possible to connect to multiple gNBs from the same location. To ensure that the two UEs connect to different gNBs, the gNBs need to operate such that the selection of gNBs can be distinct from each other (e.g. gNBs operating in different frequencies etc.).

-	The core network UPF deployment is aligned with RAN deployment and supports redundant user plane paths.

-	The underlying transport topology is aligned with the RAN and UPF deployment and supports redundant user plane paths.

-	The physical network topology and geographical distribution of functions also supports the redundant user plane paths to the extent deemed necessary by the operator.

-	The operation of the redundant user plane paths is made sufficiently independent, to the extent deemed necessary by the operator, e.g., independent power supplies.

The approach comprises the following main components.

-	gNB selection: The selection of different gNBs for the UEs in the same device is realized by the concept of UE Reliability Groups (RG) for the UEs and also for the cells of gNBs. By grouping the UEs in the device and cells of gNBs in the network into more than one reliability group and preferably selecting cells in the same reliability group as the UE, it is ensured that UEs in the same device can be assigned different gNBs for redundancy as illustrated in Figure X-2 below, where UE1 and the cells of gNB1 belong to reliability group A, and UE2 and the cells of gNB2 belong to reliability group B.

Figure X-2: Reliability group-based redundancy concept in RAN

For determining the reliability grouping of a UE, one of the following methods or a combination of them can be used:

-	It could be configured explicitly to the UE and sent in a Registration Request message to the network using an existing parameter (such as an S-NSSAI in the Requested NSSAI where the SST is URLLC; the RG can be decided by the SD part).

-	It could also be derived from existing system parameters (e.g., SUPI, PEI, S-NSSAI, RFSP) based on operator configuration.

The Reliability Group of each UE is encoded into existing parameters and sent from AMF to RAN when the RAN context is established, and maintained as part of the RAN context, so each gNB has knowledge about the reliability group of the connected UEs. For this, the UE's Allowed NSSAI can be used as input to select the RFSP index value for the UE. The RAN node uses the RFSP for RRM purposes and can based on local configuration determine the UE's RG based on the S-NSSAI in Allowed NSSAI and/or S-NSSAI for the PDU Session(s).

The reliability group of the RAN (cells of gNBs) entities are pre-configured by the O&M system in RAN. It is possible for gNBs to learn the reliability group neighbouring cells as the Xn connectivity is set up, or the reliability group of neighbouring cells are also configured into the gNBs.

In the case of connected mode mobility, the serving gNB down-prioritizes candidate target cells that belong to different reliability group than the UE. It follows that normally the UE is handed over only to cells in the same reliability group. If cells in the same reliability group are not available (UE is out of the coverage of cells of its own reliability group or link quality is below a given threshold) the UE may be handed over to a cell in another reliability group as well.

In case the UE connects to a cell whose reliability group is different from the UE’s reliability group, the gNB initiates a handover to a cell in the appropriate reliability group whenever such a suitable cell is available.

If redundant RAN coverage is available at a certain location, then UEs that belong to the same terminal device will connect to different gNBs based on the reliability group classification using the connected mode mobility scheme described above.

If no cells in the same reliability group as the UE is available, then the UE may connect to a cell in another RGs.

In case of an idle UE, it is possible to use the existing cell (re-)selection priority mechanism, with a priori UE config using dedicated signalling (in the RRCConnectionRelease message during transition from connected to idle mode) to configure the UE to reselect the cells of the appropriate reliability group for camping in deployments where the cell reliability groups use different sets of frequencies.

-	UPF selection. Existing mechanisms can be used to select different UPFs for the UEs within the device. The selection may be based either on UE configuration or network configuration of different DNNs, or different slices for the two UEs. It is possible to use the UE's RG, described above for gNB selection, as an input to the UPF selection. The proper operator configuration of the UPF selection can ensure that the path of the PDU Sessions of UE1 and UE2 are independent.

-	Control plane. The approach may optionally apply different control plane entities for the individual UEs within the device. This may be achieved by using:

-	different DNNs for the individual UEs within the device to select different SMFs,

-	or applying different slices for the individual UEs within the device either based on UE configuration or network subscription, to select different AMFs and/or SMFs.

The UEs belonging to the same terminal device request the establishment of PDU Sessions that use independent RAN and CN network resources using the mechanisms outlined above.

Optionally, the timing of the handovers for the UEs within the same device can be co-ordinated so that packet losses during handover can be avoided, since we can preferably avoid simultaneous handovers of the two UEs. The approach can be extended for handover timing co-ordination as follows. Each Reliability Group is assigned with preferred time periods for handover, so that these periods are disjoint for the Reliability Group. A UE in given reliability group will therefore be preferably scheduled for handover at a different time than a UE in another reliability group. E.g., periods of a given length can be defined, so that for UEs in reliability group 1 handover are preferably started at odd periods, while for UEs in reliability group 2 handovers are preferably started at even periods. Note that the solution defines preferences, and in typical circumstances it can be possible to defer the handover for a short duration; but in cases where the radio link quality is quickly deteriorating, it is more important to perform handover as soon as possible and in such circumstances the preference may be ignored.

* * * * End of Change * * * *