SA WG2 Meeting #130	S2-1900527

January 21 - 25, 2019, Kochi, India 	(was S2-19----)

# Source:	CATT, Telecom Italia, CAICT, China Telecom, Tencent, OPPO

# Title:	Study on System enhancement for Mission Critical services in 5GS

# Document for:	Approval

Agenda Item:	7.1

Abstract: 	New study item proposal on 5G system enhancement for mission critical services.

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title: 	Study on System enhancement for Mission Critical services in 5GS

Acronym: FS_SeMC

Unique identifier: TBD

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 780002 | Removal of 'over LTE' limitation from Mission Critical Specifications | The requirements specified in 3GPP stage 1 |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure.}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 800023 | Study on Mission Critical services support over 5G System | SA6 study item to support mission critical services over 5GS |

3	Justification

TSG SA#78 confirmed that the stage 1 Mission Critical (MC) requirements are not only applied to LTE system, but also applied to 5G system.

According to the stage 1 MC specification, the MC services has to make use of network capabilities including Group Communication System Enabler, Proximity Service, etc. However, such network capabilities are not supported in current 5G system right now. Therefore, it is necessary to study the system enhancement for suppor of Misscion Critical Services.

As MBMS has not been supported in 5G system, support of MBMS in 5G system needs to be studied first. Following up the study, the leverage of MBMS in 5G system to support mission critical services will be developed.

Regarding Proximity Service, only public safety related part needs to be studied to support MC service, e.g. UE-to-Network relay, direct communication, direct discovery, etc.

It is worth to be noted that, RAN#82 decided to support mission critical services at latest in Rel-17, and SA6 also started the study item to support mission critical services in SP-181136. So the 5GS system architecture enhancement should be studied to make the whole system work.

4	Objective

In order to support Mission Critical services in 5G, the required network capabilies need to be reviewed. If a required network capability has not been supported in 5G system, e.g. ProSe Service, it shall be studied how to support it. Based on the stage 1 requirements on support of MC services, this study item shall at least study the following system enhancement:

-	Enhancement of 5G MBMS architecture to support mission critical services

Note: The 5G MBMS architecture should be studied in a separate study item.

-	Enhancement of Uu based architecture to support mission critical services, e.g. network slicing, 5QI, etc;

-	Enhancement of PC5 based architecture to suppot mission critical services, including:

- Direct Discovery

- Direct Communication

- UE-to-Network Relay

This study will also consider the architecture impact identified by the SA6 Study, Study on Mission Critical services support over 5G System, which target complete date is SA#85(Sept,2019).

This study will consider use exisiting soultuion as much as possible, e.g. using PC5 based architecture specified in R16 V2X as a basis, etc

The RAN WGs will be involved if needed.

5	Expected Output and Time scale

| New specifications [If Study Item, one TR is anticipated] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title | 1st rsp. WG | 2nd rsp. WG(s) | Presented for information at plenary# | Approved at plenary # | Comments |
| 23.xxx | Study on System enhancement for Mission Critical services in 5GS | SA2 |  | SA#86/DEC 2019 | SA#87/MARCH 2020 |  |

| Affected existing specifications [None in the case of Study Items] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Spec No. | CR | Subject of the CR | Approved at plenary# | Comments |

6	Work item Rapporteur(s)

Deng Qiang

dengqiang1@catt.cn

7	Work item leadership

SA2.

8	Aspects that involve other WGs

Security aspects (if any) will be addressed by SA3.

9	Supporting Individual Members

Supporting IM name

CATT

Telecom Italia

CAICT

China Telecom

Tencent

OPPO