SA WG2 Meeting #130 	S2-1900530

# 21 - 25 January 2019, Kochi, India											 (revision of S2-1900530)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0862 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF Selection influenced by the indication of the identity/identities of 5G AN N3 User Plane capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TR 23.716 § 8.2.9 Conclusions for user plane optimisation |  |  |  |  |  |  |  |  |
| Summary of change: |  | The 5G-AN may add the identity/identities of its N3 User Plane terminations when sending uplink NAS signalling to the 5GC. The AMF may relay this information to the SMF, as part of session management signalling for a new PDU Session. The SMF may use the identity/identities of the N3 User Plane terminations in the 5G-AN to select a possibly co-located UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to support the TR conclusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.3.3.3	Selection of an UPF for a particular PDU Session

If there is an existing PDU Session, and the SMF receives another PDU Session request to the same DNN and S-NSSAI, and the UE subscription data indicates the support for interworking with EPC for this DNN and S-NSSAI, the same UPF should be selected. Otherwise, if UE subscription data does not indicate the support for interworking with EPS for this DNN and S-NSSAI, a different UPF may be selected.

For the same DNN and S-NSSAI if different UPF are selected at 5GC, when the UE is moved to EPC network, there is no requirement to enforce APN-AMBR. Whether and how to apply APN-AMBR for the PDN Connection associated with this DNN/APN is implementation dependent, e.g. possibly only per PDU Session AMBR enforcement applies.

The following parameter(s) and information may be considered by the SMF for UPF selection and re-selection:

-	UPF's dynamic load.

-	UPF's relative static capacity among UPFs supporting the same DNN.

-	UPF location available at the SMF.

-	UE location information.

-	Capability of the UPF and the functionality required for the particular UE session: An appropriate UPF can be selected by matching the functionality and features required for an UE.

-	Data Network Name (DNN).

-	PDU Session Type (i.e. IPv4, IPv6, IPv4v6, Ethernet Type or Unstructured Type) and if applicable, the static IP address/prefix.

-	SSC mode selected for the PDU Session.

-	UE subscription profile in UDM.

-	DNAI as included in the PCC Rules and described in clause 5.6.7.

-	Local operator policies.

-	S-NSSAI.

-	Access technology being used by the UE.

-	Information related to user plane topology and user plane terminations, that may be deduced from:

-	5G-AN-provided identities (e.g. CellID, TAI), available UPF(s) and DNAI(s);

-	indication by the 5G-AN of the identity/identities of its N3 User Plane terminations;

-	Information regarding the user plane interfaces of UPF(s). This information may be acquired by the SMF using N4;

-	Information regarding the N3 User Plane termination(s) of the AN serving the UE. This may be deduced from 5G-AN-provided identities (e.g. CellID, TAI);

-	Information regarding the N9 User Plane termination(s) of UPF(s) if needed;

-	Information regarding the User plane termination(s) corresponding to DNAI(s).

NOTE 1:	How the SMF determines information about the user plane network topology from information listed above, and what information is considered by the SMF, is based on operator configuration.

The 5G-AN may add the identity/identities of its N3 User Plane terminations when sending uplink NAS signalling to the 5GC. The AMF may relay this information to the SMF, as part of session management signalling for a new PDU Session. The SMF may use the identity/identities of the N3 User Plane terminations in the 5G-AN to select a possibly co-located UPF.

NOTE 2: 	This capability can be leveraged in case of Wireline access where the UE is not assumed to move between 5G-AN(s)

NEXT CHANGE

END OF CHANGES