3GPP TSG-SA WG2 Meeting #130	S2-1900533

21 – 25 January 2019, Kochi, India

Source:	China Mobile, Huawei, Ericsson?, Nokia?

Title:	TS 23.288 NWADF-assisted QoS profile provision

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes to add the new feature about NWADF-assisted QoS profile provision into TS 23.288, related information and description are also given in this TS.

- Discussion

As discussed and studied in TR 23.791, Solution 3 is selected as the concluded solution for KI 5:NWDAF-Assisted QoS Profile Provisioning. This PCR proposes to add the new feature about NWADF-assisted QoS profile provision into TS 23.288 as an implemention.

- Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

7.2	NWDAF-Assisted QoS Profile Provisioning and Adjustment

7.2.1	General Procedure Description

NWDAF subscribes the network data from 5GC NF(s) and the service data from AF to offline trains a Service MOS Model for the given application and then determine the Service MOS for an application, which is provided to PCF to derive suitable QoS profiles especially for non-standardized values.

7.2.2	Information for the support of NWADF-Assisted QoS Profile Provisioning and Adjustment.

The service data from the AF and the network data from 5GC NFs for QoS profile Provisioning are defined in Table 7.2.2-1 and Table 7.2.2-2 here below, which allows NWDAF to have a snapshot of service experience for specific UEs as offline reported by AF.

Table7.2.2-1: Service Data from AF related to the observed service experience

| Information | Presence | Source | Description |
| --- | --- | --- | --- |
| Correlation ID | M | AF | Could be e.g. IP address 5-tuple or newly allocated temporary ID by 5GC , which is used by NWDAF to correlate the service data from AF and the network data from 5GC NF for the service. (NOTE 1) |
| Application ID | O | AF | To identify the service and support analytics per type of service (the desired level of service) |
| Service Experience | O | AF | E.g. Service MOS, MOS or video MOS as specified in ITU-T P.1203.3 [11], to characterize the experience for the service. (The observed level of service by AF) |
| Timestamp | M | AF | A time stamp associated to the observed level of Service Experience provided by the AF, mandatory if the observed Service Experience is provided by the ASP. |
| NOTE 1: The PCF could newly assign a correlation ID per UE per service, when receiving Npcf_PolicyAuthorization Create Request from AF, and send this correlation ID to AF and 5GC NFs, e.g. SMF, allowing NWDAF to correlate the service data from AF and the network data from 5GC NF as defined in Table 7.2.2-2. NOTE 2: The service data could be exposured by AF based on the Naf_EventExposure Service as defined in TS 23.502. |  |  |  |

Table 7.2.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Presence | Source | Description |
| --- | --- | --- | --- |
| Correlation ID | M | PCF | Could be e.g. IP address 5-tuple or newly allocated temporary ID by 5GC , which is used by NWDAF to correlate the service data and the network data for the service |
| Timestamp | M |  | The timing for the service. |
| Location Info | O | AMF | The location information for the service. |
| DNN | O | PCF/SMF | To identify the DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | M | PCF/SMF | To identify the S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | M | PCF/SMF | Provided by the AF, which is used by NWDAF to identify the application service provider and application for the QoS flow |
| IP filter information | O | PCF/SMF | Provided by the AF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QoS flow Bit Rate | M | PCF/SMF | To determine the QoS parameter: GFBR/MFBR |
| QoS flow Packet Delay | M | PCF/SMF | To determine the QoS parameter: PDB |
| QoS flow Packet Error Rate | M | PCF/SMF | To determine the QoS parameter: PER |
| NOTE 1: The QoS flow level network data from AMF could be exposured by AMF based on the Namf_EventExposure Service as defined in TS 23.502. NOTE 2: The QoS flow level network data from PCF could be exposured by PCF based on the Npcf_EventExposure Service as defined in TS 23.502. NOTE 3: The QoS flow level network data from SMF could be exposured by SMF based on the Nsmf_EventExposure Service as defined in TS 23.502. |  |  |  |

7.2.3	Procedure to support NWADF-Assisted QoS Provisioning and Adjustment

For an application Id, a set of initial QoS parameter combinations per service MOS window (e.g. one is for 3<Service MOS<4 and the other is for 4<Service MOS<5) is defined in PCF (e.g. by configuration of operator policies),.

1a.	PCF send an Analytics request/subscribe (Event ID = Estimated Service Experience statistics Event Filter information = (Application ID, time window, Media/application bandwidth, S-NSSAI, DNN)) to NWDAF by invoking a Nnwdaf_EventSubscription_Subscribe.

1b~1d.	NWDAF subscribes the network data from 5GC NF(s) as defined Table7.2.2-2 by invoking a Nnf_EventExposure_Subscribe (Event ID = 5QI Statistics, Event Filter information = Application ID, Target of subscription = Any UE) and the service data from AF as defined in Table 7.2.2-1 by invoking a Naf_EventExposure_Subscribe (Event ID = Service Data, Event Filter information = Application ID, Target of subscription = Any UE). With these data, the NWDAF offline trains a Service MOS Model for the given application, which will be used to determine the Service MOS for an application later on.

NOTE 1:	The data collection call flow only shows a subscription-notification model for the simplicity instead of both request-response model and subscription-notification model.

NOTE 2:	The explicit list of input data is not finalised and whether NWDAF needs to collect management data from OAM, such as PM measurements, is to be resolved in normative phase and some discussion with SA5 is needed to see what measurements are available and needed.

NOTE 3:	QoE measurements from the applications are based on outcome of the ongoing SA5 Rel-16 WID  "Management of QoE measurement collection" which addresses how to collect the QoE measurements from the applications in the UE.

1e.	The NWDAF provides the data analytics, i.e. the estimated Service Experience statistics (maybe a range) to the PCF by means of Nnwdaf_EventSubscription_Notify, indicating how well the used QoS Parameters satisfy the Application SLA:

a)	the estimated Service Experience statistics e.g. average Service MOS .

b)	Spatial validity condition, when the estimated Service Experience applies.

c)	Time validity condition, when the estimated Service Experience applies.

d)	NWDAF transaction id.

For example, time window could be from 20180601 to 20180630, which means PCF requests NWDAF to provide the average Service MOS score from, 20180601 to 20180607.

NOTE 4:	The NWDAF service consumed here by the PCF will be decided when a solution to key issue#1 is selected.

Please note that the call flow only shows a request-response model for the simplicity instead of both request-response model and subscription-notification model.

1f.	Based on the estimated Service Experience (this can be a range) provided by NWDAF, and the operator policies that takes into account the SLA (including the required Service Experience), the PCF takes into account to determine the used QoS parameters to be applied for the service if PCF determines that application SLA is not satisfied.

NOTE 5:	The non-real time data information from AF includes the service experience data (see Table 7.2.2-1), which indicates the service quality during the service lifetime.

7.2.4	Interaction with other NFs

To assist QoS profile provisioning, the following interactions and functions should be supported by the NWDAF and other NF/AF:

-	The NWDAF Collects QoS Flow level network data from 5GC NF(s),e.g. AMF and PCF, service data from AF, PM measurements and QoE data from UE;

-	Based on the collected data, the NWADF does service MOS training for a given application and obtains estimated Service Experience statistics.

-	The NWDAF provides the estimated Service Experience statistics to the PCF, indicating how good the used QoS Parameters satisfy the Application SLA.

-	Based on the estimated Service Experience statistics from the NWDAF, the PCF determines the used QoS parameters to be applied for the service.

* * * Next change * * * *

8.2.2	NWADF-Assisted QoS Profile Provisioning ans Adjustment

To assist QoS profile provisioning and adjustment, the NWDAF services as defined in the section 8.1.1 and 8.1.2 could be used to exposure data anlytics related to QoS Profile from the NWDAF to the PCF.A new Event ID named “Estimated Service Experience statistics” is defined with the following possible event filters:

- 	Application ID

- 	Time window

-	Media/application bandwidth

-	S-NSSAI

-	DNN

* * * End of changes * * * *