3GPP TSG-SA WG2 Meeting #130	S2-1900669

21 – 25 January 2019, Kochi, India

Source:	Orange

Title:	NWDAF - Architectural Model

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes to describe the scope and the general architecture principles of TS23.288.

# Discussion

# The majority of the text is taken from TR 23.791, with the following differences:

- Analytics are either statistical information in the past, or probabilistic information in the future.

- Different NWDAF instances may be present in the 5GCN, with possible specializations per categories of analytics. The capacities of a NWDAF instance are described in the NWDAF profile stored in the NRF.

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

# 1	Scope

The present document defines the Stage 2 architecture enhancements for 5G System (5GS) to support network data analytics services in 5G Core network.

This specification covers both roaming and non-roaming scenarios.

* * * Second change * * * *

# 4		General concepts

Editor’s note: This clause describes the architecture assumption of the Analytics Framework as described in Section 4.1 from TR 23.791.

The NWDAF (Network Data Analytics Function) as defined in TS 23.503 [4] provides analytics services to 5GC NFs, AFs, and OAM.

Analytics are either statistical information in the past, or probabilistic information in the future.

An NWDAF may be used for analytics for one or more Network Slice.

Different NWDAF instances may be present in the 5GCN, with possible specializations per categories of analytics. The capacities of a NWDAF instance are described in the NWDAF profile stored in the NRF.

5GS Network Functions and OAM decide how to use the data analytics provided by NWDAF to improve the network performance.

NWDAF utilizes the existing service based interfaces to communicate with other 5GC Network Functions and OAM.

A 5GC NF may expose the result of the data analytics to any consumer NF utilizing a service based interface.

The interactions between NF(s) and the NWDAF take place in the local PLMN (the reporting NF and the NWDAF belong to the same PLMN).

The NWDAF has no knowledge about NF application logic. The NWDAF may use subscription data but only for statistical purpose.

The NWDAF, NFs, AFS, OAM cooperate adequately to contribute to consistent policies, analytics output results, and finally decision-making in the PLMN.

* * * Third change * * * *

# 5	Architectural Model

## 5.1	General

The NWDAF interact with different entities for different purposes

- Data collection based on event subscription, provided by NFs of the 5GCN (AMF, SMF, PCF, UDR, NEF), AFs, and OAM

- Retrieval of information from data repositories : UDR for subscriber-related information, NRF for NF-related information, and NSSF or slice-related information

- On demand provision of analytics to NFs of the 5GCN, AFs and OAM.

The NWDAF reuses existing mechanisms and interfaces defined in 5GCN or OAM.

NF

Data collection

Analytics

AF

NEF

AF

NF

AF

NEF

AF

OAM

Repository access

NWDAF

UDM

NRF

Other repositoriess

Data collection and analytics

Figure 5.1-1: general framework for 5G network automation

* * * End of changes * * * *