3GPP TSG-SA WG2 Meeting #130	S2-1801085

21 – 25 January 2019, Kochi, India

Source:	CATT

Title:	NWADF-assisted mobility management customization

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes to add the new feature about NWADF-assisted mobility management customization into TS 23.288, related information and description are also given in this TS.

- Discussion

As discussed and studied in TR 23.791, Solution 4, 22,  and 23 are selected as the concluded solution for KI 9:NWDAF-Assisted mobility management customization. This PCR proposes to add the new feature about NWADF-assisted mobility management customization into TS 23.288 as an implemention.

In addition, although solution 29 is selected as the basis for normative work on NWDAF assisted performance enhancement for the battery lifetime management of mIoT terminals, the main idea of this solution can also be used to optimize connection management for general UEs, so it is proposed that solution 29 is also included in this mobility management customization feature.

- Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

7.X	UE mobility Analytics

7.X.1	General

In order to provide, e.g. customized mobility management, in 5GS, NWDAF shall be able to perform data analytics on  UE mobility and provide the analytics results (i.e. UE mobility statistics or prediction) to the 5GC.

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from 5GC, OAM and/or AFs.

7.X.2	Input Data

The NWDAF supporting data analysis on UE mobility shall be able to collect UE mobility information from OAM, 5GC and AFs. The detailed information collected by the NWDAF could be MDT data from OAM, network data from 5GC and/or service data from AFs:

-	UE mobility information from OAM is MDT data;

-	Network data related to UE mobility from 5GC is defined in the table 7.x.2-1;

Table 7.x.2-1: Network Data from 5GC related to UE mobility

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE positions (1..max) | AMF | UE positions |
| >Timestamp |  | A time stamp associated to a UE location |
| >UE location |  | TA or cells that the UE stays |
| >Duration |  | The period that the UE in this location |

Editor’s Note:	The exact information carried in the UE positions is FFS.

-	Service data related to UE mobility provided by AFs is defined in the table 7.x.2-2;

Table 7.x.2-2: Service Data from AF related to UE mobility

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Application ID | Identifying the application providing this information |
| UE trajectory (1..max) | UE historical or predicted positions |
| >Timestamp | A time stamp associated to a UE location |
| >UE location | UE geographical location |
| >Duration | The duration that the UE stays in this location |

Editor’s Note:	How to correlate external UE ID and internal UE ID and the exact information carried in the UE trajectory is FFS.

7.X.3	Output Analytics

The NWDAF supporting data analysis on UE mobility shall be able to provide UE mobility analysis to comsumer NFs. The analytic results provided by the NWDAF could be UE mobility statistics, UE mobility predictions:

-	UE mobility statistics is defined in the table 7.x.3-1;

Table 7.x.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE ID | Internal UE ID |
| UE location (1..max) |  |
| >UE location | TA or cells which the UE stays |
| >Time slot | Time slot that the UE moves to this location |
| >Average Duration | Average time period that the UE stays in this location |
| >Frequncy | Number of times that the UE moves to this location |

Editor’s Note:	The exact information carried in the UE location is FFS.

-	UE mobility preditions is defined in the table 7.x.3-2;

Table 7.x.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE ID | Internal UE ID |
| Predicted UE location (1..max) |  |
| >UE location | TA or cells where the UE may move into |
| >Time slot | Time slot within which the UE may move to this location |
| >Duration | Time period that the UE may stay in this location |
| >Confidence | Confidence of this prediction |

Editor’s Note:	The exact information carried in the Predicted UE location is FFS.

* * * Next change * * * *

7.X	UE Comminication Analytics

7.X.1	General

In order to provide, e.g. customized mobility management, in 5GS, NWDAF shall be able to perform data analytics on  UE communication pattern and provide the analytics results (i.e. UE communication pattern prediction) to the 5GC.

NWDAF supporting UE communication pattern prediction shall be able to collect UE communication description from AFs.

7.X.2	Input Data

The NWDAF supporting data analysis on UE communication shall be able to collect communication information for the UE from AFs. The detailed information collected by the NWDAF could be service data related to UE communication description as defined in the table 7.x.2-1.

Table 7.x.2-1: Service Data from AF related to UE communication

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Application ID | Identifying the application providing this information |
| UE communication (1..max) | Historical or predicted communication description |
| >communication start | A time that a communication starts |
| >communication stop | A time that this communication stops |
| >periodic communication indicator | Identifies whether the communication with this application is periodical or not |
| >periodic time | Interval Time of periodic communication if this communication is periodical |

Editor’s Note:	How to correlate external UE ID and internal UE ID and the exact information carried in the UE communication is FFS.

7.X.3	Output Analytics

The NWDAF supporting data analysis on UE communication shall be able to provide the analytics to comsumer NFs. The analytic results provided by the NWDAF could be the UE communication patttern preditions as defined in the table 7.x.3-1;

Table 7.x.3-1: UE communication pattern predictions

| Information | Description |
| --- | --- |
| UE ID | Internal UE ID |
| Predicted UE communications (1..max) |  |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not |
| > Periodic time | Interval Time of periodic communication |
| > Scheduled communication time | Time zone and Day of the week when the UE may be available for communication |
| > Communication duration time | Duration interval time of periodic communication or the scheduled communication |
| > Traffic Profile | Identifies the traffic type: MO/MT, single packet or multiple packets, etc. |
| >Confidence | Confidence of this prediction |

Editor’s Note:	The exact information carried in the Predicted UE communications is FFS.

* * * End of changes * * * *