3GPP TSG-SA2 Meeting #131	S2-1901447

# Santa Cruz - Tenerife, Spain, 25th Feb 2019 - 1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0968 | rev | 1 | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Bindings between the consumer and producer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the procedures to establish binding between consumer and producer as concluded in TR 23.742. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for procedures to establish bindings between the consumer and producer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.x (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 23.502 CR XXX |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Resubmission of S2-1900614. The paper has been aligned with the CRs agreed in SA2 #130. |  |  |  |  |  |  |  |  |

**************** First Change ****************

*** BEGIN CHANGES ***

### 4.17.x Binding between consumer and producer

#### 4.17.x.1 General

This clause describes the procedures to establish binding between the Consumer and Producer.

Direct Communication or Indirect Communication procedures may be used between the Consumer and Producer. In case of Indirect Communication, an SCP is located between the Consumer and Producer.

#### 4.17.x.2 Binding created as part of service response

When the Consumer communicates with the Producer, the Producer may return a binding information to the Consumer. The Consumer stores the received binding information. Based on the received binding information the binding is established with a set of Producer instances, optionally in specific Location, or with the specific Producer instance.

Figure 4.17.x.2-1: Binding created as part of service response

1. 	If Direct Communication is used, the Consumer selects the NF Service Producer and sends the request to the selected NF Service Producer. If Indirect Communication mode is used the NF Service Consumer sends the request to the SCP and provides within the service request to the SCP the discovery and selection parameters necessary to discover and select a NF Service Producer.

2. 	The NF Service Producer sends a response to the NF Service Consumer. In the response the NF Service Producer may indicate a resource address with a binding information. If the NF Service Consumer receives a resource address, it uses it for subsequent requests regarding the concerned resource. Otherwise, the procedure ends here.

The resource address indicates a binding either with a set of NF Service Producers, optionally for a given Location, or with the specific NF Service Producer.

3.	The NF Service Consumer uses the indicated binding information received in the previous step for subsequent requests regarding concerned resource. The following applies when the NF Service Producer is selected:

-		In case of the binding is established with the NF Service Producer, the indicated NF Service Producer is selected. If the indicated NF Service Producer becomes unavailable, a different NF Service Producer within the same set of NF Service Producers is selected.

-	In case of the binding is established with a set of NF Service Producers, one of the NF Service Producers from the indicated set of NF Service Producers is selected.

-	In case of the binding is established with a set of NF Service Producers for a given Location, one of the NF Service Producers from the indicated set of NF Service Producers corresponding to the Location is selected.

4. 	The Producer sends a response to the Consumer. The Producer may respond with an updated resource address different to the one received in the previous response.

#### 4.17.x.3 Binding created as part of service request

If the Consumer instance can also be as a Producer for later communication from the contacted Producer, it may include binding information in the service request sent to the Producer.

Figure 4.17.x.3-1: Binding created as part of service request

1. 	Instance A, as an NF Service Consumer sends a service request using either Direct Communication or Indirect communication via SCP and Instance B is selected as NF Service Producer. If Instance A, or a Set to which Instance A belongs, can also be NF Service Producer for later communication from Instance B or another instance of a Set to which Instance B belongs, it may include a resource address with a binding information in the service request sent to the NF Service Producer.

The resource address indicates a binding either with Instance A or to a set to which Instance A belongs, optionally for a given Location.

2. 	Instance B as the NF Service Producer sends a response to the NF Service Consumer.

3.	Instance B as NF Service Consumer sends a service request to the resource address received in step 1 as described in Steps 3-4 in Figure 4.x.2-1.

*** END CHANGES ***