3GPP TSG- SA WG2 Meeting #131	S2-1901477

February 25 – March 1, 2019, Santa Cruz, Tenerife, Spain

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0911 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input for PCF selection for a MA PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 18-02-2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The SMF selects a PCF for the PDU session based on a list of input criteria that include DNN, S-NSSAI or a SUPI range or some local policies in the SMF. In addition, when the UE provides a "MA-PDU Request" to indicate to 5GC that the requested PDU Session is a MA-PDU Session, then the PCF selected for the PDU session needs to support Policy Control for ATSSS functionality described in TS 23.503, S2-1901464 are supported. Note that the selecting a PCF capable to handle a MA-PDU session is independent on whether the UE is connected via one or two accesses and just refers to the capabilities required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PCF discovery and selection is updated to include the PCF support for ATSSS as input criteria for PCF selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PCF discovery and selection may result in a PCF that does not support ATSSS functionality while the PDU session is a MA-PDU session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****** Start of Changes ******

#### 6.3.7.1	PCF discovery and selection for a UE or a PDU Session

PCF discovery and selection functionality is used by the AMF to select the PCF for a UE and by the SMF to select the PCF for a PDU Session, following the principle below:

-	The AMF may utilize the NRF to discover the candidate PCF instance(s) for a UE. In addition, PCF information may also be locally configured in the AMF. The AMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the AMF, depending on operator's policies.

-	The SMF may utilize the NRF to discover the candidate PCF instance(s) for a PDU Session. In addition, PCF information may also be locally configured in the SMF. The SMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the SMF, depending on operator's policies.

The following factors may be considered during the PCF selection by the SMF:

a)	Local operator policies.

b)	Selected Data Network Name (DNN) of the PDU session.

c)	Selected S-NSSAI of the PDU Session.

d)	SUPI range.

e)	MA-PDU Capability of the PCF, applicable if the PDU session establishment includes the “MA-PDU Request" indication.

The selected PCF for serving the UE and the selected PCF for serving a PDU session of this UE may be the same or may be different.

In the following scenarios, information about the PCF that has been selected by the AMF (i.e. the PCF id) can be forwarded to another NF and used instead of performing PCF discovery and selection as described above:

-	During AMF relocation, the AMF may receive a PCF id from the source AMF to enable the usage of the same PCF by the AMF, and the AMF may decide based on operator policy either to use the same PCF or select a new PCF.

-	The AMF may, based on operator policies, forward the selected PCF id to the SMF during the PDU Session Establishment procedure to enable the usage of the same PCF for the AMF and the SMF. The SMF may decide based on operator policy either to use the same PCF or select a new PCF.

-	In the roaming case, the AMF may, based on operator policies, e.g. roaming agreement, select the H-PCF in addition to the V-PCF for a UE by performing the PCF discovery and selection as described above. The AMF sends the selected H-PCF id to the V-PCF during the policy association establishment procedure.

****** End of Changes ******