SA WG2 Meeting #131 	S2-1901534

25 February – 1 March 2019, Santa Cruz - Tenerife, Spain	 (revision of S2-19xxxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	Adding/Removing a UP path to a MA-PDU Session

Document for:	Discussion

Agenda Item:	6.8

Work Item / Release:	ATSSS

Abstract of the contribution: This paper discusses the need for having a solution that works for home-routed roaming case with two AMFs and proposes to use the PDU Session Establishment to establish the additional access. It also proposes to use the same procedure for all non-roaming and roaming scenarios, which drastically simplifies the solution from both UE and network perspectives.

# Discussion

When an UE requests a MA-DU session, the first UP path is established as for a regular PDU Session, i.e. after having obtained the PCC rules from the PCF.

For non-roaming and LBO roaming scenarios, it was proposed in S2-1900850 to establish the additional UP path without explicit request from the UE, mainly because it saves one message exchange in the case the UE is simultaneously registered in both accesses. The principle of S2-1900850 was that the registration of the UE in the second access triggers the establishment of the additional UP path:

1)	During the establishment of the MA-PDU Session in the first access, the H-SMF, i.e. the one that decides whether the PDU Session is to be Multi Access or not, subscribes to AMF event "UE registered on the two accesses";

2)	When the UE registers to the second access, the AMF notifies the H-SMF about the registration on two accesses, and then the H-SMF performs the establishment of the additional UP path towards the UE. For that, H-SMF needs to follow the Network Requested PDU Session Modification procedure as described in TS 23.502 clause 4.3.3.3 (sending a Nsmf_PDUSession_Update Request to the V-SMF, which will send a N2N2MessageTransfer (N2 SM Container) to the AMF).

The problem comes from the home routed roaming scenario with different AMFs for 3GPP and N3GPP accesses: the UE registers to another AMF. Hence the H-SMF will not be notified and the additional UP path will not be established.

It is common understanding that the Home Routed roaming scenario cannot be excluded from ATSSS: it is required at least for Wireline-Wireless Hybrid case when a wireline operator, associated with a partner mobile operator for radio access, wants to offer to its subscribers an hybrid access: the RGW is connected to its wireline operator 5G Core Network simultaneously via the fixed line (as a non-roamer) and via a NR mobile access network (as a roamer). In such case, there are two separate AMFs.

One way to solve the issue is to use the PDU Session Establishment procedure to request the establishment of the additional UP path. Indeed, PDU Establishment procedure is already used for handover a PDU session between 3GPP and non-3GPP accesses, and it makes sense to reuse it for adding a leg to a MA-PDU Session.

The rationale of using the PDU Session Establishment procedure rather than Service Request or PDU Session Modification is the following:

1)	A solution using the Service Request procedure initiated by the UE (instead of PDU Session Establishment procedure) is to be ruled out because it would go against the main principles of Service Request:

a) 	Service Request is a MM procedure potentially covering several PDU Sessions. ATSSS procedure is a Session Management procedure.

b) 	Service request is used to activate an already established user plane, not to establish new user plane resources: 
" The Service Request procedure is used by a UE in CM-IDLE state or the 5GC to request the establishment of a secure connection to an AMF. The Service Request procedure is also used both when the UE is in CM-IDLE and in CM-CONNECTED to activate a User Plane connection for an established PDU Session."

c) 	Service Request would need to be updated to indicate whether a PDU Session is moved or is added (SM parameter).

2)	Using the PDU Session Modification procedure would not either be appropriate because this procedure is dedicated to QoS modification.

As said above, we consider that the establishment of the additional UP path is very similar to a handover between 3GPP and non-3GPP, which uses the PDU Session Establishment procedure with Request Type = "Existing PDU Session". For the establishment of the additional UP path, an additional value of the Request Type "Additional access to existing PDU Session" could be used instead.

Moreover, instead of using the UE-initiated PDU Session Establishment procedure just for the case the UE is registered to two separate AMFs, it is proposed to use it for all the non-roaming and roaming cases: it would drastically simplify the solution from both UE and network perspectives. Using, for non-roaming and roaming cases with a single AMF, the Network Initiated PDU Session Modification procedure, triggered by the UE registration on the second access, as proposed in the baseline text of 23.502 CR 0866r2 (S2-1900850) would result in:

- Different procedures as seen by the UE depending on whether the UE is registered via one or two AMFs;

- Additional procedures in the network: the SMF has to subscribe to AMF notification "UE registered on the two accesses" to trigger the PDU Session Modification procedure;

- More complex Inter-Operability Testing.

# Proposal

It is proposed to, for all non-roaming and roaming cases, to specify the use of PDU Session Establishment, initiated by the UE, to establish the User Plane path over the second access. A companion CR to TS 23.502 in S2-1901535 is proposed for agreement. A revision of the previosuly agreed CR to TS 23.501 (S2-1900883) is provided in S2-1901536.