3GPP TSG-SA WG2 Meeting #131	S2-1901878

# 25 February - 1 March, 2019, Santa Cruz – Tenerife, Spain	(revision of S2-1901399)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0834 | rev | 6 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NAS level congestion control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Qualcomm Incorporated, MediaTek Inc., Ericsson, Sharp, LG Electronics, OPPO, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2019-01-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | (1) There are some inaccurate description when the DNN or S-NSSAI based congestion control is activated in AMF. (2) With regard to CT1 LS In C1-187008, it described flowing scenario and asked following questions to SA2: PLMN#A’s default S-NSSAI is S-NSSAI#A. PLMN#B’s default S-NSSAI is S-NSSAI #B. - The UE registers to PLMN#A and requests SM request without S-NSSAI. - The PLMN#A rejects the SM request with a back-off timer due to the S-NSSAI only based congestion control. This means the SM request to S-NSSAI#A in PLMN#A is under the congestion control. - The UE changes the PLMN to PLMN#B where the default S-NSSAI is S-NSSAI#B and S-NSSAI#B is NOT under the S-NSSAI only based congestion control. CT1 would like to ask: Question 1: Upon PLMN change, if a back-off timer is running due to S-NSSAI only based congestion control and associated with no S-NSSAI, is the UE allowed to send a PDU SESSION ESTABLISHMENT REQUEST message without S-NSSAI in new PLMN? Question 2: Upon PLMN change, if a back-off timer is running for [S-NSSAI, DNN] based congestion control and associated with no [S-NSSAI, DNN], is the UE allowed to send a PDU SESSION ESTABLISHMENT REQUEST message without [S-NSSAI, DNN] in new PLMN? Stage 2 specification only describes the case that the back-off timer associated to an S-NSSAI or a combination of an S-NSSAI and a DNN but doesn’t cover the case that a back-off timer is not associated to an S-NSSAI or a combination of the S-NSSAI and a DNN. According to 24.501: The S-NSSAI associated with T3584 is the S-NSSAI provided by the UE during the PDU session establishment. The DNN associated with T3584 is the DNN provided by the UE during the PDU session establishment. If no S-NSSAI but DNN is provided by the UE along the PDU SESSION ESTABLISHMENT REQUEST message, then T3584 is associated with no S-NSSAI and a DNN provided to the network during the PDU session establishment. If no DNN but S-NSSAI is provided by the UE along the PDU SESSION ESTABLISHMENT REQUEST message, then T3584 is associated with no DNN and an S-NSSAI provided to the network during the PDU session establishment. If no DNN and no S-NSSAI is provided by the UE along the PDU SESSION ESTABLISHMENT REQUEST message, then T3584 is associated with no DNN and no S-NSSAI. For this purpose, the UE shall memorize the DNN and S-NSSAI provided to the network during the PDU session establishment. The timer T3584 associated with no DNN and an S-NSSAI will never be started due to any 5GSM procedure related to an emergency PDU session. If the timer T3584 associated with no DNN and an S-NSSAI is running, it does not affect the ability of the UE to request an emergency PDU session. The S-NSSAI associated with T3585 is the S-NSSAI provided by the UE during the PDU session establishment. If no S-NSSAI is provided by the UE along the PDU SESSION ESTABLISHMENT REQUEST message, then T3585 is associated with no S-NSSAI. From the above yellow highlighted, stage 3 has allowed the case that a back-off timer is associated with no DNN, no S-NSSAI or associated with no DNN and no S-NSSAI. Considering the current late stage for Rel-15 5GS maintenance, it’s better for SA2 to align with 24.501 if there is nothing wrong. (3) The back-off timer is per PLMN granularity, according to current stage 2 specification, when the UE receives a SM back-off timer, it doesn’t know whether it is from VPLMN or HPLMN in the roaming case because the S-NSSAI for the VPLMN may be the same with the S-NSSAI for the HPLMN for a PDU Session. Without clearly knowing the back-off timer belongs to which PLMN, the UE will be confused on setting the back-off timer to VPLMN or HPLMN. If for example only VPLMN is congested, but the UE sets the back-off timer associated with the S-NSSAI for both VPLMN and HPLMN, and later UE selects to another VPLMN, because there is back-off timer associated with the specific S-NSSAI of the HPLMN running, the UE will not be able to request PDU Session Establishment for the S-NSSAI in the new VPLMN; or the UE moves to HPLMN coverage, the UE cannot request PDU Session Establishment for the S-NSSAI in the HPLMN even the HPLMN is not congested. The UE behaves as following for the roaming case: - If the back-off timer with an associated S-NSSAI is for HPLMN, the UE shall not initiate any Session Management procedures for the congested S-NSSAI of HPLMN until the timer is stopped or expires; If the back-off timer with an associated S-NSSAI is for VPLMN, when the UE selects to a new VPLMN, the UE is allowed to initiate Session Management procedures for this S-NSSAI if there is no running back-off timer associated with this S-NSSAI for the new VPLMN; - If the back-off timer with an associated S-NSSAI and a DNN is for HPLMN, the UE shall not initiate any Session Management procedures for the congested combination of S-NSSAI and DNN of HPLMN until the timer is stopped or expires; If the back-off timer with an associated S-NSSAI and DNN is for VPLMN, when the UE selects to a new VPLMN, the UE is allowed to initiate Session Management procedures for this S-NSSAI and DNN combination if there is no running back-off timer associated with this S-NSSAI and DNN combination for the new VPLMN; |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the description when the DNN or S-NSSAI based congestion control is activated in AMF. - Add a new indication of HPLMN congestion, based on this indication UE knows whether a SM back-off timer is for the RPLMN only or for any other PLMN as well, and behaves accordingly as described in the changes. - Apply some editorial and wording changes to make the texts more readable. - Clarify the scenario when S-NSSAI based back-off timer is not associated with any S-NSSAI and the scenario when S-NSSAI based back-off timer is not associated with any S-NSSAI and DNN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing and wrong description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.19.7.3, 5.19.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

>>> Start Changes <<<<

#### 5.19.7.3	DNN based congestion control

The design of the DNN based congestion control is for the purpose of avoiding and handling of NAS SM signalling congestion towards the UEs with a back-off timer associated with or without a DNN regardless of the presence of an S-NSSAI. Both UE and 5GC shall support the functionality to enable DNN based congestion control.

SMFs may apply DNN based congestion control towards the UE by rejecting PDU Session Establishment Request message, or PDU Session Modification Request message except for the purpose of reporting 3GPP PS Data Off status change for a specific DNN with a running back-off timer. The SMF may release PDU Sessions belonging to a congested DNN by sending a PDU Session Release Request message towards the UE with including a back-off timer. If a back-off timer is set in the PDU Session Release Request message, the cause value of "reactivation requested" shall not be set.

When DNN based congestion control is activated at AMF e.g., configured by OAM, the AMF provides a NAS Transport Error message for the NAS Transport message carrying an SM message, and in the NAS Transport Error message it includes a back-off timer. While the back-off timer for the specific DNN is running, the UE shall not send any NAS messages for the specific DNN.

NOTE: The UE associates the received back-off timer with the DNN (i.e. no DNN, DNN only) which was included in the uplink NAS MM message carrying the corresponding NAS SM request message.

Upon reception of the back-off timer, the UE behaves as followswhen the back-off timer is running:

-	If a DNN is associated with the back-off timer, the UE shall not initiate any Session Management procedures for the congested DNN. The UE may initiate Session Management procedures for other DNNs. The UE shall not initiate any Session Management procedure for the corresponding APN when UE moves to EPS. The UE may initiate Session Management procedures for other APNs when the UE moves to EPS;

-	If no DNN is associated with the back-off timer, the UE may onlyinitiate Session Management requests of any PDU Session Type for specific DNN;

-	Upon Cell/TA/PLMN/RAT change, change of untrusted non-3GPP access network or change of Access Type, the UE shall not stop the back-off timer;

-	The UE is allowed to initiate the Session Management procedures for high priority access and emergency services;

-	The UE is allowed to initiate the Session Management procedure for reporting Data Off status change to the network;

-	If the UE receives a network initiated Session Management Request message for the congested DNN associated to a running back-off timer, the UE shall stop the back-off timer and respond to the 5GC;

-	The UE is allowed to initiate PDU Session Release procedure (e.g. sending PDU Session Release Request message). The UE shall not stop the back-off timer when the related PDU Session is released.

The UE shall maintain a separate back-off timer for each DNN that the UE may use.

To avoid that large amounts of UEs initiate deferred requests (almost) simultaneously, the 5GC should select the back-off timer value so that deferred requests are not synchronized.

The DNN based Session Management congestion control is applicable to the NAS SM signalling initiated from the UE in the Control Plane. The Session Management congestion control does not prevent the UE from sending and receiving data or initiating Service Request procedures for activating User Plane connection towards the DNN(s) that are under Session Management congestion control.

#### 5.19.7.4	S-NSSAI based congestion control

The design of the S-NSSAI based congestion control is for the purpose of avoiding and handling of NAS signalling congestion towards the UEs with back-off timer associated with or without an S-NSSAI regardless of the presence of a DNN.

S-NSSAI based congestion control is applied as follows:

-	If an S-NSSAI is determined as congested, then the SMF may apply S-NSSAI based congestion control towards the UE for SM requests and provides a back-off timer and conditionally an indication of HPLMN congestion;

NOTE: The UE associates the received back-off timer with the S-NSSAI and DNN (i.e. no S-NSSAI and no DNN, S-NSSAI only, an S-NSSAI and a DNN) which was included in the uplink NAS MM message carrying the corresponding NAS SM request message.

-	If the UE receives an S-NSSAI based back-off timer without an indication of HPLMN congestion, the UE shall apply the S-NSSAI back-off timer only in the PLMN in which the back-off timer was received. If the UE receives S-NSSAI back-off timer with an indication of HPLMN congestion, the UE shall apply the S-NSSAI based back-off timer in the PLMN in which the back-off timer was received and in any other PLMN;

-	The SMF may release PDU Sessions belonging to a congested S-NSSAI by sending a PDU Session Release Request message towards the UE with a back-off timer associated either to the S-NSSAI only (i.e. with no specific DNN) or a combination of the S-NSSAI and a specific DNN;

-	If S-NSSAI based congestion control is activated at AMF e.g., configured by OAM and an S-NSSAI is determined as congested, then the AMF applies S-NSSAI based congestion control towards the UE for UE-initiated Session Management requests. In this case, the AMF provides a NAS Transport Error message for the NAS Transport message carrying the SM message, and in the NAS Transport Error message it includes a back-off timer;

-	Upon reception of a back-off timer with either an associated S-NSSAI, or a combination of associated S-NSSAI and a DNN, or no associated S-NSSAI or no associated S-NSSAI and no DNN, the UE behaves as follows in the PLMN where the S-NSSAI based congestion control applies when the back-off timer is running:

-	If the back-off timer was associated with an S-NSSAI only (i.e. not associated with an S-NSSAI and a DNN), the UE shall not initiate any Session Management procedures for the congested S-NSSAI;

-	If the back-off timer was associated with an S-NSSAI and a DNN, then the UE shall not initiate any Session Management procedures for that combination of S-NSSAI and DNN;

-	If the UE receives a network-initiated Session Management Request message for the congested S-NSSAI only (i.e. with no specific DNN) while the back-off timer associated with the S-NSSAI only is running, the UE shall stop this back-off timer and respond to the 5GC;

-	If the UE receives a network-initiated Session Management Request message for the congested S-NSSAI, the UE shall stop this back-off timer and respond to the 5GC;

-	Upon Cell/TA/PLMN/RAT change, change of untrusted non-3GPP access network or change of Access Type, the UE shall not stop the back-off timer for any S-NSSAI or any combination of S-NSSAI and DNN;

-	The UE is allowed to initiate the Session Management procedures for high priority access and emergency services for the S-NSSAI;

-	The UE is allowed to initiate the Session Management procedure for reporting Data Off status change for the S-NSSAI or the combination of S-NSSAI and DNN.

-	If the back-off timer is not associated to any S-NSSAI, the UE may only initiate Session Management procedures for specific S-NSSAI;

-	If the back-off timer is not associated to any S-NSSAI and DNN, the UE may only initiate Session Management procedures for specific S-NSSAI and DNN;

-	The UE is allowed to initiate PDU Session Release procedure (e.g. sending PDU Session Release Request message). The UE shall not stop the back-off timer when the related PDU Session is released.

The UE shall maintain a separate back-off timer for each S-NSSAI and for each combination of S-NSSAI and DNN that the UE may use.

The back-off timer associated with an S-NSSAI or a combination of an S-NSSAI and a DNN shall onlyapply to congestion control for Session Management procedures when UE is in 5GS.

To avoid that large amounts of UEs initiate deferred requests (almost) simultaneously, the 5GC should select the value of the back-off timer for the S-NSSAI based congestion control so that deferred requests are not synchronized.

The S-NSSAI based congestion control does not prevent the UE from sending and receiving data or initiating Service Request procedure for activating User Plane connection belonging to the S-NSSAI that is under the congestion control.

>>> End of changes <<<