3GPP TSG-SA WG2 Meeting #131	S2-1901938

# 25 February - 1 March, 2019, Santa Cruz - Tenerife	(revision of S2-190xxxx)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0991 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Slice interworking LBO mode update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2019-02-19 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During PDU session establishment procedure, the UE selects an appropriate S-NSSAI for PDU session per URSP. For example, if the UE initiates an application identified by AppId 1, the UE looks URSP up and includes S-NSSAI 1 in PDU session establishment request. === URSP on the UE === AppId 1: S-NSSAI 1, DNN A AppId 2: S-NSSAI 2, DNN A ==================== In Local Breakout roaming case, the combined PGW-C+SMF interacts with: - UDM to retrieve subscribed S-NSSAIs for the UE, and - NSSF to retrieve VPLMN S-NSSAIs that are mapped to the subscribed HPLMN S-NSSAIs for the UE. Based on the information received from UDM and NSSF, and its local policy, the PGW-C+SMF allocates VPLMN S-NSSAI for PDN connection and provides it to the UE via PCO. However, it is not guaranteed that the S-NSSAI selected by PGW-C+SMF is also align with the URSP on the UE, especially in case of PGW-C+SMF serving more than one S-NSSAIs for the same DNN. For example, if the UE initiates an application identified by AppId 1, the UE includes DNN A in PDN connection establishment request. The PGW-C+SMF, which serves DNN A for S-NSSAI 1 and S-NSSAI 2, may allocate S-NSSAI 2 for the PDN connection. To avoid such a potential inconsistency, it is proposed that the PGW-C+SMF provides the corresponding HPLMN S-NSSAI, which is already available to the PGW-C+SMF, as well as the VPLMN S-NSSAI to the UE via PCO. The UE may use the information received in PCO and URSP when it generates Requested NSSAI after moving to 5G network. For example, the UE provides VPLMN S-NSSAI value received via PCO in Requested NSSAI, corresponding HPLMN S-NSSAI value received via PCO in the mapping of Requested NSSAI, and the appropriate HPLMN S-NSSAI value per URSP in the mapping of Requested NSSAI if the information received via PCO and URSP doesn’t match. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In case of Local Breakout, the PGW-C+SMF provides the corresponding HPLMN S-NSSAI as well as the VPLMN S-NSSAI to the UE via PCO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Start of the changes ***

5.15.7	Network slicing and Interworking with EPS

5.15.7.1	General

A 5GS supports Network Slicing and might need to interwork with the EPS in its PLMN or in other PLMNs as specified in clause 5.17.2. The EPC may support the Dedicated Core Networks (DCN). In some deployments, the MME selection may be assisted by a DCN-ID provided by the UE to the RAN (see TS 23.401 [26]).

Mobility between 5GC to EPC does not guarantee all active PDU Session(s) can be transferred to the EPC.

During PDN connection establishment in the EPC, the UE allocates the PDU Session ID and sends it to the PGW-C+SMF via PCO. An S-NSSAI associated with the PDN connection and the corresponding mapped HPLMN S-NSSAI value if applicable are determined based on the operator policy by the PGW-C+SMF, e.g. based on a combination of PGW-C+SMF address and APN, and are sent to the UE in PCO together with a PLMN ID that the S-NSSAI relates to. If the PGW-C+SMF supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, the PGW-C+SMF should only select an S-NSSAI that is mapped to the subscribed S-NSSAIs of the UE. The UE stores this S-NSSAI and the PLMN ID associated with the PDN connection. The UE derives Requested NSSAI by taking into account of the received PLMN ID. The Requested NSSAI is included in NAS Registration Request message and RRC carrying this Registration Request when the UE registers in 5GC if the UE is non-roaming or the UE has Configured NSSAI for the VPLMN in roaming case.

*** End of the changes ***