3GPP TSG-SA WG2 Meeting #131	S2- 1902046

24 February – 1 March 2019, Santa Cruz, Spain

Source:	Samsung

Title:	Parameters for NWDAF Analytic Info services

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This contribution proposes to clarify the parameters for analytic info services such as analytic ID, analytic target, and analytic filters.

- Discussion

This contribution proposes to clarify the parameters of NWDAF service operations such as Nnwdaf_AnalyticsSubscription_subscribe and Nnwdaf_Nnwdaf_AnalyticInfo_Request.

The release-15 NWDAF only had a single analytic information, load level information of network slice instances. However, the release-16 NWDAF is extending the type of analytic information having different service request parameters such as Analytic ID, target of analytics, and filters. Therefore, the service operation interfaces are needed to be extended to address various use-cases. Moreover, many use-cases uses inconsistent analytic IDs. For example, “MOS statistics” and “Service experience”, “network performance information” and “network condition list”.

This contribution proposes to summarize the descriptions of Section 6 into a table to capture hot to use NWDAF service at a glance.

2.	Proposal

It is proposed to include the added text into the TS 23.288. This contribution proposes to clarify the input parameters of NWDAF service operations that consumers provide.

* * * * First Change * * * *

7	Nnwdaf Services Description

7.1	General

The following table illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF |

The following table shows the analytic information provided by NWDAF service:

Table 7.1-2: Analytic information provided by NWDAF

| Analytic Information | Description | Operation semantics |
| --- | --- | --- |
| Slice Load level information | Request: Analytic ID: load level information Analytic Filter(s): network slice instance(s). Response: Requested Analytic data, including load level information of Network Slice instance(s). | Subscribe/Notify, Request/Response |
| Observed Service experience related network data analytics | Request: Analytic ID: Service Experience Analytic Filter(s): Network area, Time window, Media/application bandwidth, S-NSSAI, DNN Target of analytics: Any UE Response: QoS Parameters satisfy the Service MoS agreed between the MNO and the end user or between the MNO and the external ASP | Subscribe/Notify |
| NF load analytics | Request: Analytic ID: NF load information Analytic Filter(s): Target of analytics: NF Response: NF load analytics | Subscribe/Notify, Request/Response |
| Network Performance Information Analytics | Request: Analytic ID: Network Condition list Analytic Filter(s): Group ID, Time, Date, Network area Target of analytics: NF Response: Provides the network condition list in the area where the ASP will send background data traffic. | Subscribe/Notify, Request/Response |
| Expected UE behavioural parameters related network data analytics | Request: Analytic ID: Expected UE behaviour parameters Analytic Filter(s): Target of analytics: UE group ID or UE ID Response: The expected UE behaviour parameters |  |
| Abnormal behaviour related network data analytics | Request: Analytic ID: Abnormal behaviour Analytic Filter(s): Target of analytics: UE group ID or UE ID Response: The abnormal behaviour information together with a UE ID list from the NWDAF to the consumer NF (e.g. PCF or AF), indicating e.g. the UEs are misused or hijacked. | Subscribe/Notify, Request/Response |
| MICO Mode Parameter Optimization for mIoT Terminals | Request: Analytic ID: MICO optimization Analytic Filter(s): Target of analytics: UE ID Response: The analytic estimation of necessary parameter values and any missing external parameter values | Request/Response |
| UE mobility analytics | Request: Analytic ID: UE mobility information Analytic Filter(s): Target of analytics: UE ID Response: UE mobility statistics, UE mobility predictions | Subscribe/Notify, Request/Response |
| UE Communication Analytics | Request: Analytic ID: UE communication pattern predictions Analytic Filter(s): Target of analytics: UE ID Response: The UE communication pattern predictions | Subscribe/Notify, Request/Response |
| User plane congestion analytics | Request: Analytic ID: User plane congestion information Analytic Filter(s): location Target of analytics: UE ID Response: User plane congestion information for a specific location, a specific UE, or their combination. | Subscribe/Notify, Request/Response |

7.2	Nnwdaf_AnalyticsSubscription Service

7.2.1	General

Service Description: this service enables the consumer to subscribe/unsubscribe for network data analytics. Periodic notification and notification upon threshold exceeded can be subscribed.

Different Analytic IDs are involved for different type of network data analytic, the specific Analytic ID and their corresponding Analytic Filter are defined in clause 6 for each specific network data analytics.

7.2.2	Nnwdaf_AnalyticsSubscription_Subscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Subscribe.

Description: Subscribes to NWDAF analytic with specific parameters.

Inputs Required: (Set of) Analytic ID(s) defined in Table 7.1-2, Notification Target Address (+ Notification Correlation ID), Analytic Reporting Information.

Inputs, Optional: Target of Event Reporting, Analytic Filter(s) associated with each Analytic ID, Subscription Correlation ID (in case of modification of the analytic subscription.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription).

Outputs, Optional: None.

Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by NWDAF with this subscription.

7.2.3	Nnwdaf_AnalyticsSubscription_Unsubscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Unsubscribe.

Description: unsubscribe to NWDAF analytic.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Confirmation of the unsubscription.

Outputs, Optional: None.

7.2.4	Nnwdaf_AnalyticsSubscription_Notify service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Notify.

Description: NWDAF notifies the consumer instance of the analytic that has subscribed to the specific NWDAF service. Depending upon type of subscription this notification is either on a periodic basis or triggered whenever a threshold is crossed.

Inputs Required: Analytic ID, Notification Target Address (+ Notification Correlation ID), Notification Correlation Information.

Inputs, Optional: Analytic specific parameter.

Outputs Required: None.

Outputs, Optional: None.

7.3	Nnwdaf_AnalyticsInfo service

7.3.1	General

Service description: this service enables the consumer to request and get from NWDAF network data analytics.

Different Analytic IDs are involved for different type of network data analytic, the specific Analytic ID and their corresponding Analytic Filter are defined in clause 6 for each specific network data analytics.

7.3.2	Nnwdaf_AnalyticsInfo_Request service operation

Service operation name: Nnwdaf_AnalyticsInfo_Request.

Description: the consumer requests NWDAF operator specific analytics.

Inputs Required: Analytic ID defined in clause 6, Analytic Filter(s).

Inputs, Optional: None.

Outputs Required: Analysis with parameters.

Outputs, Optional: None.

* * * * End of change * * * *