3GPP TSG-SA2 Meeting #131	S2-1902067

# 25 February – 1 March 2019, Santa Cruz - Tenerife, Spain

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0734 | rev | 4 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 23.501: Introducing Non-public network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated?, NICT?, Intel?, Ericsson?, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-01-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FS_Vertical_LAN study has concluded to introduce support for non-public networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces non-public networks based on the conclusions documented in TR 23.734. Revision proposal: - Standalone NPN -> Exclusive NPN - Non-standalone NPN -> Integrated NPN - Remove EN for terminology - Remove EN for list of NIDs. -> Propose to keep list of NIDs per PLMN ID. - Support for Access control is clarified further (as assumed in the TR). - 3GPP should not assume whether it is a central or regional entity managing the identifier. 3GPP should only assume that the ID is unique. In general, all changes are shown in yellow/green highlight. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for non-public networks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, new clause 5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changes >>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[9]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[10]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[11]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[12]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[13]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3".

[14]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[17]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[18]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[19]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[20]	IETF RFC 7542: "The Network Access Identifier".

[21]	3GPP TS 23.002: "Network Architecture".

[22]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[23]	3GPP TS 23.221: "Architectural requirements".

[24]	3GPP TS 22.153: "Multimedia priority service".

[25]	3GPP TS 22.011: "Service Accessibility".

[26]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[27]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[28]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[29]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[32]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[33]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	3GPP TS 33.126: "Lawful Interception Requirements".

[36]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[37]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[38]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[39]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[40]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[41]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[42]	3GPP TS 38.401: "NG-RAN Architecture description".

[43]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[44]	IETF RFC 4960: "Stream Control Transmission Protocol".

[45]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[46]	3GPP TS 23.041: "Public Warning System".

[47]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[48]	3GPP TS 24.502: "Access to the 5G System (5GS) via non-3GPP access networks; Stage 3".

[49]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[50]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[51]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[52]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[53]	IETF RFC 1027: "Using ARP to Implement Transparent Subnet Gateways".

[54]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[55]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[56]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[57]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[58]	3GPP TS 29.510: "5G System: Network function repository services; Stage 3".

[59]	3GPP TS 29.502: "5G System: Session Management Services: Stage 3".

[60]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2) ".

[61]	3GPP TS 23.380: "IMS Restoration Procedures".

[62]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[63]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[64]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[65]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[66]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[67]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[68]	3GPP TS 32.255: "5G Data connectivity domain charging; Stage 2".

[69]	TS 38.306: "NR; User Equipment -UE) radio access capabilities".

[70]	TS 36.306: "Evolved Universal Terrestrial Radio Access -E-UTRA); User Equipment -UE) radio access capabilities".

[71]	3GPP TS 29.274: "5G System; Access and Mobility Management Services; Stage 3".

[72]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[X]	International Telecommunication Union (ITU), Standardization Bureau (TSB): "Operational Bulletin No. 1156."; http://handle.itu.int/11.1002/pub/810cad63-en (retrieved October 5, 2018).

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5G Access Network: An access network comprising a NG-RAN and/or non-3GPP AN connecting to a 5G Core Network.

5G Core Network: The core network specified in the present document. It connects to a 5G Access Network.

5G QoS Flow: The finest granularity for QoS forwarding treatment in the 5G System. All traffic mapped to the same 5G QoS Flow receive the same forwarding treatment (e.g. scheduling policy, queue management policy, rate shaping policy, RLC configuration, etc.). Providing different QoS forwarding treatment requires separate 5G QoS Flow.

5G QoS Identifier: A scalar that is used as a reference to a specific QoS forwarding behaviour (e.g. packet loss rate, packet delay budget) to be provided to a 5G QoS Flow. This may be implemented in the access network by the 5QI referencing node specific parameters that control the QoS forwarding treatment (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.).

5G System: 3GPP system consisting of 5G Access Network (AN), 5G Core Network and UE.

Allowed NSSAI: NSSAI provided by the Serving PLMN during e.g. a Registration procedure, indicating the S-NSSAIs values the UE could use in the Serving PLMN for the current registration area.

Allowed Area: Area where the UE is allowed to initiate communication as specified in clause 5.3.2.3.

AMF Region: An AMF Region consists of one or multiple AMF Sets.

AMF Set: An AMF Set consists of some AMFs that serve a given area and Network Slice(s). AMF Set is unique within an AMF Region and it comprises of AMFs that support the same Network Slice(s). Multiple AMF Sets may be defined per AMF Region.

Application identifier: An identifier that can be mapped to a specific application traffic detection rule.

AUSF Group ID: This refers to one or more AUSF instances managing a specific set of SUPIs.

Configured NSSAI: NSSAI provisioned in the UE applicable to one or more PLMNs.

DN Access Identifier (DNAI): Identifier of a user plane access to one or more DN(s) where applications are deployed.

Emergency Registered: A UE is considered Emergency Registered over an Access Type in a PLMN when in limited service state and registered for emergency services only over this Access Type in this PLMN.

Endpoint Address: An address used by a NF service consumer to access the NF service (i.e. to invoke service operations) provided by a NF service provider. An Endpoint Address is represented in the syntax of Uniform Resource Identifier (e.g. part of Resource URI of the NF service API).

ENPN enabled UE: A UE configured to use exclusive Non-Public Networks.

ENPN mode of operation: A UE operating in ENPN mode of operation only selects exclusive Non-Public Networks.

Expected UE Behaviour: Set of parameters provisioned by an external party to 5G network functions on the foreseen or expected UE behaviour, see clause 5.20.

Forbidden Area: An area where the UE is not allowed to initiate communication as specified in clause 5.3.2.3.

GBR QoS Flow: A QoS Flow using the GBR resource type or the Delay-critical GBR resource type and requiring guaranteed flow bit rate.

Initial Registration: UE registration in RM-DEREGISTERED state as specified in clause 5.3.2.

Local Area Data Network: a DN that is accessible by the UE only in specific locations, that provides connectivity to a specific DNN, and whose availability is provided to the UE.

Local Break Out (LBO): Roaming scenario for a PDU Session where the PDU Session Anchor and its controlling SMF are located in the serving PLMN (VPLMN).

Mobility Pattern: Network concept of determining within the AMF the UE mobility parameters as specified in clause 5.3.2.4.

Mobility Registration Update: UE re-registration when entering new TA outside the TAI List as specified in clause 5.3.2.

MPS-subscribed UE: A UE having a USIM with MPS subscription.

NGAP UE association: The logical per UE association between a 5G-AN node and an AMF.

NGAP UE-TNLA-binding: The binding between a NGAP UE association and a specific TNL association for a given UE.

Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

NOTE 2:	A network function can be implemented either as a network element on a dedicated hardware, as a software instance running on a dedicated hardware, or as a virtualised function instantiated on an appropriate platform, e.g. on a cloud infrastructure.

Network Instance: Information identifying a domain. Used by the UPF for traffic detection and routing.

Network Slice: A logical network that provides specific network capabilities and network characteristics.

Network Slice instance: A set of Network Function instances and the required resources (e.g. compute, storage and networking resources) which form a deployed Network Slice.

Non-GBR QoS Flow: A QoS Flow using the Non-GBR resource type and not requiring guaranteed flow bit rate.

NSI ID: an identifier for a Network Slice instance.

NF instance: an identifiable instance of the NF.

NF service: a functionality exposed by a NF through a service based interface and consumed by other authorized NFs.

NF service instance: an identifiable instance of the NF service.

NF service operation: An elementary unit a NF service is composed of.

NG-RAN: A radio access network that supports one or more of the following options with the common characteristics that it connects to 5GC:

1)	Standalone New Radio.

2)	New Radio is the anchor with E-UTRA extensions.

3)	Standalone E-UTRA.

4)	E-UTRA is the anchor with New Radio extensions.

Non-Allowed area: Area where the UE is allowed to initiate Registration procedure but no other communication as specified in clause 5.3.2.3.

Non-Seamless Non-3GPP offload: The offload of user plane traffic via non-3GPP access without traversing either N3IWF or UPF.

Integrated Non-Public Network: A non-public network deployed with the support of a PLMN

PDU Connectivity Service: A service that provides exchange of PDUs between a UE and a Data Network.

PDU Session: Association between the UE and a Data Network that provides a PDU connectivity service.

PDU Session Type: The type of PDU Session which can be IPv4, IPv6, IPv4v6, Ethernet or Unstructured.

Periodic Registration Update: UE re-registration at expiry of periodic registration timer as specified in clause 5.3.2.

(Radio) Access Network: See 5G Access Network.

Requested NSSAI: NSSAI provided by the UE to the Serving PLMN during registration.

Routing Indicator: Indicator that allows together with SUCI/SUPI Home Network Identifier to route network signalling to AUSF and UDM instances capable to serve the subscriber.

Service based interface: It represents how a set of services is provided/exposed by a given NF.

Service Continuity: The uninterrupted user experience of a service, including the cases where the IP address and/or anchoring point change.

Service Data Flow Filter: A set of packet flow header parameter values/ranges used to identify one or more of the packet (IP or Ethernet) flows constituting a Service Data Flow.

Service Data Flow Template: The set of Service Data Flow filters in a policy rule or an application identifier in a policy rule referring to an application detection filter, required for defining a Service Data Flow.

Session Continuity: The continuity of a PDU Session. For PDU Session of IPv4 or IPv6 or IPv4v6 type "session continuity" implies that the IP address is preserved for the lifetime of the PDU Session.

Exclusive Non-Public Network: A non-public network not relying on network functions provided by a PLMN

Subscribed S-NSSAI: S-NSSAI based on subscriber information, which a UE is subscribed to use in a PLMN

UDM Group ID: This refers to one or more UDM instances managing a specific set of SUPIs.

UDR Group ID: This refers to one or more UDR instances managing a specific set of SUPIs.

UPF Service Area: The area within which PDU Session associated with the UPF can be served by (R)AN nodes via a N3 interface between the (R)AN and the UPF without need to add a new UPF in between or to remove/re-allocate the UPF.

Uplink Classifier: UPF functionality that aims at diverting Uplink traffic, based on filter rules provided by SMF, towards Data Network.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.261 [2] apply:

Non-Public Network

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

5G-AN	5G Access Network

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5QI	5G QoS Identifier

AF	Application Function

AMF	Access and Mobility Management Function

AS	Access Stratum

AUSF	Authentication Server Function

BSF	Binding Support Function

CAPIF	Common API Framework for 3GPP northbound APIs

CP	Control Plane

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

ENPN	Exclusive Non-Public Network

FAR	Forwarding Action Rule

FQDN	Fully Qualified Domain Name

GFBR	Guaranteed Flow Bit Rate

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HR	Home Routed (roaming)

INPN	Integrated Non-Public Network

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LMF	Location Management Function

LRF	Location Retrieval Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MPS	Multimedia Priority Service

N3IWF	Non-3GPP InterWorking Function

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NSI ID	Network Slice Instance Identifier

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NWDAF	Network Data Analytics Function

PCF	Policy Control Function

PDR	Packet Detection Rule

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

QFI	QoS Flow Identifier

QoE	Quality of Experience

(R)AN	(Radio) Access Network

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

S-NSSAI	Single Network Slice Selection Assistance Information

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSP	Traffic Steering Policy

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

## <<< Next change >>>

## 5.X	Support for non-public networks

### 5.X.1	General

A Non-Public Network (NPN) is a 5GS deployed for non-public use, see TS 22.261 [2]. An NPN may be deployed as

-	an exclusive  Non-Public Network (ENPN), i.e. operated by an NPN operator independent of PLMN, not relying on network provided by a PLMN, or

-	an Integrated Non-Public Network (INPN), i.e. with the support of a PLMN, hosted by a PLMN.

ENPN 5GS deployments are based on the architecture depicted in clause 4.2.3 and the additional functionality covered in clause 5.X.2.

Exclusive NPN can be enabled using network slicing (see Annex X). To prevent unauthorized UEs from trying to access an Integrated NPN, the Closed Access Group (CAG) functionality described in clause 5.X.3 can be used in addition.

### 5.X.2	Exclusive non-public networks

#### 5.X.2.1	Identifiers

The combination of a PLMN ID and Network identifier (NID) identifies an ENPN.

NOTE 1:	The PLMN ID used for ENPNs is not required to be unique. PLMN IDs reserved for use by private networks can be used for non-public networks, e.g. based on mobile country code (MCC) 999 as assigned by ITU [X]).

The NID shall support two assignment models:

-	Locally managed NIDs are assumed to be chosen individually by ENPNs at deployment time (and may therefore not be unique in all scenarios)

-	Universally managed NIDs are assumed to be globally unique.

NOTE 2:	The number space for universally managed NIDs can be assigned by a global entity or can support indepedent NID assignment for different region, i.e. the NID number space can be split into regions.

NOTE 3:	Which legal entity or entities manage the number space is beyond the scope of this specification.

An optional human-readable network name helps to identify an ENPN during manual network selection.

#### 5.X.2.2	Broadcast system information

NG-RAN nodes which provide access to ENPNs broadcast the following information:

-	PLMN ID

NOTE 1:	The PLMN ID used for ENPNs is not required to be globally unique. Non-unique PLMN IDs reserved for private networks can be used, e.g. based on mobile country code (MCC) 999 as assigned by ITU [X]).

-	List of NIDs per PLMN ID identifying the non-public networks NG-RAN provides access to. The NID order is significant.

NOTE°2:	The presence of a list of NIDs for a PLMN ID indicates that the related PLMN ID and NID(s) identify an ENPN.

-	Optionally a human-readable network name per NID

NOTE 3:	The human-readable network name per NID is only used for manual selection.

NOTE 4: The mechanism how human-readable network name is provided (e.g. whether it is broadcasted or not) to the UE is specified in 3GPP TS 38.331[ 28].

-	Optionally information, as described in TS 38.331 [28] and in TS 38.304 [50], to prevent UEs not supporting ENPNs from accessing the cell, e.g. in case the cell only provides access to non-public networks.

#### 5.X.2.3	UE configuration and subscription aspects

An ENPN-enabled UE is configured with subscriber identifiers and credentials for one or multiple ENPNs identified by the combination of PLMN ID and NID.

A subscriber of an ENPN is identified by a SUPI containing a network-specific identifier that takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in TS 23.003 [19] clause 28.2.2. The realm part of the NAI may include the NID of the ENPN.

An ENPN-enabled UE supports the ENPN mode of operation. When the UE is set to operate in ENPN mode of operation, the UE only selects and registers with ENPNs and it does not perform normal PLMN selection procedures as defined in clause 4.4 of TS 23.122 [17].

Editor's note: Need for support of emergency services in ENPN mode of operation is FFS.

If a UE is not set to operate in ENPN mode of operation, even if it is ENPN-enabled, the UE does not select and register with ENPNs. A UE not set to operate in ENPN mode of operation performs PLMN selection procedures as defined in in clause 4.4 of TS 23.122 [17].

NOTE:	Details of activation and deactivation (i.e. how and when activation/deactivation is performed) of ENPN mode of operation are up to UE implementation.

Editor's note: Details of access to PLMN services in ENPN mode of operation are FFS.

#### 5.X.2.4	Network selection in ENPN mode of operation

UEs operating in ENPN mode of operation read the available PLMN IDs and list of available NIDs from the broadcast system information and take them into account during network selection.

For automatic network selection, the UE selects and attempts to register with the available ENPN identified by a PLMN ID and NID for which the UE has SUPI and credentials. If multiple ENPNs are available that the UE has credentials for, then the priority order for selecting and attempting to register with ENPNs is based on UE implementation.

For manual network selection UEs operating in NPN mode present the list of NIDs and related human-readable names (if available) of the available ENPNs the UE has SUPI and credentials for.

NOTE:	The details of ENPN selection will be defined in TS 23.122 [17].

When a UE performs Initial Registration to an ENPN, the UE shall indicate the selected NID and the corresponding PLMN ID to NG-RAN. NG-RAN shall inform the AMF of the selected PLMN ID and NID.

#### 5.X.2.5	Network access control

If a UE performs Initial registration in an ENPN identified by a PLMN ID and a locally assigned NID and there is no subscription for the UE, then the AMF shall reject the UE with an appropriate cause code to ensure that the UE does not retry to register with the same ENPN immediately.

There are two parts to access control:

- Prohibiting access to UE(s) that are unauthorized to access the Non-public network.

- To prevent access to Non-public network for authorized UE(s) due to network congestion/overload.

For access control as in item 1), the NPN cell shall broadcast information preventing non-supporting UEs from accessing the cell.

For access control as in item 2), reuse the existing access control framework as is. Access control information is configured per non-public network (i.e. as part of the subscription information that the UE has for a given non-public network).

-	The existing UAC framework can be reused as-is to prevent access for authorized UE(s) when the non-public network is overloaded. Access control information is configured in the UE per non-public network (i.e. as part of the subscription information that the UE has for a given non-public network).

#### 5.X.2.6	Cell (re-)selection in ENPN mode of operation

UEs operating in ENPN mode of operation only select cells and networks broadcasting both PLMN ID and NID.

NOTE:	Further details on the NR idle mode procedures for ENPN cell selection will be defined in TS 38.331 [28] and in TS 38.304 [50].

#### 5.X.2.7	Access to PLMN services via stand-alone non-public networks

A UE in ENPN mode of operation may access PLMN services following the same architectural principles as specified in clause 4.2.8 and the ENPN taking the role of “Untrusted non-3GPP access”.

Editor's note: The need to support QoS differentiation for access to PLMN services via non-public networks is FFS.

#### 5.X.2.8	Access to stand-alone non-public network services via PLMN

A UE may access ENPN services following the same architectural principles as specified in clause 4.2.8 and the PLMN taking the role of “Untrusted non-3GPP access”.

Editor's note: The need to support QoS differentiation for access to PLMN services via non-public networks is FFS.

## <<< End of changes >>>