3GPP TSG-SA WG2 Meeting #131 	S2-1902069

# 25 February – 1 March 2019, Santa Cruz - Tenerife, Spain

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0957 | rev | 4 | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eSBA communication schema Model C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBA |  |  |  |  |  |  | Date: |  | 2019-01-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FS_eSBA |  |  |  |  |  |  |  |  |
| Summary of change: |  | eSBA communication schema Model C support changes rev3rev4 option C should be possible to use with Rel 15 deployed consumers.. |
| Consequences if not approved: |  | Agreed communication schema not incorporated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.y.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 4.y.x	Indirect Communication without delegated discovery Procedure

This clause provides the call flow for indirect communication model without delegated discovery (as in Annex X).

Figure 4.y.x-1: Procedure for Indirect Communication without delegated discovery

The NF/NF service discovery procedure is defined in clauses 4.17.4 and 4.17.5. In a successful discovery the NF service consumer gets the NF profile(s) matching the search criteria provided in the Nnrf_NFDiscovery_Request message.

- When the NF Service Consumer needs to send a Service Request and has stored results from the Discovery Procedure, the NF Service Consumer selects an appropriate Producer NF instance from the list of NF profiles provided by the NRF. The NF Service Consumer considers the NF and NF service parameters (e.g. TAI, S-NSSAI, locality, priority etc) in the NF profiles. The NF Service consumer requests service from the NF Service producer by sending a service request message to the NF service producer via the SCP. The address of a NF Service producer instance may be unique per NF service producer instance or may be shared by multiple NF service producers. i.e. the address used by the NF service consumer may point to multiple NF service producer instances, or a single NF service producer instance.

- If the NF Service consumer provides an End Point address pointing to multiple producer instances, the SCP performs selection of NF Service Producer instance from the set of NF services that the address is pointing to. Otherwise the SCP uses the NF Service producer Instance that the address points to. The SCP forwards the Service Request to selected NF Service producer Instance.

- The NF Service Producer responds via SCP.

- SCP forwards the response.

*** END CHANGES ***