3GPP TSG-SA WG2 Meeting #131 	S2-1902422

# Santa Cruz - Tenerife, Spain, 25 Feb - 1 Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0894 | rev | 3 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CIoT Introduction of Overload Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | AMF and SMF overload and control plane data back-off timer features of EPC are being reused in 5GC as part of solution #27 for KI#7 in TR 23.724. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For AMF and SMF overload control the addition of exception data for NB-IoT is included. Addition of control plane data back-off timer as part of NAS level congestion control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.19.5.2, 5.19.7.x (new), 5.x.a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The new 5.x.a subclause should be added under new “Support for Cellular IoT” subclause. |  |  |  |  |  |  |  |  |

***** Start of Changes *****

#### 5.19.5.2	AMF Overload Control

Under unusual circumstances, if AMF has reached overload situation, the AMF activates NAS level congestion control as specified in Clause 5.19.7 and AMF restricts the load that the 5G-AN node(s) are generating, if the 5G-AN is configured to support overload control. N2 overload control can be achieved by the AMF invoking the N2 overload procedure (see TS 38.300 [27] and TS 38.413 [34]) to all or to a proportion of the 5G-AN nodes with which the AMF has N2 connections. The AMF may include the S-NSSAI(s) in N2 overload control message sent to 5G-AN node(s) to indicate the Network Slice(s) with which NAS signalling is to be restricted. To reflect the amount of load that the AMF wishes to reduce, the AMF can adjust the proportion of 5G-AN nodes which are sent NGAP OVERLOAD START message, and the content of the overload start procedure.

The AMF should select the 5G-AN node(s) to which it triggers overload start procedure at random to avoid that multiple AMFs in an AMF Set request reduction of load from the same subset of 5G-AN node(s).

A 5G-AN node supports rejecting or releasing of 5G-AN signalling connection establishments for certain UEs connected to 5GC as specified in TS 38.331 [28] and TS 36.331 [51]. Additionally, a 5G-AN node provides support for the barring of UEs as described in TS 22.261 [2]. These mechanisms are further specified in TS 38.331 [28] and TS 36.331 [51].

NOTE 1:	These mechanisms are applied only to the UEs that has accessed to 5GC or attempt to access to 5GC.

Using the overload start procedure, the AMF can request the 5G-AN node to:

a)	reject 5G-AN signaling connection (RRC Connection over 3GPP access or UE-N3IWF connection over N3GPP access) requests that are for non-emergency, non-exception reporting and non-high priority mobile originated services; or

b)	reject new 5G-AN signaling connection requests for uplink NAS signalling transmission to that AMF;

c)	release 5G-AN signalling connection where the Requested NSSAI at AS layer only includes the indicated S-NSSAI(s) in the N2 overload control message.

d)	only permit 5G-AN signaling connection requests for emergency sessions and mobile terminated services for that AMF; or

e)	only permit 5G-AN signaling connection requests for high priority sessions, exception reporting and mobile terminated services for that AMF;

NOTE 1:	The 5G-AN signaling connection requests listed in this clause also include the request from UE in RRC-Inactive state.

The AMF can provide percentage value that indicates how much amount of signalling traffic to be rejected in the overload start message, and the 5G-AN node may consider this value for congestion control.

When rejecting a 5G-AN signaling connection request for overload reasons (cases a and b above) the 5G-AN indicates to the UE an appropriate wait timer value that limits further 5G-AN signaling connection requests until the wait timer expires.

When releasing a 5G-AN signalling connection for the case c) above, the 5G-AN indicates to the UE an appropriate wait timer that limits further 5G-AN signalling connection requests until the wait timer expires.

During an overload situation, the AMF should attempt to maintain support for emergency services and for MPS.

When the AMF is recovering, the AMF can either:

-	trigger overload start procedure with new percentage value that permit more signalling traffic to be carried, or

-	the AMF trigger overload stop procedure.

to some or all of the 5G-AN node(s).

***** Next Change *****

### 5.19.7	NAS level congestion control

…

#### 5.19.7.x Control Plane data specific NAS level congestion control

Under overload conditions the AMF may restrict requests from UEs for data transmission via Control Plane CIoT 5GS Optimisation. A Control Plane data back-off timer may be returned by the AMF (e.g. in Registration Accept messages, Service Reject message or Service Accept message). While the Control Plane data back-off timer is running, the UE shall not initiate any data transfer via Control Plane CIoT 5GS Optimisation, i.e. the UE shall not send any Control Plane Service Request with uplink data as defined in TS 24.501 [47]. The AMF shall store the Control Plane data back-off timer per UE and shall not process any further requests (other than exception reporting and a response to paging) for Data Transport via Control Plane Service Request from that UE while the Control Plane data back-off timer is still running.

Editor’s note: Whether a new or existing NAS message (Control Plane Service Request) is used for Data Transport in Control Plane CIoT 5GS is to be defined by CT WGs

NOTE 1:	The Control Plane data back-off timer does not affect any other mobility management or session management procedure.

NOTE 2:	The Control Plane data back-off timer does not apply to user plane data communication.

If the UE is allowed to send exception reporting, the UE may initiate Control Plane Service Request for exception reporting even if Control Plane data back-off timer is running.

The UE may respond to paging with a Control Plane Service Request without uplink data to paging even if the Control Plane data back-off timer is running.

If the AMF receives a Control Plane Service Request in reponse to a paging, and the AMF has a Control Plane data back-off timer running for the UE, and the AMF is not overloaded, and AMF decides to accept the Control Plane Service Request, then the AMF shall respond with Service Accept without the Control Plane data back-off timer and stop the Control Plane data back-off timer. If the UE receives a Service Accept without the Control Plane data back-off timer from the AMF while the Control Plane data back-off timer is running, the UE shall stop the Control Plane data back-off timer. The Control Plane data back-off timer in the UE and the AMF is stopped at PLMN change.

If the AMF receives a Control Plane Service Request, and decides to send the UE a Control Plane data back-off timer, the AMF may decide to process the Control Plane Service Request, i.e. decrypt and forward the data payload, or not based on the following:

-	If the UE has indicated Release Assistance Information that no further Uplink or Downlink Data transmissions are expected, then the AMF may process (integrity check/decipher/forward) the received Control Plane data packet, and send a Service Accept to the UE with Control Plane data back-off timer. The UE interprets this as successful transmission of the Control Plane data packet starts the Control Plane data back-off timer.

-	For all other cases, the AMF may decide to not process the received Control Plane data packet and send a Service Reject to the UE with Control Plane data back-off timer. The UE interprets this indication as unsuccessful delivery of the control plane data packet and starts the Control Plane data back-off timer. Then the AMF may take into consideration whether the PDU Session is set to Control Plane only to make the decision whether to reject the packet and send Service Reject or move the PDU Session to user plane and process the data packet.

-	Alternatively, if UE has not provided Release Assistance Information, and the PDU Session not set to Control Plane only, and UE supports N3 data transfer, then the AMF may initiate establishment of N3 bearer according to the procedure defined in clause X.Y.Z. In this case the AMF may also return a Control Plane data back-off timer within the NAS message.

The AMF only includes the Control Plane data back-off timer if the UE has indicated support for Control Plane data back-off timer in the Registration Request.

NOTE 3:	If the AMF is overloaded or close to overload, but the UE has not indicated support for Control Plane data back-off timer, the AMF can use other overload control mechanisms, e.g. Mobility Management back-off timer or use user plane data communication.

***** Next Change *****

### 5.x.a Control Plane Data Transfer Congestion Control

NAS level congestion control may be applied in general for all NAS messages. To enable congestion control for control plane data transfer, a Control Plane data back-off timer is used, see clause 5.19.7.x.

***** End of changes *****