SA WG2 Meeting #131	S2-1902841
Tenerife, Spain, 25th Feb- 1st Mar 2019	(revision of S2-1902671)

# Source:	China Mobile, AT&T, Telstra, Vodafone, ZTE, CATT, Interdigital, Tencent, Samsung, Deutsche Telekom，SK，Intel

# Title:	New SID: Study on introducing UPF service in 5GC

# Document for:	Approval

Agenda Item:	7.1

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title: 	Study on introducing UPF service in 5GC

Acronym: FS_5G_UPF service

Unique identifier:

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X |  | X |  |  |
| Don’t know |  | X |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure.}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| TR 23.742 | Study on Enhancements to the Service-Based Architecture (Release 16) | Study of extending SBA to user plane but without standardization |

3	Justification

- It is important to allow deployments where User Plane handling can be made modular as UPF services in order to be able to dynamically insert in the data path of a PDU Session functionalities able e.g. to

  - monitor traffic

  - do DPI

  - support MP-TCP

  - others

- Improve ease for exposure for reporting of information by the UPF

Saving duplicate data transfer and reducing transmission path

  - example 1: PCF services can directly subscribe/unsubscribe UPF services for QoS monitoring  latency report

Retrieving original status or real-time service flow information from UPF

  - example 2: NWDAF services can subscribe/unsubscribe to UPF for retrieving real-time service flow information e., to help big-data collection and analysis considering efficient sampling intervals for the different services

UPF event exposure

  - example 3: 5G IoT solutions would require interfacing of UPF to NEF for 3rd party exposure,.

4	Objective

The study item will consider following aspects:

- Support for modular design of User Plane (UP) handling:

  - a)	identify the UP features as UPF services

  - b)	to support modular deployments identified above: evaluate the usage of the existing architecture (evolution of N4/PFCP constructs such as PDR, FAR … is a potential solution) as well as the usage of UPF services base architecture with a cloud native approach and determine the best solution

2)   Mechanism to support efficient service communication:

-	Identify UPF service(s) that would support:

- Communication between UPF services and PCF services, NWDAF services, CHF services or NEF services.

NOTE 0: use case(s) should be studied first,.

-	Study potential architectural impacts of these UPF service(s)

NOTE 1:In order to support backward compatibility, protocols on the interfaces between UP handling entities within 5GC and on N3 are kept un-changed with regard to their R16 definition;

NOTE 2: SMF is responsible for controlling UPF packet processing services

NOTE 3: The performance UP traffic handling shall be considered in this study.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Rapporteur(s) (see note 2) | For info at TSG# | For approval at TSG# | Remarks |
| New TR 23.7xy | Internal TR | Dan Wang (wangdanyjy@chinamobile.com) | TSG#86 (Dec 2019) | TSG#87 (Mar 2020) | Including the study outputs on key issues, solutions and architecture impacts. |

Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
“Internal TR” is intended for 3GPP internal use only whereas “External TR” may be transposed by Ops.

Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as “Remarks”.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |

6	Work item Rapporteur(s)

Dan Wang, China Mobile, wangdanyjy@chinamobile.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Mobile

AT&T

CATT

Deutsche Telekom

Intel

Interdigital

Samsung

Sandvine

SK

Telstra

Tencent

Vodafone

ZTE

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"