3GPP TSG-SA WG2 Meeting #132 	S2-1903175

Xi’an, China, 8 - 12 April, 2019 	(revision of S2-19xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1153 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating how binding works between NF service consumers and NF service producers. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBA |  |  |  |  |  | Date: |  |  | 2019-03-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Updating how binding works between NF service consumers and NF service producers per DP S2-1903174 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not following agreed DP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.11.2, 4.17.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 4.17.11	Binding between consumer and producer

#### 4.17.11.1	General

This clause describes the procedures to establish binding between the Consumer and Producer.

Direct Communication or Indirect Communication procedures may be used between the Consumer and Producer. In the case of Indirect Communication, an SCP is located between the Consumer and Producer.

#### 4.17.11.2	Binding created as part of service response

When the NF service consumer communicates with the NF Service producer, itr may return a resource address to the NF service consumer. The resource address consists of an address (e.g. FQDN:port and path) of a service instance, service type and a resource identifier (e.g. SM context ID). The NF service consumer stores the received resource address information.

Figure 4.17.11.2-1: Binding created as part of service response

1.	The NF Service consumer or SCP selects the NF Service producer and sends the request to the selected NF Service producer.

2.	The NF Service producer sends a response towards the NF Service consumer. In the response the NF Service producer may include a resource address of a created resource. If the NF Service consumer does not receive a resource address the procedure ends here.

3.	The NF Service consumer uses the indicated resource address received in the previous step for subsequent requests regarding concerned resource. The following applies when the NF Service producer instance is selected by the NF Service consumer or SCP:

-	In the case the previously selected NF service does not belong to a NF service set, the NF Service consumer uses the resource address received in the response. Re-selection can be done according to TS 23.501 [2] clause 6.3.1

-	In the case the previously selected NF Service instance belongs to a NF Service set (as indicated by the NF profile), one of the NF Service producer instances from the NF Service set may be selected by including the received resource identifier in the subsequent request towards this selected NF Service producer instance. The NF Service consumer should primarily select an NF service instance from the same location.

4.	The NF Service producer sends a response to the NF Service consumer. The NF Service producer may respond with an updated resource address different to the one received in the previous response.

#### 4.17.11.3	NF binding created as part of service request

There are two cases when an NF Service consumer sends a reference to a NF Service producer:

1)	The NF A as Service consumer sends its NF ID to the NF B as NF Service producer. E.g. AMF sends AMF ID and GUAMI to SMF. In this case when the NF B as a NF Service consumer wants to access a NF service in NF A, normal NF/NF service selection applies. For selection of a specific NF type, see TS 23.501 [2] subclauses under clause 6.3.

2)	A NF Consumer may add a notification endpoint in a request. In this case the NF Service producer shall use the notification endpoint when NF Service producer sends a Notification request to the NF Service consumer.

*** END CHANGES ***