3GPP TSG-SA WG2 Meeting #132	S2-1903401

8 – 12 April, 2019, Xi’an, China

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1197 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures and call flows for SMF Delegated Discovery and Selection in HR scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBA |  |  |  |  |  |  | Date: |  | 2019-03-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SMF discovery in a delegated mode in the HR scenario is not covered by the generic delegated discovery procedures described in 23.502 clauses 4.17.9 and 4.17.10. Consequently, a new specific clause 4.17.X has to be added to 23.502 4.17 in order to capture that. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause 4.17.X is added to capture the concepts and generic functionality required for addressing the scenario of SMF Discovery and Selection in a delegated mode for Home Routed. Two alternatives for performing the Delegated Discovery and Selection of SMF in the Home PLMN are defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedural description of SMF Discovery and Selection in a delegated mode for Home Routed is missing from 23.502. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.X (new), 4.17.X.1 (new), 4.17.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 4.17.X	Delegated SMF discovery in the Home Routed scenario

There are two alternatives for performing SMF discovery in the Home Routed scenario.

In the first alternative, the SCP in the Visited PLMN discovers and selects both the SMF in the Visited PLMN and the SMF in the Home PLMN. This alternative is referred to as Visited based Delegated Discovery of SMF in the HPLMN.

In the second alternative, the SCP in the Visited PLMN discovers and selects the SMF in the Visited PLMN, and then the SCP in the Home PLMN discovers and selects the SMF in the Home PLMN. This alternative is referred to as Home based Delegated Discovery of SMF in the HPLMN.

Both of the alternatives are described below. Which alternative is selected is based on operator configuration and policies.

#### 4.17.X.1 Visited based Delegated Discovery of SMF in the HPLMN

Figure 4.17.X.1: Visited based Delegated Discovery of SMF in the HPLMN

1. 	The AMF intends to communicate with an SMF. The AMF sends the Nsmf_PDUSession_CreateSMContext Request to an SCP. The request includes at least the source PLMN ID and the target PLMN ID in the discovery and selection parameters necessary for the SCP to discover and select an SMF instance in PLMN-1 and an SMF instance in PLMN-2. The discovery and selection parameters are included in the request by the AMF in a way that the SCP does not need to parse the request body. The SCP recognises that the request is for both PLMN-1 and PLMN-2.

2.	SCP intends to discover the SMF in PLMN-1 by interacting with NRF using the Nnrf_NFDiscovery service.

3.	SCP gets Nnrf_NFDiscovery service response with profile(s) of SMF.

4. 	The SCP selects an SMF instance in PLMN-1.

5.	SCP intends to discover the SMF in PLMN-2 by interacting with NRF using the Nnrf_NFDiscovery service. SCP includes in the Nnrf_NFDiscovery Request an indication of PLMN-2.

6.	NRF in PLMN-1 and NRF in PLMN 2 interact using the Nnrf_NFDiscovery service. See step 2 in clause 4.17.5.

7.	SCP gets Nnrf_NFDiscovery service response with profile(s) of SMF in PLMN-2.

8.	SCP selects an SMF instance in PLMN-2.

9.	SCP forwards the Nsmf_PDUSession_CreateSMContext Request to the selected SMF instance in PLMN-1. The request includes at least the ID/address information of the SMF in PLMN-2.

10. The SMF instance in PLMN-1 forwards the Nsmf_PDUSession_CreateSMContext Request to the selected SMF instance in PLMN-2.

#### 4.17.X.2 Home based Delegated Discovery of SMF in the HPLMN

Figure 4.17.X.2: Home based Delegated Discovery of SMF in the HPLMN

1. 	The AMF intends to communicate with an SMF. The AMF sends the Nsmf_PDUSession_CreateSMContext Request to an SCP. The request includes at least the source PLMN ID and the target PLMN ID in the discovery and selection parameters necessary for the SCP to discover and select an SMF instance in PLMN-1, as well as information necessary to assist the SCP in PLMN-2 to discover and select an SMF instance in PLMN-2. The discovery and selection parameters are included in the request by the AMF in a way that the SCP does not need to parse the request body.

2.	SCP intends to discover the SMF by interacting with NRF using the Nnrf_NFDiscovery service.

3.	SCP gets Nnrf_NFDiscovery service response with profile(s) of SMF.

4. 	The SCP selects an SMF instance.

5.	SCP forwards the Nsmf_PDUSession_CreateSMContext Request to the selected SMF instance. The request includes at least the ID/address information of the NRF in PLMN-2.

6. 	The SMF instance forwards the Nsmf_PDUSession_CreateSMContext Request to SCP in PLMN-2.

7. 	SCP in PLMN-2 intends to discover the SMF by interacting with NRF using the Nnrf_NFDiscovery service.

8.	SCP gets Nnrf_NFDiscovery service response with profile(s) of SMF.

9. 	The SCP selects an SMF instance.

### 10. SCP forwards the Nsmf_PDUSession_CreateSMContext Request to the selected SMF instance.

* * * * End of Changes * * * *