3GPP TSG-SA2 Meeting #132	S2-1903569

# Xi'an, China, 8th Apr 2019 - 12th Apr 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1234 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding NIDD SMF-NEF Connection Release service and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, UANGEL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current text of TS 23.502 does not cover how to release the SMF-NEF connection established for NIDD support. This CR proposes to add SMF-NEF Connection Release procedures and the corresponding services for NIDD support which may be initiated by SMF or NEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 4.25.y, added a new procedure for SMF initiated SMF-NEF connection release - In clause 4.25.z, added a new procedure for NEF initiated SMF-NEF connection release based on a request from AF and NIDD authorization update. - In clause 5.2.6.1, 5.2.6.13.x, and 5.2.6.15.x, added new NEF service operations for SMF-NEF connection release procedures: Nnef_NIDDConfiguration_Delete (Request/Response), Nnef_SMContext_Delete (Request/Response) - In clause 5.2.8.1, added NEF as an example consumer of Nsmf_PDUSession_ReleaseSMContext |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification because the case of SMF-NEF connection release initiated by SMF or NEF cannot be handled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.25.y, 4.25.z, 5.2.6.1, 5.2.6.13.x, 5.2.6.15.x, 5.2.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE

4.25.y	SMF Initiated SMF-NEF Connection Release procedure

When the UE perform the PDU Session release with PDU Session type of “Unstructured” and the subscription information corresponding to the UE requested DNN includes the "Invoke NEF Selection" flag, then the SMF initiates a SMF-NEF Connection Release procedure towards the NEF corresponding to the "NEF ID" for that DNN / S-NSSAI Combination.

Figure 4.25.y-1: SMF Initiated SMF-NEF Connection Release procedure

1.	The UE performs steps 1-2 of PDU Session Relese Procedure (see clause 4.3.4.2).

2.	If a NEF has been selected as anchor of the Control Plane CIoT 5GS Optimisation enabled PDU session which is Unstructured PDU Session Type as descrbied in clause 4.3.2.2, the SMF initiates the SMF-NEF Connection release for this PDU Session by sending Nnef_SMContext_Delete Request (User Identity, PDU session ID, NEF ID, S-NSSAI, DNN) message to the NEF.

3.	The NEF deletes the NEF PDU session Context associated with the User Identity and the PDU session ID. The NEF sends Nnef_SMContext_Delete Response (User Identity, PDU session ID, NEF ID, S-NSSAI, DNN) to the SMF confirming release of the SMF-NEF session for the UE.

4.25.z	NEF Initiated SMF-NEF Connection Release procedure

NEF initiates a SMF-NEF connection release procedure in the following cases:

-	when an NIDD Authorization Update Request from the UDM indicates that the User is no longer authorized for NIDD, or

-	failure of AF or failure of AF connection, or

-	based on a request from the AF.

The Figure 4.25.z-1 illustrates the NEF Initiated SMF-NEF Connection Release procedure based on a request from AF.

Figure 4.25.z-1: NEF Initiated SMF-NEF Connection Release procedure on the AF request

1.	AF may indicate that a User's NIDD SMF-NEF connection is no longer needed by invoking Nnef_NIDDConfiguration_Delete Request (TLTRI) toward NEF.

2.	The NEF deletes the NEF PDU session Context associated with the TLTRI. The NEF sends Nsmf_PDUSession_ReleaseSMContext Request (SUPI, PDU session ID) to the SMF to request the release of the PDU session.

3.	The SMF performs steps 2-11 of PDU Session Relese Procedure (see clause 4.3.4.2).

4.	The SMF sends Nsmf_PDUSession_ReleaseSMContext Response to the NEF confirming release of the PDU session.

5.	The NEF sends Nnef_NIDDConfiguration_Delete Response to the AF confirming release of the SMF-NEF session.

The Figure 4.25.z-2 illustrates the NEF Initiated SMF-NEF Connection Release procedure based on the NIDD Authorization Update.

Figure 4.25.z-2: NEF Initiated SMF-NEF Connection Release procedure on the NIDD Authorization Update

- On NIDD Authorization Update by UDM, NEF may determine that it needs to release the corresponding SMF-NEF Connection.

2.	The NEF sends Nsmf_PDUSession_ReleaseSMContext Request (SUPI, PDU session ID) to the SMF to request the release of the PDU session.

3.	The SMF performs steps 2-11 of PDU Session Relese Procedure (see clause 4.3.4.2).

4.	The SMF sends Nsmf_PDUSession_ReleaseSMContext Response to the NEF confirming release of the PDU session.

2nd CHANGE

5.2.6	NEF Services

5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | SMF |
|  | Trigger | Trigger | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF, SMF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request / Response | AF |

3rd CHANGE

5.2.6.13	Nnef_NIDDConfiguration service

5.2.6.13.1	General

See clause 4.25.3.

5.2.6.13.2	Nnef_NIDDConfiguration_Create service operation

Service operation name: Nnef_NIDDConfiguration_Create

Description: This service operation is used by the consumer to request NIDD Configuration between NF consumer and NEF to support NIDD via NEF.

Inputs (required): GPSI or External Group Identifier, AF ID, NIDD Duration, T8 Destination Address, MTC Provider Information).

Outputs (required): TLTRI, Cause.

5.2.6.13.3	Nnef_NIDDConfiguration_Trigger service operation

Service operation name: Nnef_NIDDConfiguration_Trigger

Description: NEF triggers NIDD Configuration Create Rquest if there is no NIDD Configuration between NF consumer and NEF.

Inputs (required): GPSI, AF ID, NEF ID.

Outputs (required): None.

5.2.6.13.x	Nnef_NIDDConfiguration_Delete service operation

Service operation name: Nnef_NIDDConfiguration_Delete

Description: This service operation is used by the NF consumer to request NIDD Configuration delete between NF consumer and NEF to support NIDD via NEF.

Inputs (required): TLTRI.

Outputs (required): Cause.

4th CHANGE

5.2.6.15	Nnef_SMContext service

5.2.6.15.1	General

The service provides the capability to create, update or release the SMF-NEF Connection. See clause 4.25.2 for the detailed procedure.

5.2.6.15.2	Nnef_SMContext_Create service operation

Service operation name: Nnef_ SMContext_Create

Description: This service operation is used by the consumer to request connection establishment between NF consumer and NEF to support NIDD via NEF.

Inputs (required): User Identity, PDU session ID, NEF ID, NIDD information, S-NSSAI, DNN.

Outputs (required): User Identity, PDU session ID, NEF ID, S-NSSAI, DNN.

5.2.6.15.x	Nnef_SMContext_Delete service operation

Service operation name: Nnef_ SMContext_Delete

Description: This service operation is used by the NF consumer to request SMF-NEF Connection release between NF consumer and NEF to support NIDD via NEF.

Inputs (required): User Identity, PDU session ID, NEF ID, S-NSSAI, DNN.

Outputs (required): Cause.

5th CHANGE

5.2.8	SMF Services

5.2.8.1	General

The following table shows the SMF Services and SMF Service Operations.

Table 5.2.8.1-1: NF services provided by the SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsmf_PDUSession | Create | Request/Response | V-SMF/I-SMF |
|  | Update | Request/Response | V-SMF/I-SMF, H-SMF |
|  | Release | Request/Response | V-SMF/I-SMF |
|  | CreateSMContext | Request/Response | AMF |
|  | UpdateSMContext | Request/Response | AMF |
|  | ReleaseSMContext | Request/Response | AMF, NEF |
|  | SMContextStatusNotify | Subscribe/Notify | AMF |
|  | StatusNotify | Subscribe/Notify | V-SMF/I-SMF |
|  | Context | Request/Response | AMF, I-SMF |
| Nsmf_EventExposure | Subscribe | Subscribe/Notify | NEF, AMF |
|  | Unsubscribe |  | NEF, AMF |
|  | Notify |  | NEF, AMF |
|  | AppRelocationInfo |  | AF |

END OF CHANGES