3GPP TSG-SA WG2 Meeting #132 	S2-1904282

# Xi’an, China, 8th Apr – 12th Apr, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1178 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SSC mode 2/3 PDU Session in ETSUN scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the TS, it says: When the SMF cannot control the UPF terminating the N3 Interface used by a PDU Session and SSC mode 2/3 procedures are not applied to the PDU Session, an I-SMF is inserted between the SMF and the AMF However the I-SMF is selected by the AMF, so does this means that AMF need be aware which SSC mode the PDU Session is? As the SSC mode 2/3 is for path optimization or no service continuity support due to deployment restriction, it is not need to be linked with the I-SMF insertion. It is proposed to remove the limition that the I-SMF is not inserted between SMF and AMF for PDU Session with SSC mode 2/3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the limitation that the I-SMF is not inserted for PDU Session with SSC mode 2/3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMF is required to be aware the SSC mode of the PDU Session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.34	Support of deployments topologies with specific SMF Service Areas

5.34.1	General

The Control Plane of a PDU Session involves an I-SMF when the SMF cannot control the UPF terminating the N3 Interface used by a PDU Session. In this case, an I-SMF is inserted between the SMF and the AMF. The I-SMF has a N11 interface with the AMF and a N16a interface with the SMF and is responsible of controlling the UPF(s) that the SMF cannot directly control. The exchang of the SM context and forwarding tunnel information if needed are interacted between two SMFs directly without involvement of AMF.

NOTE: 	The insertion of an I-SMF is to preserve the connectivity service. However for SSC mode 2/3 this connectivity service preservation is not needed. When SSC mode 2/3 is applicable for a PDU Session, even if an I-SMF has been inserted, the SMF can still release the PDU Session if necessary.

Depending on scenario, a PDU Session in non-roaming case or local breakout is either served by a single SMF or served by an SMF and an I-SMF. When a PDU Session is served by both an SMF and an I-SMF, the SMF is the NF instance that has the interfaces towards the PCF and CHF.

END OF CHANGES