SA WG2 Meeting #132	S2-1904305

April 8 – 12, 2019, Xi’an, China	(was S2-1903597)

Source:	CATT, Samsung

Title:	GMLC Services

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	5G_eLCS / Rel-16

Abstract of the contribution: This paper proposes to add the description on GMLC services.

1.	Discussion

The following GMLC services are introduced in the current procedures in TS 23.273:

-	Ngmlc_ProvideLocation Request/ Response in clauses 6.1.2 and 6.5.1.

-	Ngmlc_LocationUpdateNotify Request/ Response in clause 6. 2.

-	Ngmlc_EventNotify in clauses 6.3.1 and 6.3.2.

-	Ngmlc_CancelLocation Request/ Response in clause 6.3.3.

This P-CR is proposed to add the description on above GMLC services in clause 8.4.

2.	Proposal

It is proposed to update TS 23.273 as follows.

*** First change***

8.4	GMLC Services

8.4.1	General

The following table shows the GMLC Services and GMLC Service Operations.

Table 8.4.1-1: List of GMLC Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ngmlc_ Location | ProvideLocation | Request / Response | GMLC, NEF |
|  | LocationUpdateNotify | Request / Response | AMF, GMLC |
|  | CancellLocation | Request / Response | GMLC, NEF |
|  | EventNotify | Notify | GMLC, NEF |

8.4.2	Ngmlc_Location service

8.4.2.1	General

Service description: This service enables an NF to request location determination for a target UE. The following are the key functionalities of this NF service.

-	Allow the consumer NF to request the current geodetic and optionally civic location of a target UE.

-	Allow the consumer NF to subscribe/unsubscribe the geodetic and optionally civic location of a target UE for some certain events.

-	Allow the consumer NF to get notified about the geodetic and optionally civic location of a target UE when some certain events are detected.

8.4.2.2	Ngmlc_Location_ProvideLocation service operation

Service operation name: Ngmlc_Location_ProvideLocation

Description: Provides UE location information to the consumer NF.

NOTE: For deferred location request, this service operation is used to implicitly subscribe to the notification of the UE location information.

Input, Required: UE identifier (GPSI or SUPI).

Input, Optional: Required QoS, Supported GAD shapes, Client Type, Event Type (defined in clause 4.1a.5.1), and

-	For periodic event type, optional input further includes the time interval between successive location reports, the total number of reports, location QoS.

-	For area event type, optional input further includes target area, whether the event to be reported is the UE being inside, entering into or leaving the target area, the duration of event reporting, the minimum and maximum time intervals between successive event reports, the maximum event sampling interval, whether location estimates and associated location QoS shall be included in event reports, and whether only one location report is required or more than one.

-	For motion event type, optional input further includes the threshold linear distance, the duration of event reporting, the minimum and maximum time intervals between successive event reports, the maximum event sampling interval, whether location estimates and associated location QoS shall be included in event reports, and whether only one location report is required or more than one.

Output, Required: Success/Failure indication

Output, Optional: Geodetic location, civic location, age of location, location QoS, position methods used (in the case of success indication provided), failure cause (in the case of failure indication provided).

See clause 6.3.1 for example of usage of this service operation.

8.4.2.3	Ngmlc_Location_LocationUpdateNotify service operation

Service operation name: Ngmlc_Location_LocationUpdateNotify

Description: Provides UE location information to the consumer NF.

Input, Required: UE identifier (GPSI or SUPI), event causing the location estimate (5GC-MO LR), location estimate, age of location estimate, accuracy indication, LCS QoS class.

Input, Optional: pseudonym indicator, identity of the LCS client, GMLC address, service identity specified by UE.

Output, Required: Success/Failure indication

Output, Optional: Failure Cause (in the case of failure indication provided).

See clause 6.2 for example of usage of this service operation.

8.4.2.4	Ngmlc_Location_EventNotify service operation

Service operation name: Ngmlc_Location_EventNotify

Description: Allow the consumer NF to get notified about the geodetic and optionally civic location of a target UE when some certain events are detected.

Input, Required: LDR reference number, UE (SUPI and if available GPSI).

Input, Optional: Geodetic Location, Civic Location, Position Methods Used (in the case of success indication provided), Failure Cause (in the case of failure indication provided).

Output, Required: None.

Output, Optional: None.

See clause 6.3 for example of usage of this service operation.

8.4.2.5	Ngmlc_Location_CancelLocation service operation

Service operation name: Ngmlc_Location_CancelLocation

Description: The consumer NF uses this service operation to cancel a deferred 5GC-MT-LR procedure for periodic or triggered location.

Input, Required: LDR reference number.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

See clause 6.3.3 for example of usage of this service operation.

*** 2nd  change ***

5.x	GMLC Discovery and Selection

Editor's note:	GMLC Discovery and Selection is FFS.

*** End of changes ***