3GPP TSG-SA WG2 Meeting #132	S2-1904774

# Xi'an, China, 8 - 12 April, 2019 	(revision of S2-1904600)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1051 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Proposed update to 5G LAN terminology |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The term “5G LAN”, “5G LAN group” etc is misleading since the term LAN (Local Area Network) usually denotes a layer 2 network based on Ethernet or some other IEEE 802 technology. In fact IEEE defines LAN as “LAN: A computer network, located on a user’s premises, within a limited geographical area.” The 5G LAN work in 3GPP however covers both Ethernet and IP, as well as local and less local networks (stage 1 requirements include e.g. that “5G LAN-VN may span multiple countries”). IETF e.g. talks about Layer 2 Virtual Private Networks (L2 VPN) and Layer 3 VPNs, i.e. using a more general terminlogy. Stage 1 does not talk about “5G LAN groups” or “5G LAN” as such. Stage 1 uses the terms “5G LAN-type service” and “5G LAN-virtual network”, indicating that a 5G LAN is indeed a Virtual Network: 5G LAN-type service: a service over the 5G system offering private communication using IP and/or non-, i.e., UEs that are members of the same 5G LAN-VN IP type communications. 5G LAN-virtual network: a virtual network capable of supporting 5G LAN-type service. It is also clear that stage 1 scope if bigger than “LANs”, covering both LAN and more general VPNs: “5G will need to provide services with similar functionalities to Local Area Networks (LANs) and VPN’s”. The VPN part is however missing in the SA2 descriptions. It is therefore proposed to make the terminology in SA2 specifications more general, either by using the same terms as used in stage 1, i.e. “5G LAN VN” and “5G LAN type services”, or more preferrably to avoid the “LAN” term completely and only talk about “5G Virtual Networks” (5G VN). If this is agreed, SA2 could inform SA1 about the decisions and recommend a terminology alignmment. In addition, clause 5.25.2 contains references to clauses in 23.502 that does not seem suitable (one of the clauses does not even contain any 5G LAN related description). Better references are available in 23.501. Also, clause 5.25.2 indicates that UDM provides subscription data to AMF, SMF and PCF. This is incorrect since only AMF and SMF use subscription data provided by UDM |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change temrinology from “5G LAN” to “5G VN” Remove incorrect references and remove PCF from the text where subscription data is provided by UDM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Terminology that may be misinterpreted by community outside 3GPP Incorrect references and subscription data handling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.1, 5.28.3, 5.28.4, 5.8.2.11.3, 5.8.2.11.6, 5.8.2.13, 5.25.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change *****

### 5.28.1	General

The service requirements for 5G LAN-type service are specified in TS 22.261 [2].

A 5G Virtual Network (VN)  group consists of a a set of UEs using private communication for 5G LAN-type

**** Next Change ****

### 5.28.3	PDU Session management

Session management as defined for 5GS in clause 5.6 is applicable to 5G-LAN-type services with the following clarifications:

-	A UE gets access to 5G LAN-type services via a PDU Session of IP PDU Session type or Ethernet PDU Session type.

-	A PDU Session provides access to one and only one 5G VN group

-	A dedicated SMF is responsible for all the PDU Sessions for communication of a certain 5G VN group.

Editor's note:	Whether and how the above bullet needs to be updated due to eSBA is FFS.

Editor's note:	Whether there are impacts to SMF selection to select this dedicated SMF is FFS.

NOTE:	Having a dedicated SMF serving a 5G VN does not contradict that redundancy solutions can be used to achieve high availability.

-	A DNN is associated with a 5G- VN group

Editor's note:	The above bullet is FFS.

-	The UE provides a DNN associated with the 5G VN group to access the 5G LAN-type services for that 5G VN, using the PDU Session Establishment procedure described in TS 23.502 [3], clause 4.3.2

-	During establishment of the PDU Session, secondary authentication as described in clause 5.6.6 and in TS 23.502 [3], clause 4.3.2.3, may be performed in order to authenticate and authorize the UE for accessing the DNN associated with the 5G VN group. Authentication and authorization for a DNN using secondary authentication implies authentication and authorization for the associated 5G VN group. There is no 5G VN group specific authentication or authorization defined.

-	The SM level subscription data for a DNN and S-NSSAI available in UDM, as described in clause 5.6.1, applies to a DNN associated to a 5G VN group.

-	Session management related policy control for a DNN as described in TS 23.502 [3], is applicable to a DNN associated to a 5G VN group. This includes also usage of URSP, for the UE to determine how to route outgoing traffic to a PDU Session for a DNN associated to a 5G VN group.

**** Next Change ****

### 5.28.4	User Plane handling

User Plane management as defined for 5GS in clause 5.8 is applicable to 5G-LAN-type services with the following clarifications:

-	There are three types of traffic forwarding methods for 5G VN communication:

-	N6-based, where the UL/DL traffic for the 5G VN communication is forwarded to/from the DN;

-	Nx-based, where the UL/DL traffic for the 5G VN communication is forwarded between PSA UPFs of different PDU sessions via Nx.

-	Local switch, where traffic is locally forwarded by a single UPF if this UPF is the common PSA UPF of different PDU Sessions for the same 5G VNgroup.

-	The SMF handles the user plane paths of the 5G VN group, including:

-	Establishing Nx tunnels between PSA UPFs to support Nx-based traffic forwarding.

-	For Ethernet PDU Session, the SMF may instruct the UPF(s) classify frames based on VLAN tags, and to add and remove VLAN tags, on frames received and sent on N6, as described in clause 5.6.10.2.

NOTE:	For handling VLAN tags to traffic on N6, TSP ID could also be used as described in clause 6.2.2.6 of TS 23.503 [45].

Further description on User Plane management for 5G VNs groups is available in clause 5.8.2.13.

Editor's note:	Whether PCF can indicate to SMF the allowed traffic forwarding methods for a 5G-LAN group is FFS.

**** Next Change ****

##### 5.8.2.11.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.2.11.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR |  |
| Rule ID |  | Unique identifier to identify this rule |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied |  |
| Packet | Source interface | Contains the values "access side", "core side", "SMF", "N6-LAN" , "5G VN internal", "5G VN Nx" | Combination of UE IP address (together with Network instance, if necessary), CN tunnel info, |
| detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length (NOTE 3) | packet filter set, application ID, Ethernet PDU Session |
| information | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet | Information and QFI are used for traffic detection. Source interface identifies the |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID | interface for incoming packets |
|  | Packet Filter Set | Details see clause 5.7.6, TS 23.501. | where the PDR applies, e.g. from access side (i.e. up-link), |
|  | Application ID |  | from core side (i.e. down-link), |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI | from SMF, from N6-LAN (i.e. the |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. | DN or the local DN), from "5G VN internal" (i.e. local switch), or from "5G VN Nx" (i.e. Nx interface). |
|  | Framed Route Information | Refers to Framed Routes defined in clause 5.6.14 | Details like all the combination possibilities on N3, N9 interfaces are left for stage 3 decision. |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains. - UPF "local switch" and Nx forwarding is used for different 5G LAN groups. NOTE 2: Either a FAR ID or a MAR ID is included, not both. NOTE 3: The SMF may provide an indication asking the UPF to allocate one IPv4 address and/or IPv6 prefix. When asking to provide an IPv6 Prefix the SMF provides also an IPv6 prefix length. |  |  |  |

**** Next Change ****

##### 5.8.2.11.6	Forwarding Action Rule

The following table describes the Forwarding Action Rule (FAR) that defines how a packet shall be buffered, dropped or forwarded, including packet encapsulation/decapsulation and forwarding destination.

Table 5.8.2.11.6-1: Attributes within Forwarding Action Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this FAR. |  |
| Rule ID | Unique identifier to identify this information. |  |
| Action | Identifies the action to apply to the packet | Indicates whether the packet is to be forwarded, duplicated, dropped or buffered. When action indicates forwarding or duplicating, a number of additional attributes are included in the FAR. For buffering action, a Buffer Action Rule is also included. |
| Network instance (NOTE 2) | Identifies the Network instance associated with the outgoing packet (NOTE 1). |  |
| Destination interface (NOTE 3) (NOTE 7) | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal" or "5G VN Nx". | Identifies the interface for outgoing packets towards the access side (i.e. down-link), the core side (i.e. up-link), the SMF, the N6-LAN (i.e. the DN or the local DN), to 5G VN internal (i.e. local switch), or to 5G VN Nx (i.e. Nx interface). |
| Outer header creation (NOTE 3) | Instructs the UP function to add an outer header (e.g. IP+UDP+GTP+QFI, VLAN tag), IP + possibly UDP to the outgoing packet. | Contains the CN tunnel info, N6 tunnel info or AN tunnel info of peer entity (e.g. NG-RAN, another UPF, SMF, local access to a DN represented by a DNAI). Any extension header stored for this packet shall be added. |
| Send end marker packet(s) (NOTE 2) | Instructs the UPF to construct end marker packet(s) and send them out as described in clause 5.8.1. | This parameter should be sent together with the "outer header creation" parameter of the new CN tunnel info. |
| Transport level marking (NOTE 3) | Transport level packet marking in the uplink and downlink, e.g. setting the DiffServ Code Point. |  |
| Forwarding policy (NOTE 3) | Reference to a preconfigured traffic steering policy or http redirection (NOTE 4). | Contains one of the following policies identified by a TSP ID: - an N6-LAN steering policy to steer the subscriber's traffic to the appropriate N6 service functions deployed by the operator, or - a local N6 steering policy to enable traffic steering in the local access to the DN according to the routing information provided by an AF as described in clause 5.6.7. or a Redirect Destination and values for the forwarding behaviour (always, after measurement report (for termination action "redirect")). |
| Request for Proxying in UPF | Indicates that the UPF shall perform ARP proxying and / or IPv6 Neighbour Solicitation Proxying as specified in clause 5.6.10.2. | Applies to the Ethernet PDU Session type. |
| Container for header enrichment (NOTE 2) | Contains information to be used by the UPF for header enrichment. | Only relevant for the uplink direction. |
| Buffering Action Rule (NOTE 5) | Reference to a Buffering Action Rule ID defining the buffering instructions to be applied by the UPF (NOTE 6) |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or NG-RAN node in different IP domains; - UPF "local switch" and Nx forwarding is used for different 5G LAN groups. NOTE 2: These attributes are required for FAR action set to forwarding. NOTE 3: These attributes are required for FAR action set to forwarding or duplicating. NOTE 4: The TSP ID is preconfigured in the SMF, and included in the FAR according to the description in clauses 5.6.7 and 6.1.3.14 of 23.503 [45] for local N6 steering and 6.1.3.14 of 23.503 [45] for N6-LAN steering. The TSP ID action is enforced before the Outer header creation actions. NOTE 5: This attribute is present for FAR action set to buffering. NOTE 6: The buffering action rule is created by the SMF and associated with the FAR in order to apply a specific buffering behaviour for DL packets requested to be buffered, as described in clause 5.8.3 and clause 5.2.4 in TS 29.244 [65]. NOTE 7: The use of "5G VN Internal" instructs the UPF to send the packet back for another round of ingress processing using the active PDRs pertaining to another N4 session of the same 5G VN. To avoid that the packet matches again the same PDR, it is assumed that the packet has been modified in some way (e.g. tunnel header has been removed, the packets being received from the source interface 5G VN Internal of the same 5G VN). |  |  |

**** Next Change ****

#### 5.8.2.13	Support for 5G VN group communication

In addition to N6-based forwarding of user plane traffic for 5G VN group communication, the SMF and UPF may support local switch and Nx-based forwarding:

-	Local switch, where traffic is locally forwarded by a single UPF if this UPF is the common PSA UPF of different PDU Sessions for the same 5G VN group;

-	Nx-based, where the UL/DL traffic for the 5G VN communication is forwarded between PSA UPFs of different PDU sessions via Nx. Nx is based on a shared User Plane tunnel connecting PSA UPFs of a single 5G LAN group.

Editor's note:	The way to distinguish the traffic applying N6-based forwarding and traffic applying local switch/Nx-based forwarding in a PDU Session is FFS.

The SMF may configure the UPF(s) to apply different traffic forwarding methods to route traffic between PDU Sessions for a single 5G VN group. For example, depending on the destination address, some packet flows may be forwarded locally, while other packet flows are forwarded via Nx and other packet flows are forwarded to N6.

The local switch and Nx-based forwarding methods require that a common SMF is controlling the PSA UPFs for the 5G-LAN group.

Editor's note:	The above text may be updated based on eSBA outcome.

To enable local switch in a UPF, the following applies:

-	The SMF provides FARs for 5G VN group members' N4 Sessions (i.e. N4 Session corresponding to PDU Session) with FAR(s) with Destination Interface set to "5G VN internal" and Network Instance set to represent the 5G VN group in order to instruct the UPF to again perform classification of packets using the corresponding 5G VN group relevant PDRs.

-	The SMF also provides PDRs for 5G VN group members' N4 Session (i.e. N4 session corresponding to PDU Session) with PDR(s) with Source Interface set to "5G VN internal" and Network Instance set to represent the 5G VN group in order to instruct the UPF to detect packets that have been sent to classification via the local switch. This is done by configuring the PDRs in the target 5G VN group members' N4 Session to includes the destination IP/MAC address(es) of these UEs.

To enable Nx-based forwarding in a UPF, the following applies:

-	The SMF provides FARs for 5G VN group members' N4 Sessions (i.e. N4 Session corresponding to PDU Session) with FAR(s) with Destination Interface set to "5G VN Nx" and Network Instance set to represent the 5G VN group in order to instruct the UPF to again perform classification of packets using the corresponding 5G VN group relevant PDRs.

-	SMF creates a group-level N4 session to each involved UPF for forwarding packets to a Nx tunnel. The SMF optionally configures the group-level N4 session for processing packets received from a Nx tunnel. Such a N4 session is created per UPF and 5G VN group (i.e. a group-level N4 session not corresponding to a PDU Session).

-	The SMF provides relevant N4 rules (e.g. PDRs, QERs, URSs and FARs) for the group-level N4 session applicable for the Nx tunnel(s) in the UPF. The SMF may provides PDRs for the group-level N4 session with Source Interface set to "5G VN Nx" and Network Instance set to represent the 5G VN group in order to instruct the UPF to detect packets that are targeting the Nx-based forwarding. In case of multiple Nx tunnels between different PSA UPFs, the SMF may provide N4 rules corresponding to the different Nx tunnels in a single group-level N4 Session.

-	To match the traffic of the UE(s) whose traffic should be forwarded across the Nx tunnel, the PDRs of the group-level N4 Session includes the destination IP/MAC address(es) of these UEs.

-	The SMF may proactively update N4 rules for group level N4-Session to enable correct routing of packets towards UE whose PSA UPF has been reallocated.

-	The SMF may also configure a default PDR in the group-level N4 Session to capture the packets pertaining to 5G VN group communication with an unknown destination address and a URR to trigger reporting of such events. This enables the SMF to reactively configure relevant PDR and FAR to forward the packets to another UPF via Nx, based on the reports from UPF.

Editor's note:	Whether N4 rules (URRs and FARs) for the group-level N4 session applicable to the traffic applying the local switch/N6-based routing is FFS.

Editor's note:	Handling of multicast traffic with local switch and Nx-based forwarding is FFS.

Editor's note:	Whether there is any impact on MAC address learning in SMF/UPF and the ARP/ND proxying features is FFS. Whether and how to support "local switch" without specific SMF involvement, similar to MAC address learning in UPF, is FFS.

**** Next Changes ****

### 5.25.2	Support of OAM-based 5G VN group management

5GS supports 5G LAN-type service as defined in clause 5.28. 5G LAN-type service includes the 5G VN group management that can be configured by a network administrator.

The parameters for 5G VN group is defined in clause 5.28.

The 5G VN group parameters about a UE may be configured in subscription data of the UE and delivered together with other subscription data by the UDM towards the AMF, SMF.

**** End of Changes ****