3GPP TSG-SA2 Meeting #132	S2-1903004

# April 8 - 12, 2019, Xi’an, China

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.734 | CR | 0018 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  |  | UICC apps |  |  | ME |  | X | Radio Access Network |  |  |  |  | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Conclusion for key issue TSN Time Synchronization |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  |  |  |  |  |  | Date: |  | 2019-03-27 |  |  |  |
| Category: | C |  |  |  |  |  |  |  |  |  |  | Release: |  | Rel-16 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explan7ations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |  |  |
| Reason for change: |  | For Key issue #3.2: Time synchronization aspect, there are 6 options, which can be categorized into 2 classes. - Class1) Conveying timing to the UE: Solution #11 Option 2 ; - Class2) Transparently conveying external PTP message(802.1AS message) in the user plane to the UE and Assuming all 5GS node are synchronized based on the internal 5GS GM : Solution #11 Option 1 ; Solution #11 Option 3 ; Solution #11 Option 4 ; Solution #19 ; Solution #28. Solution#19 and Solution #28.1 are not preferred by RAN. Method 2 in Solution#28.2 is as the same as solution#19, so it is not preferred as well. The propagation delay calculated via the 802.1AS may not be correct as the Uu interface transmission delay between the uplink and the downlink can be different at any point of time. The previous calculated propagation delay cannot be used to correct the time information in the subsequent PTP message, as the Uu interface transmission delay for each PDU could be changed at any point of time. So Solution#11 option1, solution#11 option3, and Method 2 in Solution#28.2 are not preferred. Solution#28.2 can only be applied for (g)PTP and the other external synchronization protocols cannot be applied and Solution #11 Option 2 can be applied to mutiple external synchronization protocol, e.g. (g)PTP(802.1AS), GPS between the NG RAN and external GM. By now, RAN has agreed a WID(RP-190728) : Support of NR Industrial Internet of Things (IoT)> to specify accurate reference timing delivery from gNB to UE using broadcast and unicast RRC signalling (with EUTRA Rel-15 signalling solution as baseline) for synchronization requirements defined in TS 22.104) [RAN2]. It seems Solution#11option2 has already been captured in the RAN scope. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | To enable support for TSN time synchronization, it is recommended to select solution11#2 as the basis for normative work. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Key issue #3.2: Time synchronization aspect is not concluded. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 8.X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

8.X	Conclusion for key issue 3.2

To enable support for TSN time synchronization, it is recommended to select solution11#2 as the basis for normative work.

* * * * End of Changes * * * *