SA WG2 Meeting #132 	S2-1903103

# 8 – 12 Feb 2019, Xian, P.R. China											 (revision of S2-1903103)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1049 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 5G LAN management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-03-24 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 5.29.2 5G LAN group management is sometimes unclear about parameter mapping |  |  |  |  |  |  |  |  |
| Summary of change: |  | ### Clarifications done within 5.29.2 5G LAN group management |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccurate and incomplete 5G LAN group management |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29.2 ; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.29.2	5G LAN group management

5G System supports 5G LAN Group Management. 5G LAN groups can be configured by a network administrator (OA&M) or can be managed dynamically by an AF (via the NEF).

The group as described in clause 5.2.3.3.1 of TS 23.502 [3] is applicable to 5G LAN-type services.

In order to support dynamic 5GLAN Group Management, the NEF exposes a set of service operations to manage (e.g. add/delete/modify) 5G LAN groups. This includes managing the list of 5G LAN group members and the type of service (IP or Ethernet). The AF identifies groups by their External Group Identifier and group members by their GPSI.

The AF may update the list of members (GPSI) of a 5G LAN group at any time after the initial provisioning. The type of service (IP or Ethernet) cannot be changed once the group has been created.

The 5G LAN group configuration is stored in the UDR.

When configuration is provided by an AF, the NEF service operations information flow procedure described in TS 23.502 [3] clause 4.15.6.2 applies. The 5G LAN group configuration in UDR may include the following parameters: GPSI (i.e. UE Identities of the 5G LAN group), PDU session type, DNN, S-NSSAI. The NEF determines the (DNN, S-NSSAI) based on information received from the AF and on local configuration. The UDM updates the individual subscription of SUPI(s) accordingly to the operations done on the groups the SUPI start to belong to or ceases to belong to.

Editor's note:	Relation between DNN and 5G LAN group, e.g. whether and how a 1:1 and/or 1:N relation is supported, is FFS.

The UE is configured with information corresponding to the group using the same mechanisms than used for a Session Management Subscription data update. The PCF delivers 5G LAN group configuration information (DNN, S-NSSAI, PDU session type) to the UE for each GPSI that belongs to a 5G-LAN group. The 5G LAN group configuration information is delivered in the URSP from the PCF to the UE using the UE Configuration Update procedure for transparent UE Policy delivery as described in TS 23.502 [3] clause 4.2.4.3 and TS 23.503 [45] clause 6.1.2.2.

NEXT CHANGE

END OF CHANGES