SA WG2 Meeting #132	S2-1903599

April 8 – 12, 2019, Xi’an, China	(was S2-19----)

Source:	CATT

Title:	MO-LR Procedure when UE is served by the Different PLMNs via 3GPP Access and Non-3GPP Access

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	5G_eLCS / Rel-16

Abstract of the contribution: This paper proposes to solve the FFS in clause 6.9.3.

1.	Discussion

This pCR is proposed to solve the following editor’s note in clause 6.9.3 in TS 23.273:

Editor's note:	MO-LR Procedures when UE is served by the Different PLMNs via 3GPP Access and Non-3GPP Access depends on the progress of the study and FFS.

When UE is served by the different PLMNs via 3GPP access and non-3GPP access, the UE has to select one access to initiate the MO-LR procedure. It is proposed that local configuration or URSP rule is used by UE to select the access. The procedure is the same with the the 5GC-MO-LR Procedure in clause 6.2. If Non-3GPP access is selected, the following difference exists:

-	The NG-RAN in 5GC-MO-LR Procedure in clause 6.2 is corresponding to the N3IWF/TNGF;

-	Step 5 in 5GC-MO-LR Procedure in clause 6.2 is the same as step 3d or 3e in Common Positioning Procedure when UE is served by the same PLMN in clause 6.9.1.

2.	Proposal

It is proposed to update TS 23.273 as follows.

*** First change ***

6.9.3	MO-LR Procedures when UE is served by the Different PLMNs via 3GPP Access and Non-3GPP Access

When UE is served by the different PLMNs via 3GPP access and Non-3GPP access, UE uses either the URSP rules or the UE Local Configuration to determine the access to initiate MO-LR procedure which is the same as the 5GC-MO-LR Procedure in clause 6.2. If Non-3GPP access is selected, the following difference exists:

-	The NG-RAN in 5GC-MO-LR Procedure in clause 6.2 is corresponding to the N3IWF/TNGF;

-	Step 5 in 5GC-MO-LR Procedure in clause 6.2 is the same as step 3d or 3e in Common Positioning Procedure when UE is served by the same PLMN in clause 6.9.1.

*** End of change ***