3GPP TSG-SA WG2 Meeting #132	S2-1903975

08 – 12 April, 2019,  Xi’an, China

Source:                         Ericsson

Title:                              Update on MICO mode analytics

Document for:              Approval

Agenda Item:                6.11

Work Item / Release:    eNA / Rel-16

Abstract of the contribution: This contribution updates the MICO mode analytics.

# Discussion

In last meeting TS 23.501 was updated through contribution S2-1902084 (Use of NWDAF analytics for decision of MICO mode parameters), with “If NWDAF is deployed, the AMF may also use analytics UE behaviour and/or UE communication provided by NWDAF (see 3GPP TS 23.288 [xx]) to decide MICO mode parameters.”

It is thereby defined what type of analytics AMF will receive from NWDAF, namely UE behaviour and/or UE communication. 
It is unclear what is meant by distinguish between UE behaviour and UE communication. In TS 23.502, chapter 4.15.6.3 Expected UE Behaviour parameters the UE expected UE behaviour parameters are defined containing both Expected UE Moving Trajectory and Communication behaviour according to S2-1901282 (Expected UE Behaviour parameters for CIoT). UE communication is thereby a sub category of UE behaviour. If this is agreed, suggestion is to clean up the sentence in 23.501 by removing “and/or UE communication”. It will then become “If NWDAF is deployed, the AMF may also use analytics UE behaviour provided by NWDAF (see 3GPP TS 23.288 [xx]) to decide MICO mode parameters.”

Since the output type of analytics are now given for MICO Mode Parameter Optimization the Below is an attempt to remove unnecessary input parameters, that has no relevance for the generated output, nor any relevance for AMF.

The only subscription data from UDM seen needed for MICO mode analytics is Expected UE Behaviour parameters and Network Configuration parameters. If more subscriber data is needed, it shall be explicitly written. Therefore, following changes are suggested, until further explicit subscription parameters are described.

External provisioned parameters shall be renamed to Expected UE Behaviour parameters and Network Configuration parameters

The Description for this row in the table shall be updated to:

“To extract the provisioned parameters such as Network Configuration parameters and the provisioned Expected UE Behaviour parameters. If in the process new parameters are derived they may be stored via External provisioning into UDR via UDM.

The row for subscription Data shall be removed.

# Proposal

Following updates shall be done for MICO Mode Parameter Optimization.

**** Start of Change ****

## 6.9	MICO Mode Parameter Optimization for mIoT Terminals

### 6.9.1	General

To enhance the mIoT terminal's battery lifetime, the proper configuration of MICO mode related parameters are required. The NWDAF may provide analytics information to the AMF to assist the AMF to decide configuration parameters for MICO mode such as 1) Allow/disallow MICO mode, 2) periodic registration timer, 3) Active Time and Connected Time.

Editor's note:	The exact parameters for MICO mode configuration may be further updated based on CIoT conclusion.

Depending on the decision algorithm in the AMF, the AMF may utilize the external parameters provisioned by AFs such as the behavioural pattern and the communication pattern in TS 23.502 [3]. The AMF may request the NWDAF to include recommended values for any missing external parameters in the analytics information.

### 6.9.2	Input Data

The information to be collected by the NWDAF for generation of analytic data related to MICO mode parameter optimization can be classified into three groups, 1) network data from NFs, 2) service data from AFs, and 3) management data from the OAM.

The network data to collect from NFs in the 5GC is defined in Table 6.9.2-1. The input data consist of data for identification of a specific target and data for extracting analytic estimation from the similar past situations or the similar group of targets.

Table 6.9.2-1: Network data collection for the analysis

| Information | Source | Description |
| --- | --- | --- |
| Location Info | AMF | The location information of the UE. |
| DNN | SMF or PCF | To identify the DNN |
| S-NSSAI | SMF or AMF | To identify the S-NSSAI for the PDU Session which contains the QoS flow |
| UE ID | AMF | To identify UE |
| Internal Group ID | SMF | To identify UE group if available |
| Traffic usage report | UPF or SMF | To extract traffic characteristics such as flow length, size, and inter packet arrival time. |
| Expected UE Behaviour parameters | UDR/UDM | To extract the Expected UE Behaviour parameters. |
| NF resource status | OAM/NRF | The status of assigned resources such as CPU and memory. |
| NF load | OAM/NRF | The load of specific NFs. |
| Service experience | AF | User or service provide feedback on service experience such as MOS. |

NOTE:	The correlation of analysed traffic information per DNN, per Slice, per service, per group of UE is dependent on usage of the information. Timestamp can be used to correlate the collected data from different sources.

### 6.9.3	Output Analytics

The AMF may consult NWDAF to obtain the analytics information for MICO mode parameter optimization. The NWDAF may respond to the AMF request with the analytic estimation of necessary parameter values and any missing external parameter values. The accuracy and confidence interval related to the analytic assistant information can be optionally contained together in the response. It provides that the confidence on the results of the NWDAF compared to the past observed data. The examples of parameters include behavioural pattern and communication pattern in TS 23.502 [3].

Editor's note:	The exact analytics information for MICO mode parameter optimization is FFS.

**** End Change ****