SA WG2 Meeting #133 	S2-1904912

# May 13 – 17, 2019 ; Reno, Nevada, USA	 	     (revision of S2-1904912)

Source:	Nokia, Nokia Shanghai Bell

Title:	Update to PDU Session handling for FN-RG

Document for:	Approval

Agenda Item:	6.7

Work Item / Release:	5WWC / Rel-16

Abstract of the contribution: Update to PDU Session handling for FN-RG

# Introduction

Generally minor corrections

- Especially avoid repeating information already in 23.502

# Proposal

It is proposed to update 23.316 as follows:

FIRST CHANGE

#### 7.3.4	FN-RG related PDU Session Establishment via W-5GAN

The procedure below is based on the PDU Session Establishment procedure specified in TS 23.502 [3] clause 4.3.2.2.1.

Figure 7.3.4-1: FN-RG related PDU Session Establishment via W-5GAN

1.	After the registration procedure is completed, the W-AGF may by establish PDU session(s) on behalf of the FN-RG. The trigger for W-AGF to initiate a PDU establishment process is defined in BBF Specifications.

The W-AGF generates a PDU session ID and derives the parameters for the PDU Session (S-NSSAI, DNN, SSC mode, etc.) based on local configuration, on URSP rules received from the 5GC and on information received during the Registration procedure and stored on the W-AGF.

If W-AGF has received a DHCPvV4/DHCPv6 request from the FN-RG, it may request deferred IP address allocation.

The W-AGF sends a NAS PDU Establishment Request to the AMF. This request contains the PDU Session ID, and may contain a Requested PDU Session Type, a Requested SSC mode, 5GSM Capability PCO, SM PDU DN Request Container, Number of Packet Filters.

The W-AGF sends NAS PDU Establishment Request in a N2 Uplink NAS transport message (NAS message, User location information, W-AGF identities).

The W-AGF identities parameter carries a list of identities of W-AGF serving the FN-RG for the PDU session that is being established.

2a.	The PDU session request is processed in the 5GC as per steps 2-11 of TS 23.502 [3] clause 4.3.2.2.1. These steps are for UPF selection and resource reservation/allocation in the UPF. An additional parameter may be sent from AMF to SMF that carries a list of identities of W-AGF serving the FN-RG for the PDU session that is being established. The SMF can use it in step 8 of TS 23.502 [3] clause 4.3.2.2.1 as input for UPF selection.

2b.	.

As described in step 12 of clause 4.3.2.2.1 in TS 23.502 [3], the AMF shall send a N2 PDU Session Resource Setup Request message

3.	Based on its own policies, configuration and based on the QoS flows, QoS parameters received in the previous step, the W-AGF shall determine what W-UP resources are needed for the PDU session.

The W-AGF may perform BBF specific resource reservation with the AN, that is, it sets up the W-UP resources for the PDU session. This step is specified by BBF for W-5GBAN and by CableLabs for W-5GCAN.

4.	The W-AGF allocates AN N3 tunnel information for the PDU Session and includes the AN N3 tunnel endpoint information in the N2 PDU Session Setup Response message to the AMF.

5.	The PDU session setup procedure is completed in 5GC. All steps after step 13 as specified in TS 23.502 [3] figure 4.3.2.2.1 are executed.

6.	If W-AGF requested deferred IP address allocation in step 1a and this was accepted by the network, the W-AGF sends on the user Plane of the PDU Session any DHCP or RS message received beforehand from the FN-RG. If the UE IP address was delivered in NAS in step 2b, the W-AGF provides this IP address to the FN-RG using access network specific means. This allows the IP address allocation to complete with the RG.

NOTE:	If the DHCP request is sent by FN-RG during the authentication procedure; the W-AGF may cache this and complete the address allocation via 5GC after the PDU session setup.

NEXT CHANGE

END OF CHANGES