3GPP TSG-SA2 Meeting #133	S2-1904930

# Reno, Nevada, United States, 13th May 2019 - 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1120 | rev | 4 | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the EN in Bindings between the consumer and producer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-03-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Align the binding procedures with the NF Set and NF Service Set concepts, and resolve the related Editor’s Note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the supported binding addresses, aligned with the NF Set and NF Service Set concepts. Add a new clause for update of the bindings between the consumer and producer. Rev 4: Only the bindings to NF instance, NF service instance, and NF Set are possible when the binding is created as part of service request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in the specification. Unclear how the bindings are updated and released. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.11, 4.17.11.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************** First Change ****************

*** BEGIN CHANGES ***

### 4.17.11 Binding between NF service consumer and NF service producer

#### 4.17.11.1 General

This clause describes the procedures to establish binding between the NF service consumer and producer.

Binding can be used to indicate suitable target producers for NF service instance selection and reselection.

When the binding is established, it is set either with the NF instance or the NF Set; the NF service consumer or SCP should select all concerned NF service instances from this NF instance or NF Set, respectively. The binding may additionally be set either with NF service instance or NF Service Set, as described in this clause.

It is up to the NF service producer to establish and update the binding. The binding is released implicity when the concerned context is deleted, e.g. when the UE context is deleted as part of PDU session release procedure.

Direct Communication or Indirect Communication procedures may be used between the Consumer and Producer. In case of Indirect Communication, an SCP is located between the Consumer and Producer. The NF service consumer is able to maintain the binding to the NF service producer by using the binding indication included in the service request or response, so the SCP does not need to store the binding indication.

#### 4.17.11.2 Binding created as part of service response

When the NF service consumer communicates with the NF service producer, the producer may return a binding indication to the consumer. The consumer stores the received binding indication and uses it for the subsequential requests concerning the data context.

Figure 4.17.11.2-1: Binding created as part of service response

1. 	If Direct Communication is used, the NF service consumer selects the NF service producer and sends the request to the selected NF service producer. If Indirect Communication mode is used the NF service consumer sends the request to the SCP and provides within the service request to the SCP the discovery and selection parameters necessary to discover and select a NF cervice producer.

2. 	The NF service producer sends a response to the NF service consumer. In the response the NF cervice producer may include a a binding indication. If the NF service consumer receives a binding indication as specified in Table 6.3.1.0-1 in TS 23.501 [2], it uses it for subsequent requests regarding the concerned resource. Otherwise, the procedure ends here.

3.	The NF Service Consumer uses the binding indication received in the previous step for subsequent requests regarding the concerned resource.

4. 	The NF service producer sends a response to the consumer. The NF service producer may respond with an updated resource address and updated binding indication different to the one received in the previous response.

#### 4.17.11.3 Binding created as part of service request

Binding can be created as part of the implicit or explicit subscription request. In this case, the subscription request or the service request that creates the implicit subscription may include a binding indication. In addition, if the NF service consumer can also be as a NF service producer for later communication from the contacted producer, a service request sent to  the producer may include binding indication

Figure 4.17.11.3-1: Binding created as part of service request

1. 	Instance A, as an NF service consumer sends a service request using either Direct Communication or Indirect communication via SCP and Instance B is selected as NF service producer. If the request creates a subscription, or if Instance A, can also be NF service producer for later communication for the concerned data context, it may include binding  indication referring to NF service instance, NF instance or NF Set as specified in Table 6.3.1.0-1 in TS 23.501 [2] in the request sent to the NF service producer.

2. 	Instance B as the NF service producer sends a response to the NF service consumer.

3.	When the producer sends a notification or when Instance B as NF service consumer needs to invoke the service provided by Instance A, Instance B sends a request with the binding indication received in step 1 as described in Steps 3-4 in Figure 4.17.11.2-1.

#### 4.17.11.4 Binding update

The binding indication can be updated by the NF service producer in-band as part of the service operation request or response, or out-of-band via notification operation.

Figure 4.17.11.4-1: Binding update

Case A)	NF service producer updates the binding indication in the response to the service request.

Case B)	NF service producer updates the binding indication out-of-band by notifying the NF service consumer.

Case C)	NF service consumer that can also take a role of NF service producer updates the binding indication as part of the service request.

Case D)	NF service consumer that can also take a role of NF service producer updates the binding indication by notifying the NF service consumer.

*** END CHANGES ***