3GPP TSG-SA WG2 Meeting #133	S2-1904969

13 - 17 May 2019, USA,

Source:	ORANGE, Huawei

Title:	NF Load Analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes for NWDAF “NF Load analytics” to precise the exact references for data collection via OAM and NRF

# Discussion

This discussion addresses two topics:

- The exact references to use for data collection

- The question of actual resources versus potential resources in EN

POINT 1 : References for data collection

The present definition stated in §6.5 presents the general principle of NF load analytics, on the basis of the following data collection:

Table 6.5.2-1: Data collected by NWDAF for NF load analytics

| Information | Source | Description |
| --- | --- | --- |
| NF resource status | OAM | The status of assigned resources such as CPU and memory for a specific NF instance |
| NF load | OAM or NRF | The load of a specific NF instance |

The OAM document serie (28.55x Performance) provides :

Basic informations about mean usage of NF virtual resources(CPU, memory, disk. No additional information) per NF type (e.g. AMF, SMF) under the assumption that one VNFC instance supports no more than one NF (TS 28.552 clause 5.7) . This is the sole information available as a percentage.

Other information specific to NFs (e.g. Performance measurement for AMF TS 28.552 clause 5.2 “Mean number of registered subscribers”) which cannot be converted into a percentage, because the global capacity is not available.

No information on resource status (i.e availability) is simply available (maybe via fault management ?)

The NRF specification provides:

Information on NF and NF services about load (percentage) and status (availability)  as defined in Nnrf_NFManagement Service as stated in TS 23.502 (NF profile, NF status) and TS 29.510 (details on load in NF profile, and NF status).

The NRF specification does not provide any information on ressources (CPU, memory, disk, other).

- POINT 2 : actual resources versus potential resources

Editors notes address also the question of actual resources versus potential resources:

Editor's note:	The need for other data to collect is FFS. It is also FFS if the resource status is the resources currently in use by an NF or the resources available for the NF scale. In addition it is FFS if load is reported in relation to used or available resources

The question of available resources is not easy, for the following reasons:

Such information is not available

The limits of what is indeed an “available resource” are difficult to bound:

  - Free resources available for scaling versus occupied ressources which could be really pre-empted

  - Resources of the same pool of ressources (availability zone, data center) versus another pool

The “potential availability” of a resource does not define in which delay it could be really available for operation: even NF scaling using free resources may required delays not compatible with a CN real-time decision. This question is more difficult for the case of preemption.

- CONSEQUENCES

- It is propose to modify the text accordingly

Reference the clauses of OAM performance (TS 28.552) and NRF (23.501 and 29.510)

Remove Editors notes and specify that the ratios provided address the ressources currently in use

The term “usage” to denote the ratio of usage of resources ‘CPU, memory, disk”.

The term “status” is used to denote the availability of the NF itself

The OAM information can be used as a complement to NRF information on resources, or as a way to correlate NRF information or as an alternative if NRF information on load is not available.

# Proposal

The following changes are proposed in TS 23.288.

* * * First change * * * *

## 6.5 NF load analytics

### 6.5.1	General

The clause 6.5 describes how NWDAF can provide NF load analytics, in the form of statistics or predictions or both, to another NF.

The service consumer may be an NF, an AF, or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics Id set to "NF load information”,

-	The Target of Analytics Reporting,

-	Analytics Filter Information: an optional list of Instance IDs,

-	An Observation period indicates the time when the statistics or predictions are requested,

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.5.3.

If the NF Instance IDs are provided, the NWDAF shall provide the analytics for each designated NF instance. In such case the Target of Analytics Reporting should be ignored. Otherwise, the NWDAF shall use the specific UE ID to determine which instances (AMF, N3IWF, SMF, PCF) are serving this specific UE.

NOTE:	The list of possible instances is limited to AMF, N3IWF, SMF, PCF.

### 6.5.2	Input data

For the purpose of NF load analytics, the NWDAF may collect the information as listed in Table 6.5.2-1.

Table 6.5.2-1: Data collected by NWDAF for NF load analytics

| Information | Source | Description |
| --- | --- | --- |
| NF load | NRF | The load of a specific NF instance in the NF profile as defined per TS.29.510 |
| NF status | NRF | The status of a specific NF instance as defined per TS 29.510 |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for a specific NF instance (mean usage of virtual CPU, memory, disk) as defined per TS 28.552 clause 5.7 |

NOTE: The OAM information can be used as a complement to NRF information on resources, or as a way to correlate NRF information or as an alternative if NRF information on load is not available.

Editor's note:	It is FFS if information about changes in the used resource  should also be collected (e.g., NF moved from a resource with 90% of reliability to a resource with 80% of reliability)

### 6.5.3	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

Load statistics information is defined in the Table 6.6.3-1.

Table 6.5.3-1: NF load statistics

| Information | Description |  |
| --- | --- | --- |
| List of resource status (1..n) | List of observed load information for each NF instance |  |
| > Instance ID | Identification of the NF instance |  |
| > NF status | The availability status of the NF on the Observation period, expressed as a percentage of time |
| > NF resource usage | The average usage of assigned resources (CPU, memory, disk) |  |
| > NF load | The average load of the NF instance over the observation period |  |
| > NF peak load (optional) | The maximum load of the NF instance over the observation period |  |

Load predictions information is defined in the Table 6.6.3-2.

Table 6.5.3-2: NF load predictions

| Information | Description |
| --- | --- |
| List of resource status (1..n) | List of predicted load information for each NF instance |
| > Instance ID | Identification of the NF instance |
| > NF status | The availability status of the NF on the Observation period, expressed as a percentage of time |
| > NF resource usage | The average usage of assigned resources (CPU, memory, disk) |
| > NF load | The average load of the NF instance over the observation period |
| > NF peak load (optional) | The maximum load of the NF instance over the observation period |
| > Confidence |  | Confidence of this prediction |  |

### 6.5.4	Procedures

The procedure depicted in Figure 6.5.4-1 allows a consumer NF to request analytics to NWDAF for NF load of various NF instances as defined in 6.5.1.

Figure 6.5.4-1: NF load analytics provided by NWDAF

1.	The NF sends a request to the NWDAF for analytics for NF load for a specific NF, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The Analytics ID is set to NF load information, the target for analytics is set to NF id. The NF can request statistics or predictions or both and can provide a time window.

2-5. If the request is authorized, and in order to provide the requested analytics, the NWDAF may need for each NF targeted instance to subscribe to OAM services to retrieve the target NF resource usage following steps captured in Clause 6.2.3.3 for data collection from OAM. Steps 2-5 may be skipped when e.g. the NWDAF already has the requested analytics.

Editor's note:	The exact SA5 service used is FFS.

NOTE:	The call flow only shows a subscription/notification model for the simplicity, however both request-response and subscription-notification models should be supported.

6.	The NWDAF retrieves the NF load and NF status information from NRF using Nnrf_NFManagement_NFStatusSubscribe service operation for each NF instance.

7.	The NWDAF derives requested analytics.

8.	The NWDAF provide requested NF load analytics to the NF, using either the Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

9-11. If at step 1 the NF has subscribed to receive continuous reporting of NF load analytics, the NWDAF may generate new analytics and provide them to the NF upon reception of notification of new NF load information from OAM.

* * * End of changes * * * *