SA WG2 Meeting #133	S2-1905004

May 13 – 17, 2019, Reno, Nevada, US	(revision of S2-19xxxx)

Source:	Ericsson

Title:	PDU Session Modification and Release Procedures for FN-RG

Document for:	Approval

Agenda Item:	6.7

Work Item / Release:	5WWC/Rel-16

Abstract of the contribution: This contribution proposes PDU Session Modification and PDU Session Release procedures for FN-RG via W-5GAN.

# Introduction

This contribution proposes deltas for FN-RG procedures related to PDU session modification and session release to TS 23.316, when compared to the existing procedures. It uses solution #23 in 23.716 as baseline as agreed during the study phase.

# Proposal

It is proposed to update TS 23.316 as follows:

**** All text is new ****

#### 7.3.X	FN-RG or Network Requested PDU Session Modification via W-5GAN

The PDU session modification procedure for the FN-RG is similar to that of 5G-RG described in clause 7.3.2 but with the following differences:

- The 5G-RG is replaced with an FN-RG.

- W-AGF acts on behalf of FN-RG, as an endpoint for N1 signalling.

- In step 1, the W-AGF initiates a PDU Session Modification Request to the AMF on behalf of FN-RG.

- W-AGF may issue L-W-CP resource modification procedure (out of scope of 3GPP) with the FN-RG that is related with the information received from the SMF as in step 4.

- In step 4, L-W-UP resources may be modified by the W-AGF.

- Steps 7a & 7b of clause 7.3.2 are not valid for the FN-RG. The W-AGF creates an Uplink NAS transport message to the AMF, which contains the PDU Session Modification Ack as in step 8.

#### 7.3. Y	FN-RG or Network Requested PDU Session Release via W-5GAN

The PDU session release procedure for the FN-RG is similar to that of 5G-RG described in clause 7.3.3 but with the following differences:

- The 5G-RG is replaced with an FN-RG.

- W-AGF acts on behalf of FN-RG, as an endpoint for N1 signalling.

- In step 2, the W-AGF sends a PDU session release request to the AMF on behalf of FN-RG.

- In step 5, upon receiving AN session release request message from the AMF, the W-AGF can trigger the release of the corresponding L-W-UP resource with procedure out of scope of 3GPP.

- Steps 8 & 9 of clause 7.3.3 are not valid for the FN-RG. The W-AGF creates an Uplink NAS transport message to the AMF, which contains the PDU Session Release Ack as in step 10.

**** End of changes ****