3GPP TSG-SA WG2 Meeting #132 	S2-1905061

Reno, NV, USA, May 13 - 17	(revision of S2-19xxxx)

Source:	Ericsson

Title:	Handling of hidden producer services

Document for:	Agreement

Agenda Item: 	6.19

Work Item / Release:	5G_eSBA / Rel-16

Abstract of the contribution: This contribution discusses “hidden” NF service instances and propose to add a note to 23.501 CR 1222.

# 1	Discussion

In SA2 #132 it was mentioned that a service producer may have “hidden” NF service instances, i.e. service instances or termination points that the service producer has not registered in NRF. These termination points may be returned in responses to requests that create resources. On protocol level the termination point is provided as a URI transported in the Location header parameter in HTTP 201 message. Now if the binding indicator proposed in S2-1904734 is used as a way to indicate to what a created resource is bound to (NF service instance, NF instance, NF service set, NF set), then these “hidden” service instance creates a problem if the service producer indicates something in the binding indicator that is not NF service instance. To remedy this problem, a service producer that want to bind a created service to something else than a service instance, the service producer should make sure all service instances in the NF instance/NF service set/NF set (depending on what the resource is bound to) are registered in NRF.

Proposal 1: If a NF service producer wants to bind a resource to a construct equal to NF instance, NF service set, or NF Set, all service instances within that construct shall be registered in NRF.

# 2	Proposal

It is proposed that the proposal 1 is captured in an appropriate place in a note provided below. The note should be included in CR1222 for 23.501 when CR becomes more mature.

NOTE:	It is up to the NF service producer to make sure that it registers the NF Service instances that may be part of a NF, NF Service Set or NF Set if the binding information indicates that a resource is bound to any of these constructs.