3GPP TSG-SA WG2 Meeting #133	S2-1905260

# Reno, USA, 13 - 17 May, 2019 	(revision of S2-1904775)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1052 | rev | 5 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further detailing of 5G LAN group management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson?, Nokia?, Nokia Shanghai Bell?, Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-05-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The description of 5G LAN group management is on high level and required details are missing to fully specify the solution. Furthermore there is an open issue on the cardinality between DNN and a 5G LAN. In TS 22.261, the following requirement is stated: “The 5G system shall be able to restrict private communications within a 5GLAN-VN based on UE’s location (i.e. when the UE moves out of the area it can no longer communicate on the 5G LAN-VN).” However, this requirement is not yet considered in Stage 2 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Further detail how 5G LAN group management is supported. “VN service area” is introduced as part of group configuration. Resolve Editor’s note on relation between DNN and 5G LAN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of 5G LAN group management and remaining open issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.2, 5.28.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change *****

### 5.28.2	5G VN group management

5G System supports management of 5G VN Group identification and membership (i.e. definition of 5G VN group identifiers and membership) and 5G VN Group data (i.e. definition of 5G VN group data). The 5G VN Group management can be configured by a network administrator or can be managed dynamically by AF.

A 5G VN group is characterized by the following:

-	5G VN group identities: External Group Id and Internal Group Id is used to identify the 5G VN group.

-	5G VN group membership: The 5G VN group members are uniquely identifed by GPSI. The group as described in clause 5.2.3.3.1 of TS 23.502 [3] is applicable to 5G LAN-type services.

-	5G VN group data. The 5G VN group data may include the following parameters: PDU session type, DNN,  S-NSSAI, Application descriptor and VN Service Area.

Editor’s note: Whether additional parameters are needed in the 5G VN group data is FFS

In order to support dynamic management of 5G VN Group identification and membership, the NEF exposes a set of services to manage (e.g. add/delete/modify) 5G VN group and 5G VN member. The NEF also exposes services to dynamically manage 5G VN group data.

A 5G VN group is identified by the AF using External Group Id. The NEF maps the External Group Id to Internal Group Id by invoking Nudm_SDM_Get service operation (External Group Id, Group Identifier Translation).

Editor’s note: It is FFS how to handle External Group Id and Internal Group Id for newly created 5G VN groups.

An External Group Id for a 5G VN group corresponds to a unique set of 5G VN group data parameters.

A VN Service Area is a list of 3GPP access area identifiers (e.g. TAIs or Cell IDs). The UE is only able to communicate with the 5G VN Group when its location is within the VN Service Area.

The 5G VN group configuration is either provided by OA&M or provided by an AF to the NEF.

When configuration is provided by an AF, the NEF service operations information flow procedure described in TS 23.502 [3] clause 4.15.6.2 applies for storing the 5G VN group identifiers, group membership information and group data in the UDR.

Editor’s note: It is FFS whether an existing Data Set / Data subset can be used, or new Data Set/Subset is to be defined, e.g. whether 5G VN group data is stored as part of Application data and/or in some other Data Set / Data subset.

When managing the group members of a 5G VN group, the NEF / UDM maps the GPSI of each member of the 5G VN group to their corresponding SUPIs. The NEF / UDM also updates the list of subscribed Internal Group identifiers of the corresponding UE’s subscription data using the procedure described in TS 23.502 [3] clause 4.15.6.2.

Editor’s note: It is FFS whether in the above paragraph the mappings are carried out by the NEF or by the UDM.

If the UE is member of a 5G VN Group, UDM provides the Internal Group identifier and the corresponding 5G VN Group data to AMF and/or SMF.

The third party AF may update the UE Identities of the 5G VN group at any time after the initial provisioning.

The PCF delivers 5G VN group configuration information (Application descriptor, DNN, S-NSSAI, PDU session type) to the UE for each GPSI that belongs to a 5G VN group. The 5G VN group configuration information is delivered in the URSP from the PCF to the UE using the UE Configuration Update procedure for transparent UE Policy delivery as described in TS 23.502 [3] clause 4.2.4.3 and TS 23.503 [45] clause 6.1.2.2.

**** Next Change ****

### 5.28.3	PDU Session management

Session management as defined for 5GS in clause 5.6 is applicable to 5G-LAN-type services with the following clarifications:

-	A UE gets access to 5G LAN-type services via a PDU Session of IP PDU Session type or Ethernet PDU Session type.

-	A PDU Session provides access to one and only one 5G-LAN group

-	A dedicated SMF is responsible for all the PDU Sessions for communication of a certain 5G-LAN group.

Editor's note:	Whether and how the above bullet needs to be updated due to eSBA is FFS.

Editor's note:	Whether there are impacts to SMF selection to select this dedicated SMF is FFS.

NOTE:	Having a dedicated SMF serving a 5G-LAN group does not contradict that redundancy solutions can be used to achieve high availability.

-	A DNN is associated with a 5G-LAN group

-	The UE provides a DNN associated with the 5G-LAN group to access the 5G LAN-type services for that 5G-LAN group, using the PDU Session Establishment procedure described in TS 23.502 [3], clause 4.3.2

-	During establishment of the PDU Session, secondary authentication as described in clause 5.6.6 and in TS 23.502 [3], clause 4.3.2.3, may be performed in order to authenticate and authorize the UE for accessing the DNN associated with the 5G-LAN group. Authentication and authorization for a DNN using secondary authentication implies authentication and authorization for the associated 5G-LAN group. There is no 5G-LAN group specific authentication or authorization defined.

-	The SM level subscription data for a DNN and S-NSSAI available in UDM, as described in clause 5.6.1, applies to a DNN associated to a 5G-LAN group.

-	Session management related policy control for a DNN as described in TS 23.502 [3], is applicable to a DNN associated to a 5G-LAN group. This includes also usage of URSP, for the UE to determine how to route outgoing traffic to a PDU Session for a DNN associated to a 5G-LAN group.

-	When the UE is out of the VN Service Area, the PDU Session establishment or activation should be rejected by the network.

**** End of Changes ****