3GPP TSG-SA WG2 Meeting #133	S2-1905338

# Reno,USA 13th -17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1376 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Enhanced Coverage Restriction Control via NEF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To introduce support for Enhanced Coverage Restriction Control via NEF as per the agreed conclusion of Key Issue 6 in the TR 23.724. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhanced Coverage Restriction Control via NEF to query status of, enhanced coverage restriction or enable/disable enhanced coverage restriction per individual UE is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhanced Coverage Restriction Control via NEF as per the agreed conclusion of Key Issue 6 in the TR 23.724 will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.12, 7.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Referenced clause 4.XX of TS 23.502 is introduced in S2-1905339. |  |  |  |  |  |  |  |  |

***** Start of Changes *****

### 5.31.12	Restriction of use of Enhanced Coverage

Support of UEs in Enhanced Coverage is specified in TS 36.300 [30].

The usage of Enhanced Coverage requires use of extensive resources (e.g. radio and signalling resources). Specific subscribers can be restricted to use Enhanced Coverage feature through Enhanced Coverage Restricted information that is stored in the UDM as part of subscription data and specifies per PLMN whether the Enhanced Coverage functionality is restricted or not for the UE. Enhanced Coverage Restricted information indicates whether CE mode B is restricted for the UE, or both CE mode A and CE mode B are restricted for the UE, or both CE mode A and CE mode B are not restricted for the UE.

The AMF receives Enhanced Coverage Restricted information from the UDM during the Registration procedure.The AMF based on local configuration, UE Usage setting, UE subscription information and network policies, or any combination of them, determines whether Enhanced Coverage (i.e. CE mode B or both CE mode B & CE mode A) is restricted for the UE and stores updated Enhanced Coverage Restriction information in the UE context in the AMF. If the UE usage setting indicated that UE is "voice centric", then the AMF shall set CE mode B restricted for the UE in Enhanced Coverage Restriction information.

If the UE includes the support for restriction of use of Enhanced Coverage, the AMF sends Enhanced Coverage Restricted information to the UE in the Registration Accept message. The UE shall use the value of Enhanced Coverage Restricted information to determine if enhanced coverage feature is restricted or not. The AMF provides an Enhanced Coverage Restricted information to the RAN via N2 signalling whenever the UE context is established in the RAN, e.g. during N2 Paging procedure, Service Request procedure, Initial Registration and Periodic Registration procedure.

For roaming UEs, if the UDM doesn't provide any Enhanced Coverage Restricted information or the provided Enhanced Coverage Restricted information is in conflict with the roaming agreement, the AMF uses default Enhanced Coverage Restricted information locally configured in the VPLMN based on the roaming agreement with the subscriber's HPLMN.

The UE indicates its capability of support for restriction of use of Enhanced Coverage to the AMF in the Registration procedure for the RAT it is camping on. The UE that indicates its support Enhanced Coverage over N1 interface shall also support restriction of the Enhanced Coverage over N1 interface.

The UE shall assume that restriction for use of Enhanced Coverage is the same in the equivalent PLMNs.

The support for Enhanced Coverage Restriction Control via NEF enables 3rd party service providers to query status of enhanced coverage restriction or enable/disable enhanced coverage restriction per individual UEs. The procedure for Enhanced Coverage Restriction Control via NEF is described in clause 4.XX of TS 23.502 [3].

***** Next Change *****

### 7.2.8	NEF Services

The following NF services are specified for NEF:

Table 7.2.8-1: NF Services provided by NEF

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Nnef_EventExposure | Provides support for event exposure | 5.2.6.2 |
| Nnef_PFDManagement | Provides support for PFDs management | 5.2.6.3 |
| Nnef_ParameterProvision | Provides support to provision information which can be used for the UE in 5GS | 5.2.6.4 |
| Nnef_Trigger | Provides support for device triggering | 5.2.6.5 |
| Nnef_BDTPNegotiation | Provides support for background data transfer policy negotiation and optionally notification for the renegotiation. | 5.2.6.6 |
| Nnef_TrafficInfluence | Provide the ability to influence traffic routing. | 5.2.6.7 |
| Nnef_ChargeableParty | Requests to become the chargeable party for a data session for a UE. | 5.2.6.8 |
| Nnef_AFsessionWithQoS | Requests the network to provide a specific QoS for an AS session. | 5.2.6.9 |
| Nnef_ServiceParameter | Provides support to provision service specific information | 5.2.6.11 |
| Nnef_APISupportCapability | Provides support for awareness on availability or expected level of a service API | 5.2.6.12 |
| Nnef_AnalyticsExposure | Provides support for exposure of network analytics | 5.2.6.16 |
| Nnef_ECRestriction | Provides support for quering status of enhanced coverage restriction, or enable/disable enhanced coverage restriction per individual UEs | 5.2.6.x |

***** End of Changes *****