3GPP TSG-SA WG2 Meeting #133 	S2-1905350

Reno, NV, USA, 13 – 17 May 2019	(revision of S2-190xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1377 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCF selection for multiple PDU Sessions to the same DNN and S-NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2019-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is a proposal to be able to select the same PCF for all PDU sessions to the same DNN, S-NSSAI, for the purpose to enable aggregation of the usage over all PDU sessions to the same DNN, S-NSSAI and then comparation with the allowed usage for the DNN, S-NSSAI. The proposed solution reuses the existing capability to select the same SMF for all PDU sessions to the same DNN, NSSAI of a UE and then the SMF selects the same PCF as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new conditions to select the same SMF for all UE PDU sessions to the same DNN, S-NSSAI is included. The PCF selection for the PDU session is extended to allow to select the same PCF for all PDU sessions of the UE to the same DNN. S-NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to select the same PCF for any PDU session to the same DNN, S-NSSAI of a UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2; 6.3.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 1385 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Companion CR: 23.502 CR1385 (S2-1905351)) |  |  |  |  |  |  |  |  |

Start of Changes

### 6.3.2	SMF discovery and selection

The SMF selection functionality is supported by the AMF and SCP and is used to allocate an SMF that shall manage the PDU Session. The SMF selection procedures are described in clause 4.3.2.2.3 of TS 23.502 [3].

The SMF discovery and selection functionality follows the principles stated in clause 6.3.1.

If the AMF does discovery, the AMF shall utilize the NRF to discover SMF instance(s) unless SMF information is available by other means, e.g. locally configured on AMF. The AMF provides UE location information to the NRF when trying to discover SMF instance(s). The NRF provides NF profile(s) of SMF instance(s) to the AMF. In addition, the NRF also provides the SMF service area of SMF instance(s) to the AMF. The SMF selection functionality in the AMF selects an SMF instance and an SMF service instance based on the available SMF instances obtained from NRF or on the configured SMF information in the AMF.

NOTE 1:	Protocol aspects of the access to NRF are specified in TS 29.510 [58].

The SMF selection functionality is applicable to both 3GPP access and non-3GPP access.

The SMF selection for Emergency services is described in clause 5.16.4.5.

The following factors may be considered during the SMF selection:

a)	Selected Data Network Name (DNN).

b)	S-NSSAI.

c)	NSI-ID.

NOTE 2:	The use of NSI -ID in the network is optional and depends on the deployment choices of the operator. If used, the NSI ID is associated with S-NSSAI.

d)	Access technology being used by the UE.

e)	Support for Control Plane CIoT 5GS Optimisation.

f)	Subscription information from UDM, e.g.

-	per DNN: whether LBO roaming is allowed.

-	per S-NSSAI: the subscribed DNN(s).

-	per (S-NSSAI, subscribed DNN): whether LBO roaming is allowed.

g)	per (S-NSSAI, subscribed DNN): whether EPC interworking is supported, and whether selecting the same SMF for all PDU sessions to the same S-NSSAI and DNN is required.

h)	Local operator policies.

i)	Load conditions of the candidate SMFs.

j)	Analytics (i.e. statistics or predictions) for candidate SMFs' load as received from NWDAF (see TS 23.288 [86]), if NWDAF is deployed.

k)	UE location (i.e. TA).

l)	Service Area of the candidate SMFs.

m)	MA-PDU Capability of the SMF.

In the case of delegated discovery, the AMF, shall send all the available factors a)-d) to the SCP and shall also indicate if interworking with EPS is required and if the PDU session shall be home routed.

In addition, the AMF may indicate to the SCP which NRF to use (in case of NRF dedicated to the target slice).

Editor's note:	It is FFS how to capture that required discovery and selection information should be conveyed to the SCP with as minimal impact on the end-to-end signalling performance latency between AMF and SMF.

If there is an existing PDU Session and the UE requests to establish another PDU Session to the same DNN and S-NSSAI and either the UE subscription data indicates the support for interworking with EPS for this DNN and S-NSSAI, or the indication that the same SMF shall be selected for all PDU sessions to the same S-NSSAI, DNN is set, then the same SMF shall be selected. In addition, if the UE Context in the SMF received from UDM provides a SMF ID for an existing PDU session to the same DNN, S-NSSAI, the AMF uses the stored SMF ID. In any such a case where the AMF can determine which SMF should be selected, if delegated discovery is used, the AMF shall indicate a desired NF Instance ID so that the SCP is able to route the message to the relevant SMF. Otherwise, if UE subscription data does not indicate the support for interworking with EPS for this DNN and S-NSSAI, a different SMF may be selected. For example, to support a SMF load balancing or to support a graceful SMF shutdown (e.g., a SMF starts to no more take new PDU Sessions). The AMF also provides the indication the same PCF shall be selected if received as part of the SMF and PCF selection information.

In the home-routed roaming case, the SMF selection functionality selects an SMF in VPLMN as well as an SMF in HPLMN. In the context of Network Slicing this is specified in clause 4.3.2.2.3.3 of TS 23.502 [3].

When the UE requests to establish a PDU Session to a DNN and an S-NSSAI, if the UE MM Core Network Capability indicates the UE supports EPC NAS and optionally, if the UE subscription indicates the support for interworking with EPS for this DNN and S-NSSAI, the selection functionality (in AMF or SCP) selects a combined SMF+PGW-C. Otherwise, a standalone SMF may be selected.

If the UDM provides a subscription context that allows for handling the PDU Session in the visited PLMN (i.e. using LBO) for this DNN and S-NSSAI and, optionally, the AMF is configured to know that the visited VPLMN has a suitable roaming agreement with the HPLMN of the UE, the following applies:

-	If the AMF does discovery, the SMF selection functionality in AMF selects an SMF from the visited PLMN.

-	If delegated discovery is used, the AMF indicate to the SCP that home routing is not needed, the delegated discovery and selection functionality in SCP shall discover and select an SMF from the visited PLMN.

If an SMF in VPLMN cannot be derived for the DNN and S-NSSAI, or if the subscription does not allow for handling the PDU Session in visited PLMN using LBO, then the following applies:

-	If the AMF does discovery, both an SMF in VPLMN and an SMF in HPLMN are selected, and the DNN and S-NSSAI is used to derive an SMF identifier from the HPLMN.

Editor's note:	It is FFS how the SCP handles the selection of the vSMF and hSMF in the HR roaming case and it is also FFS how the vSMF is informed of the selected hSMF in the case of delegated discovery and HR case.

If the initially selected SMF in VPLMN (for roaming with LBO) detects it does not understand information in the UE request, it may reject the N11 message (related with a PDU Session Establishment Request message) with a proper N11 cause triggering the AMF to select both a new SMF in the VPLMN and a SMF in the HPLMN (for home routed roaming).

The AMF selects SMF(s) considering support for CIoT 5GS optimisations (e.g. Control Plane CIoT 5GS Optimisation).

Additional details of AMF selection of an I-SMF are described in the clause 5.34.

In the case of home routed scenario, the AMF selects a new V-SMF if it determines that the current V-SMF cannot serve the UE location. The selection/relocation is same as an I-SMF selection/relocation as described in the clause 5.34.

Next Change

#### 6.3.7.1	PCF discovery and selection for a UE or a PDU Session

PCF discovery and selection functionality is implemented in AMF, SMF and SCP, and follows the principles in clause 6.3.1. The AMF uses the PCF services for a UE and the SMF uses the PCF services for a PDU Session.

When NF service consumer performs discovery and selection, the following applies:

-	The AMF may utilize the NRF to discover the candidate PCF instance(s) for a UE. In addition, PCF information may also be locally configured in the AMF. The AMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the AMF, depending on operator's policies.

-	The SMF may utilize the NRF to discover the candidate PCF instance(s) for a PDU Session. In addition, PCF information may also be locally configured in the SMF. The SMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the SMF, depending on operator's policies. In the non roaming case, the AMF selects a PCF instance for AM policy association and selects the same PCF instance for UE policy association. In the roaming case, the AMF selects a V-PCF instance for AM policy association and selects the same V-PCF instance for UE policy association.

The following factors may be considered during the PCF selection for a PDU session:

a)	Local operator policies.

b)	Selected Data Network Name (DNN).

c)	S-NSSAI of the PDU Session.

d)	SUPI range.

e)	PCF selected by the AMF for the UE.

f)   The indication that same PCF shall be selected all PDU sessions to the same DNN, S-NSSAI.

If the SMF receives the indication that same PCF shall be selected all PDU sessions to the same DNN, S-NSSAI, then the SMF shall select the same PCF for the new SM Policy Association as the stored PCF ID for an existing SM Policy Association to the same DNN and S-NSSAI for the same UE.

In the case of delegated discovery and selection in SCP, the SMF shall include the available factors in the first request.

Editor's note:	It still needs to be clarified which actual set of parameters the sentence above is referring to.

The selected PCF instance for serving the UE and the selected PCF instance for serving a PDU session of this UE may be the same or may be different.

In the following scenarios, information about the PCF instance that has been selected i.e. the PCF ID may be forwarded to another NF. If the NF service consumer performs discovery and selection, this NF may use this PCF instance. In the case of delegated discovery and selection, this NF may include PCF ID in the request and the SCP may use this information to select the associated PCF instance (discovery may still be needed depending on what level of information is sent by the AMF, e.g. the address of the PCF instance may not be present):

When NF service consumer performs discovery and selection, the following applies:

-	During AMF relocation, the target AMF may receive a PCF ID from the source AMF to enable the usage of the same PCF by the target AMF, and the target AMF may decide based on operator policy either to use the same PCF or select a new PCF.

-	The AMF may, based on operator policies, forward the selected PCF ID to SMF instance(s) during the PDU Session Establishment procedure(s) to enable the usage of the same PCF for the AMF and the SMF instance(s). The SMF may decide based on operator policy either to use the same PCF or select a new PCF.

-	In the roaming case, the AMF may, based on operator policies, e.g. roaming agreement, select the H-PCF in addition to the V-PCF for a UE by performing the PCF discovery and selection as described above. The AMF sends the H-PCF ID of the selected H-PCF instance to the V-PCF during the policy association establishment procedure.

In the case of delegated discovery and selection in the SCP, the following applies:

-	During AMF relocation, the AMF may receive a PCF ID from the source AMF to enable the usage of the same PCF instance by the AMF. The AMF includes that information in the request. SCP may decide based on operator policy either to use the PCF instance or select another PCF instance that is not interchangeable to the used PCF instance.

-	The AMF may, based on operator policies, forward the selected PCF ID to the SMF during the PDU Session Establishment procedure to enable the usage of the same PCF instance for the AMF and the SMF. The SMF may include that information in the request. SCP may decide based on operator policy either to use the PCF instance or select another PCF instance that is not interchangeable to the used PCF instance.

-	In the roaming case, SCP may, based on operator policies, e.g. roaming agreement, select the H-PCF in addition to the V-PCF for a UE by performing the PCF discovery and selection as described above.

Editor's note:	If the sentence above applies, it is FFS how the AMF can send the selected H-PCF ID to the V-PCF during the policy association establishment procedure or it is FFS whether the sentence above means that the SCP needs to add an extra information H-PCF ID in the application part of the request; it is also FFS whether "SCP" means a SCP in VPLMN or SCP in HPLMN.

End of Changes