3GPP TSG-SA2 Meeting #133	S2-1905369

# Reno, NV, USA, May 13 – 17, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1386 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Support for Delivering Background Data Transfer Polices to the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | xBDT |  |  |  |  |  | Date: |  |  | 05-07-2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Support for xBDT requires that the AF be able invoke an NEF service that will trigger the URSP framework to be used to send a background data transfer policy to the UE. Support for xBDT requires that the AF or NEF be able invoke a PCF service that will trigger the URSP framework to be used to send a background data transfer policy to the UE. Support for xBDT requires that the AF or NEF be able to discover a BSF that is serving the UE for AM policies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new procedure to detail how an AF may invoke an NEF service to trigger the network to use the URSP framework to send a background data transfer policy to the UE. Added a new PCF Service that can be called to request that a background data transfer policy be applied to a UE. Added a new PCF Service that can be called to request that a background data transfer policy be applied to a UE. Updated the BSF services to allow a PCF to register and be discovered when it is serving the UE for AM policies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of the xBDT WID would not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.x (New), 5.2.5.1, 5.2.5.3.x (New), 5.2.6.1, 5.2.6.8.x (New), 5.2.13.2.2, 5.2.13.2.3, 5.2.13.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.15.6.x	Set a policy for a future AF session

Figure 4.15.6.x-1: Set a policy for a future AF session

1.	The AF may request that a previously negotiated policy for background data transfer be applied to the UE by invoking the Nnef_ApplyPolicy_Update service operation (AF Identifier, External Identifier, Reference ID). The Reference ID parameter identifies a previously negotiated transfer policy for background data transfer as defined in clause 4.16.7. The NEF assigns a Transaction Reference ID to the Nnef_ApplyPolicy_Create request.

2.	The NEF invokes Nudm_SDM_Get (Identifier Translation, GPSI) to resolve the GPSI (External Identifier) to a SUPI. The NEF consumes Nbsf_Management_Discovery service operation (SUPI) to find out the address of the relevant PCF if the PCF address is not available on the NEF based on local configuration. The NEF authorizes the AF request and stores the AF Identifier and the Transaction Reference ID. If the authorisation is not granted, steps 3 and 4 are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

NOTE:	Based on operator configuration, the NEF may skip authorization. In this case the authorization is performed by the PCF in step 3.

3.	The NEF interacts with the PCF by triggering a Npcf_UEPolicyAuthorization_Create request message and provides the Background Data Transfer Reference ID and SUPI to the PCF.

4.	The PCF determines whether the request is allowed and notifies the NEF if the request is not authorized. If the request is not authorized, NEF responds to the AF in step 5 with a Result value indicating that the authorization failed. If the request is authorized, the PCF may send the policy to the UE as part of a URSP rule as described in clause 6.1.2.4 of TS 23.502 [20].

5.	The NEF sends a Nnef_ApplyPolicy_Update response message (Transaction Reference ID, Result) to the AF. Result indicates whether the request is granted or not.

SECOND CHANGE

#### 5.2.5.1	General

The following table illustrates the PCF Services.

Table 5.2.5.1-1: NF services provided by PCF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Npcf_AMPolicyControl | Create | Request/Response | AMF |
|  | Update | Request/Response | AMF |
|  | UpdateNotify | Subscribe/Notify | AMF |
|  | Delete | Request/Response | AMF |
| Npcf_Policy Authorization | Create | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
|  | Notify | Subscribe/Notify | AF, NEF |
|  | Subscribe |  | AF, NEF |
|  | Unsubscribe |  | AF, NEF |
| Npcf_SMPolicyControl | Create | Request/Response | SMF |
|  | UpdateNotify | Subscribe/Notify | SMF |
|  | Update | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
| Npcf_BDTPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| Npcf_UEPolicyControl | Create | Request/Response | AMF, V-PCF |
|  | Update | Request/Response | AMF, V-PCF |
|  | UpdateNotify | Subscribe/Notify | AMF, V-PCF |
|  | Delete | Request/Response | AMF, V-PCF |
| Npcf_EventExposure | Subscribe | Subscribe/Notify | NEF |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| Npcf_UEPolicyAuthorization | Create | Request/Response | AF, NEF |

THIRD CHANGE

##### 5.2.5.3.x	Npcf_UEPolicyAuthorization_Create service operation

Service operation name: Npcf_UEPolicyAuthorization_Create

Description: Authorize the request and send the policy to the UE as part of a URSP rule.

Inputs, Required: SUPI, Background Data Transfer Reference ID.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

FOURTH CHANGE

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | SMF |
|  | Trigger | Trigger | AF |
| Nnef_NIDD | Delivery | Request/Response | AF, SMF |
| Nnef_SMContext | Create | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request / Response | AF |
| Nnef_ApplyPolicy | Update | Request / Response | AF |

FIFTH CHANGE

##### 5.2.6.8.x	Nnef_ApplyPolicy_Update service operation

Service operation name: Nnef_ApplyPolicy Create

Description: The consumer requests to apply a policy to the UE.

Inputs (required): External Identifier, Reference ID for a previously negotiated policy of a background data transfer.

Inputs (optional): None.

Outputs (required): Transaction Reference ID, result.

Output (optional): None.

SIXTH CHANGE

##### 5.2.13.2.2	Nbsf_Management_Register service operation

Service Operation name: Nbsf_Management_Register.

Description: Registers the tuple (UE address(es), SUPI, GPSI, DNN, DN information (e.g. S-NSSAI), PCF id) or registers the tuple (SUPI, PCF id).

Inputs, Required: PCF id

Inputs, Optional: UE address(es), DNN, SUPI, GPSI, DN information (e.g. S-NSSAI)

UE address can contain IP address/prefix or Ethernet address as defined in TS 23.501 [2]. It can optionally refer to Frame Routes.

Frame Routes are defined in Table 5.2.3.3.1-1.

Outputs, Required: Result indication.

Outputs, Optional: None.

SEVENTH CHANGE

##### 5.2.13.2.3	Nbsf_Management_Deregister service operation

Service Operation name: Nbsf_Management_Deregister

Description: Removes the tuple (UE address(es), SUPI, GPSI, DNN, DN information (e.g. S-NSSAI), PCF id) or removes the tuple (SUPI, PCF id).

Inputs, Required: None.

Inputs, Optional: UE address(es), DNN, SUPI, GPSI, DN information (e.g. S-NSSAI)

UE address can contain IP address/prefix or Ethernet address as defined in TS 23.501 [2].

Outputs, Required: Result indication.

Outputs, Optional: None.

EIGHTH CHANGE

##### 5.2.13.2.4	Nbsf_Management_Discovery service operation

Service Operation name: Nbsf_Management discovery

Description: Discovers the PCF selected for the tuple (UE IP address(es), SUPI, GPSI, DNN, DN information (e.g. S-NSSAI)) or discovers the PCF that is serving the UE for AM policies.

Inputs, Required: None. UE IP address, DNN [Conditional], DN information (e.g. S-NSSAI) [Conditional]

Inputs, Optional: SUPI, GPSI, UE IP address, DNN [Conditional], DN information (e.g. S-NSSAI) [Conditional]

Outputs, Required: PCF id

Outputs, Optional: None.