3GPP TSG SA2 Meeting #133	S2-1905379

# Reno, USA, May 13 – 17 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 23.228 | CR | xxxx | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding SLF services to 5GC for public identifiers in NF discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eIMS5G_SBA |  |  |  |  |  | Date: |  |  | 05/06/2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NF discovery in the NRF may utilize subscriber identifiers – however 5GC assumes that private identifiers will be available to the NF performing the discovery. For IMS the CSCF may only have the public identifer which must be mapped to the private identifier to ensure reliable NF discovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describes HSS discovery using SLF and NRF using IMPU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Public identity cannot be used to determine network function instance ID. IMS CSCF (acting as a AF) will not be able to use service based interfaces to interact with 5GC as it will not be able to determine the HSS instance for a subscriber in response to a SIP invite containing only IMPU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Y.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

# Y.X	HSS Discovery

In IMS networks where multiple HSS instances are deployed the CSCF or AS will need to determine which instance to invoke for the HSS service.  For service-based HSS instances the following apply:

-	Unless local configuration is used, when a HSS instance is initialized it shall perform NF function registration indicating the supported SUPI values (ranges, domains etc), as described in TS 23.501[93].

-	If the network uses proxy routing or indirect routing then the routing network may be configured to resolve the HSS instance, in this case the CSCF or AS may send the HSS service request using either the IMPU or SUPI (derived from IMPI) as the target address.  The routing network shall then perform the necessary actions to determine the correct instance to route the request to.

Otherwise:

-	If the CSCF or AS knows the IMPI it shall derive the SUPI and use the NF discovery procedure (using SUPI) to determine the HSS instance as described in TS 23.501 [93].

-	If the CSCF or AS only knows the IMPU then it shall use the NF discovery procedure (using IMPU) to determine the HSS instance as described in TS 23.501 [93].  Based on implementation the CSCF or AS may invoke the SLF directly to obtain SUPI (or instance ID) or the NRF may invoke the SLF on behalf of the CSCF or AS.

NOTE:	For a subscriber that only has 4G subscription information, a suitable SUPI based on their IMSI can be derived if they are supported by a service-based HSS and returned to ensure appropriate routing occurs.

In networks with mixed support for HSS supporting Diameter and service-based interfaces the CSCF/AS shall invoke the service-based discovery first, if the response does not include an instance ID or SUPI the CSCF/AS may try to use Diameter interfaces instead (based on local configuration) toward the HSS.

*** End of Changes ***