3GPP TSG-SA WG2 Meeting #133	S2-1905684

# RENO, NEVADA, USA, MAY 13 – 17, 2019	(revision of S2-190xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1429 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of restrictions on 5G VN group communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Each particular 5G LAN-type service (i.e. a 5G VN) may have an allowed area. The 5GS shall configure and enforce restrictions on 5G VN group communication, e.g. based on location of group member, for a particular 5G VN. When UE is the member of a 5G VN group with an allowed area: - At UE side, The UE restricts communication within this 5G VN with respect to the 5G VN specific service area: - When UE is out of the 5G VN specific service area, the UE shall not initiate the group communication. - When UE is in the 5G VN specific service area, the UE can initiate the group communication. - At network side, The network obtains the 5G VN group’s allowed area and authorizes the UE restricing to the group communication for a particular 5G LAN-type service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of restrictions on 5G VN group communication |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for 5G LAN-type service |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29.2, 5.29.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

5.29	Support for 5G LAN-type service

5.29.2	5G LAN group management

5G System supports 5G LAN Group Management can be configured by a network administrator or can be managed dynamically by AF.

The 5G LAN group member is uniquely identifed by GPSI. The group as described in clause 5.2.3.3.1 of TS 23.502 [3] is applicable to 5G LAN-type services.

In order to support dynamic 5GLAN Group Management, the NEF exposes a set of service to manage (e.g. add/delete) 5G LAN group and 5G LAN member.

The 5G LAN group configuration is either provided by OA&M or provided by an AF to the NEF. The 5G LAN group configuration is stored in the UDR.

When configuration is provided by an AF, the NEF service operations information flow procedure described in TS 23.502 [3] clause 4.15.6.2 applies. The 5G LAN group configuration in UDR may include the following parameters: GPSI (i.e. UE Identities of the 5G LAN group), PDU session type, DNN, S-NSSAI.

The 5G VN group configuration may include the group area where the group communication is allowed for the group member. The 5GS is informed of the group area to enforce restrictions on the group communication.

The third party AF may update the UE Identities of the 5G LAN group at any time after the initial provisioning.

Editor's note:	Relation between DNN and 5G LAN group, e.g. whether and how a 1:1 and/or 1:N relation is supported, is FFS.

The PCF delivers 5G LAN group configuration information (DNN, S-NSSAI, PDU session type) to the UE for each GPSI that belongs to a 5G-LAN group. The 5G LAN group configuration information is delivered in the URSP from the PCF to the UE using the UE Configuration Update procedure for transparent UE Policy delivery as described in TS 23.502 [3] clause 4.2.4.3 and TS 23.503 [45] clause 6.1.2.2.

5.29.5	Restriction on 5G LAN-type service

The 5G LAN-type service can span over wide area mobile network or restrict to a certain service area. The 5GS shall configure and enforce restrictions on 5G VN group communication based on location of group member for a particular 5G LAN-type service (i.e. a 5G VN). When the UE is the member of a 5G VN group with the group area, if the UE is out of the 5G VN specific service area, the UE shall not initiate the group communication; if the UE is in the 5G VN specific service area, the UE can initiate the group communication. The network shall authorize the UE using 5G VN group communication according to the group area of the 5G VN group.

**** End of Changes *****