3GPP TSG-SA WG2 Meeting #133	S2-1905690

# RENO, NEVADA, USA, MAY 13 – 17, 2019 	(revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1431 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of User Plane Optimisations in Preferred and Supported Network Behaviour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A UE includes in its Preferred Network Behaviour indication the Network Behaviour the UE can support and what it would prefer to use. In SA2#132, User Plane CIOT 5GS Optimization is adopted for small data transmission. The following information is added in the Preferred Network Behaviour: - Support of User Plane CIOT 5GS Optimization, - Preference between Control Plane CIoT 5GS Optimisation and User Plane CIOT 5GS Optimization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following information in the PNB for 5GC and EPC: - upport of User Plane CIOT 5GS Optimization, - Preference between Control Plane CIoT 5GS Optimisation and User Plane CIOT 5GS Optimization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PNB for 5GC and EPC is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.2, 5.31.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

5.31.2	Preferred and Supported Network Behaviour

At registration, a UE includes its 5G Preferred Network Behaviour indicating the network behaviour the UE can support and what it would prefer to use.

NOTE:	If the UE supports S1-mode then the UE will indicate the supported EPS Network Behaviour Information in the S1 UE network capability IE.

The 5G Preferred Network Behaviour includes this information:

-	Whether Control Plane CIoT 5GS Optimisation is supported.

-	Whether User Plane CIoT 5GS Optimisation is supported.

-	Whether Control Plane CIoT 5GS Optimisation is preferred or whether User Plane Plane CIoT 5GS Optimisation is preferred.

-	Whether N3 data transfer is supported.

-	Whether header compression for Control Plane CIoT 5GS Optimisation is supported.

If N3 data transfer is supported is indicated by the UE, the UE supports data transfer that is not subject to CIoT 5GS Optimisations. If the UE indicates support of User Plane CIoT 5GS Optimisation then it shall also indicate support of N3 data transfer.

The AMF indicates the network behaviour the network accepts in the 5G Supported Network Behaviour information. This indication is per Registered Area. The AMF may indicate one or more of the following:

-	Whether Control Plane CIoT 5GS Optimisation is supported.

-	Whether User Plane CIoT 5GS Optimisation is supported.

-	Whether N3 data transfer is supported.

-	Whether header compression for Control Plane CIoT 5GS Optimisation is supported.

If the AMF indicates support of User Plane CIoT 5GS Optimisation then it shall also indicate support of N3 data transfer.

For NB-IoT UEs that only support Control Plane CIoT 5GS Optimisation, the AMF shall include support for Control Plane CIoT 5GS Optimisation in the Registration Accept message.

A UE that supports WB-E-UTRA shall always indicate support for N3 data transfer.

The 5G Preferred Network Behaviour indication from the UE may be used to influence policy decisions that can cause rerouting of the Registration Request from an AMF to another AMF.

**** Second Change ****

5.31.3	Selection, steering and redirection between EPS and 5GS

The UE selects the core network type (EPC or 5GC) based on the broadcast indications for both EPC and 5GC, and the UE's EPC and 5GC Preferred Network Behaviour. For a nework that supports NB-IoT, it shall broadcast an indication of whether N3 data transfer is supported or not in system information.

When the UE performs the registration procedure it includes its Preferred Network Behaviour (for 5G and EPC) in the Registration Request message and the AMF replies with the 5G Supported Network Behaviour in the Registration Accept message.

In the networks that support CIoT features in both EPC and 5GC, the operator may steer the UEs from a specific CN type due to operator policy, e.g., due to roaming agreements, Preferred and Supported Network Behaviour, load redistribution, etc. Operator policies in EPC and 5GC are assumed to avoid steering UEs back and forth between EPC and 5GC.

To redirect a UE from 5GC to EPC, when the UE requests Registration, the AMF sends a Registration Reject with an EMM cause value indicating the UE should not use 5GC. The UE disables N1 mode and re-enables S1 mode, if it was disabled. The UE then performs either an Attach or TAU with EPC as described in clause 5.17.2.

To redirect a UE from EPC to 5GC, when the UE requests an Attach or TAU procedure, the MME sends a reject message with an EMM cause indicating the UE should not use EPC. The UE disables S1 mode and re-enables N1 mode, if it was disabled. The UE then performs a Registration with 5GC as described in clause 5.17.2.

When determining whether to redirect the UE the AMF/MME takes into account the UE support of S1/N1 mode (respectively) and the UEs (5G) Preferred Network Behaviour and the (5G) Supported Network Behaviour of the network the UE is being redirected towards.

If after redirection the UE cannot find a cell supporting connectivity, the UE may re-enable the disabled N1/S1 mode and then perform Registration, Attach or TAU.

**** End of Changes *****