3GPP TSG-SA WG2 Meeting #133 	S2- 1905759

Reno, Nevada, USA, 13 - 17 May, 2019	(revision of S2-19xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1123 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TSN QoS mapping and 802.1Qbv parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | DP S2-1903376 “ TSN QoS parameter mapping” explains the background for identifying a minimum set of parameters from TSN system that should be included in the TSN-5GS mapping table, as well as the fundaments to build this mapping table. This CR also addresses the EN on the location of the mapping table and the analysis is described in DP “TSN-5GS QoS mapping partly at AF and PCF” as described in S2-1903374. Also DP “Principle of 5G bridge port configuration” S2-1903376 introduces traffic class to QoS Flow mapping principle. DP :” Support for IEEE 802.1Qbv scheduling” S2-1903378 analyses which parameters and addresses the EN about additional Qvb parameters needed or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Identify specific parameters from TSN standards as a minimum requirements for 5G QoS to be able to fulfil the service requirements. Specify the fundaments to build the TSN-5GS mapping table. The TSN AF is responsible for interacting with TSN network to collect the related information and mapping these information into appropriate service requirements towards PCF using existing AF to PCF communication. AF may provide additional information, so combined with this and PCF internal mapping table, PCF provides the appropriate 5QI value(s) towards the PDU session. Include additional set of parameters to be Qvb compliant with minimal set feasible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the TSN-5GS QoS mapping and Qbv support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29.1, 5.29.2, 5.29.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.29.1    5GS logical TSN bridge management

5GS functions acts as one or more TSN Bridges of the TSN network. The 5GS Bridge is composed of the ports on the UPF (i.e. PSA) side, the user plane tunnel between the UE and UPF, and the ports on the UE/TT side. For each 5GS Bridge of a TSN network, the ports on the UPF/TT side support the connectivity to the TSN network, the ports on the UE/TT side are associated to the PDU Session providing connectivity to the TSN network.

Editor's note: The granularity of the 5GS Bridge is FFS.

In order to support TSN traffic scheduling over 5GS Bridge, the 5GS supports the following functions:

-    Report the bridge information of 5GS Bridge to TSN network.

-    Map the configuration information obtained from TSN network for 5GS Bridge into 5GS QoS within PDU Session and TSC Assistance Information for efficient time-aware scheduling, as defined at clause 5.29.2.

The bridge information of 5GS Bridge is used by the TSN network to make appropriate management configuration for the 5GS Bridge. The bridge information of 5GS Bridge includes the following:

-    Bridge ID of 5GS Bridge.

-    Capabilities of 5GS Bridge as defined in 802.1Qcc:

-    5GS Bridge delay per port pair per traffic class, including 5GS Bridge delay (dependent and independent of frame size, and their maximum and minimum values: independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), ingress port ID, egress port ID and traffic class.

-    Propagation delay per port (txPropagationDelay), including transmission propagation delay, egress port ID.

-    Topology of 5GS Bridge as defined in 802.1AB.

-    Traffic classes and their priorities per port as defined in IEEE 802.1Q.

The following parameters: independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin, and txPropagationDelay, how to calculate them is left to implementation and not defined in this standard.

Editor's note: It is FFS whether and how the topology of 5GS Bridge as defined in 802.1AB is needed and if additional information is needed.

The AF is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the TSN network.

Editor's note: How and when to collect/generate the user plane information of 5GS Bridge within 5GS is FFS.

### 5.29.2	QoS parameters mapping for 5GS Bridge configuration

In order to schedule TSN traffic over 5GS Bridge, the configuration information of 5GS Bridge is mapped to 5GS QoS within the corresponding PDU Session. The configuration information of 5GS Bridge as defined in 802.1Qbv [96], includes the following:

-	Bridge ID of 5GS Bridge.

-	Configuration information of scheduled traffic on ports of the TT:

-	Egress ports of 5GS Bridge, e.g., ports on UE side or ports on UPF side.

-	Traffic classes and their priorities.

-	Ingress ports of 5GS Bridge, if needed.

-	AdminBaseTime and AdminControlList with its GateControlEntries that include: GateStateValue and TimeIntervalValue.

-	Configuration parameters to be exposed to the CNC:

-	TickGranularity and ExclusiveGating.

The following “optional” parameters may be configured by the CNC: AdminCycleTime and AdminControlListLength.

NOTE: In this release of the specification, only support simplified IEEE 802.1Qbv [96], Annex Q.2 for 5GS with “exclusive gating mechanism”. The execution/operation of 5GS output pacing/scheduling for IEEE 802.1Qbv support (i.e. implementation of Qbv scheduling state machines) is out of 3GPP scope.

The association between binding information in the UE, 5GS Bridge ID and port on UE side is maintained and further used to assist to bind the TSN traffic with the UE.

Editor's note: The details on the binding information in the UE is FFS.

### 5.29.x	QoS mapping tables

The mapping tables between the traffic class and 5GS QoS Profile is provisioned and further used to find suitable 5GS QoS profile to transfer TSN traffic over the PDU Session. QoS mapping procedures are performed in two phases: (1) QoS capability report phase as described in clause 5.29.1, and (2) QoS configuration phase as in 5.29.2.

(1) The TSN Translator AF shall be pre-configured via OAM the mapping table and represents what is being reported or exposed to the TSN system (i.e. the CNC) through the relevant TSN information objects, such as the Traffic Class Table for every port, see the IEEE 802.1Q [X] and IEEE 802.1Qcc [95].

(2) CNC distributes the TSN QoS requirements and TSN scheduling parameters (specific for current node) to 5G virtual bridge via TSN AF. Alternatively, the 5GS virtual bridge may pre-request or query CNC for the TSN QoS and traffic information.

The PCF mapping table provides a mapping from TSN QoS information to 5GS QoS profile. Based on trigger from TSN AF, the PCF may trigger PDU session modification procedure to establish a new 5G QoS flow for the requested traffic class according to the selected QoS policies from the TSN AF traffic requirements.

Figure 5.29.2-x illustrates the functional distribution of the mapping tables.

Figure 5.29.3-x QoS Mapping Function distribution between PCF and TSN AF

The minimum set of TSN QoS-related parameters that are relevant for mapping the TSN QoS requirements in the 5GS are specified in clause 5.29.1.

QoS mapping table between port traffic classes and 5QI should be matching the delay and priority, while preserving the priorities in the 5GS. An operator enabling TSN services via 5GS can choose up to eight traffic classes to be mapped to 5GS QoS profiles. The decision may be based on, for example, the requirements from factory needs and applications using TSN. The selection of up to eight 5QIs should enable the needs for deterministic services and TSC.

Once the set of 5QIs to be used for TSN is set, then it is possible to enumerate as many bridge port traffic classes as the number of selected 5QIs. The lowest priority value in 5QI maps to the highest priority value of the port traffic class. The rest of priorities in 5QIs are ordered in ascending values, and the port traffic class is ordered in descending values. In this way they are matched one to one, while preserving the 5QIs priority levels.  This mapping table includes the parameters that are reported to the CNC as possible QoS classes that can be configured in the 5GS virtual bridge: Class, Priority, and Delay (where PDB+ UE residence time is mapped to independentDelayMax) mapped from the respective 5QI. There are also parameters that are applicable to the 5GS such as PER, MDBV, and the averaging window.

Once the CNC has received the necessary information, it proceeds to calculate scheduling and paths. The configuration information is then set in the bridge per port and per traffic class. The most relevant information received is the scheduling for every traffic class and port of the bridge. At this point, it is possible to retrieve the real QoS requirements by identifying the traffic class of the port. Then the traffic class to 5QI mapping can be performed using the QoS mapping table. Subsequently, the real QoS flow can be now configured using the 5QI retrieved from the QoS mapping table. This feedback approach uses the reported information to the CNC and the feedback of the configuration information coming from the CNC to perform the mapping and configuration in the 5GS. The scheduling configuration information per traffic class is mapped to trigger creation/modification of a QoS flow in 5GS.

In a 5G virtual bridge, each traffic class may be mapped into a QoS flow. In case the traffic classes need to be differentiated within 5GS, then the different traffic classes can not be mapped into same QoS flow.

*** END CHANGES ***