3GPP TSG-SA2 Meeting #133	S2-1905768

# Reno, Nevada, United States, 13th May 2019 - 17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1446 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NF Service Bulk Context Transfer Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom AG, KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The eSBA Study concluded to specify transfer of Service Context of the NF/NF Service from the Source NF/NF Service to the Target NF/NF Service in order to further improve the 5GC towards cloud environments. This CR covers the use case in which the OAM system would require (e.g. based on system load distribution or planned maintenance) to transfer contexts between instances (e.g. transfer contexts between sets). For such a use case, individual transfer of contexts would not be efficient (e.g. transfer of thousands of IoT contexts would require thousands of individual requests). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR includes the following changes: - Introduction of Nsmf_ContextTransfer service - Addition of bulk transfer procedures - Corrected typo in Table 5.2.8.1-1: The operation is called ContextRequest, not Context and other minor typos |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions of eSBA study on NF/NF Service Context Transfer procedures would not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 4.xx.3 (new), 5.2.X (new), 5.2.8.3.1, |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### **** First Change ****

### 4.xx.3 Context Bulk Transfer

Figure 4.xx.3-2: NF/NF Service Context Bulk Transfer procedures

- [Prerequisite] In order to be able to notify the results of the context transfer to the requester, a subscription to context transfer Events is required.

- [Prerequisite] In order to be able to notify the results of the context transfer to any NF/NF Service making use of the context, a subscription to context transfer Events is required.

- A bulk context transfer request (e.g. for UE context in case of an AMF or SM context in case of an SMF) is sent, e.g. by an OAM system or an NF/NF Service, to the source NF/NF Service. The request includes a list of the requested context IDs, an endpoint for the source NF/NF Service to push the requested contexts to, and the maximum request size.

- If the source NF/NF service acknowledges the Context Bulk Request without an error, a transaction ID and a transaction validity time are returned.

- The source NF/NF Service pushes a set of contexts to the target NF/NF Service as soon as they are available for transfer. This may trigger several other procedures that ensure all necessary NF/NF services are being updated and set up with necessary information about the new context location.
The request size shall not exceed the maximum request size received in step 3.

- [Optional] If the requested contexts would result in a request bigger than the maximum request size received in step 3, additional requests containing further contexts are sent. This step may be repeated as many times as needed.

- After all contexts have been transferred or after the end of the validity time, the  the source NF/NF Service notifies als subscribers of Context Transfer Events a list of the contexts that were successfully transferrend and of the contexts that could not be transferred, optionally with a cause.

*************** Next change  ***************

#### 5.2.8.1	General

The following table shows the SMF Services and SMF Service Operations.

Table 5.2.8.1-1: NF services provided by the SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsmf_PDUSession | Create | Request/Response | V-SMF/I-SMF |
|  | Update | Request/Response | V-SMF/I-SMF, H-SMF |
|  | Release | Request/Response | V-SMF/I-SMF |
|  | CreateSMContext | Request/Response | AMF |
|  | UpdateSMContext | Request/Response | AMF |
|  | ReleaseSMContext | Request/Response | AMF |
|  | SMContextStatusNotify | Subscribe/Notify | AMF |
|  | StatusNotify | Subscribe/Notify | V-SMF/I-SMF |
|  | Context | Request/Response | AMF, I-SMF |
| Nsmf_ContextTransfer | ContextBulkTransferRequest | Request/Response | Any |
|  | ContextBulkPush | Request/Response | SMF |
| Nsmf_EventExposure | Subscribe | Subscribe/Notify | NEF, AMF |
|  | Unsubscribe |  | NEF, AMF |
|  | Notify |  | NEF, AMF |
|  | AppRelocationInfo |  | AF |

*************** Next change  ***************

#### 5.2.8.X	Nsmf_ContextTransfer Service

##### 5.2.8.X.1	General

Service description: This service allows contexts to be transferred to the SMF and to request bulk transfer of contexts. The following are the key functionalities of this NF service:

##### 5.2.8.X.2	Nsmf_ContextTransfer_ContextBulkTransferRequest service operation

Service operation name: Nsmf_PDUSession_ ContextBulkRequest.

Description: This service operation is used by the requester (e.g. OAM, SMF) as Service Consumer to request an asynchronous transfer of a list o SM Contexts to another SMF as NF Service Producer.

Input, Required: List of SM Context IDs, ContextBulkPush target endpoint, maximum ContextBulkPush request size.

See Table 5.2.8.2.10-1 for single SM Contexts that may be transferred.

Output, Required: Result Indication, transaction ID, transaction validity time.

Output, Optional: Cause.

See clause 4.xx.3 for an example usage of this service operation.

##### 5.2.8.X.3	Nsmf_ContextTransfer_ContextBulkPush service operation

Service operation name: Nsmf_PDUSession_ContextBulkPush.

Description: This service operation is used by the SMF as Service Consumer to push a set of SM Contexts to another SMF as NF Service Producer.

Input, Required: List of SM Contexts of identified PDU sessions, ContextBulkTransferRequest transaction ID.

See Table 5.2.8.2.10-1 for single SM Context that may be transferred.

Output, Required: Result Indication.

Output, Optional: Cause.

See clause 4.xx. 3 for an example usage of this service operation.

*************** Next change  ***************

##### 5.2.8.3.1	General

Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on PDU Sessions. The following are the key functionalities of this NF service.

-	Allow consumer NFs to Subscribe and unsubscribe for an Event ID on PDU Session(s); and

-	Notifying events on the PDU Session to the subscribed NFs.

-	Allow consumer NFs to acknowledge or respond to an event notification.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	UE IP address / Prefix change: The event notification may contain a new UE IP address / Prefix or an indication of which UE IP address / Prefix has been released.

-	PDU Session Release.

-	UP path change: a notification corresponding to this event is sent when the UE IP address / Prefix and / or DNAI and /or the N6 traffic routing information has changed.

The event notification may contain following information:

-	the type of notification ("EARLY" or "LATE").

-	for both the source and target UP path between the UE and the DN, the corresponding information is provided when it has changed:

-	DNAI.

-	UE IP address / Prefix.

-	N6 traffic routing information.

NOTE 1:	UP path change notification, DNAI and N6 traffic routing information are further described in TS 23.501 [2] clause 5.6.7.

-	Change of Access Type; The event notification contains the new Access Type for the PDU Session.

-	PLMN change; The event notification contains the new PLMN Identifier for the PDU Session.

-	Downlink data delivery status. The event notification contains the status of downlink data packets in the core network including:

-	Downlink packet(s) in extended buffering and Estimated maximum wait time.

-	Downlink packet(s) discarded.

-	Downlink packet(s) transmitted.

-	Context Transfer.

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for SMF exposure.

Table 5.2.8.3.1-1: Example of Event Filters for SMF exposure events

| Event ID for SMF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| DNAI Change | None |
| PDU Session Release |  |
| SM context Transfer |  |

The target of SMF event reporting may correspond to a PDU Session ID, an UE ID (SUPI) an Internal Group Identifier or an indication that any UE is targeted (on a specific DNN).

The consumer NF can provide the following event-specific information when acknowledging an event notification:

-	For UP path change event:

-	N6 traffic routing information related to the target DNAI.

NOTE 2:	Acknowledgement to a UP path change event notification is further described in TS 23.501 [2] clause 5.6.7.

*************** End of changes  ***************