3GPP TSG-SA WG2 Meeting #133	S2-1906108

13–17 May 2019, Reno, China	(Revision of S2-1905535)

Source:	Huawei, HiSilicon

Title:	Update the input data and output analytics for UE mobility analytics and UE communication analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This contribution proposes to:

- Add the Type Allocation code (TAC) as the input data of the UE Mobility and Communication analytics.

- Add the UE group ID into the UE Mobility analytics and UE Communication analytics.

- Discussion

Observation 1):

Type Allocation Code (TAC) is the initial eight-digit portion of the 15-digit IMEI or 16-digit IMEISV codes, which identifies a particular model (and often revision) of vendor for wireless network, and is defined in clause 6.2.1, TS 23.003:

In general, devices with the same TAC may have the similar expected behavior, for example, electricity meters made by company A may have the same TAC, and they never move and communicate to the network once a month. Thus, devices with the same TAC may have similar mobility and communication behavior. Thus, TAC can be an input data and output analytics for UE mobility and communication parameters analytics.

From security aspect, devices with the same TAC may also have the similar abnormal behavior. The attacker always hijacks the devices who have the same vulnerability using malicious application, and utilizes them to trigger some attack, e.g. DDoS attack. The devices with the same TAC may have the same vulnerability, e.g. software vulnerability, hardware vulnerability, etc. because they are batch production, once a vulnerability exists, all the devices are suffering the same vulnerability. For example, Mirai compromises thousands of camera devices using the same vulnerability to trigger a DDoS attack to DNS Server. Those devices are specific versions of specific companies. Thus, TAC can be an input data and output analytics for abnormal behavioral related network data analytics too.

It is proposed to add the TAC as the input data and output analytics for the mobility analytics, communication analytics and abnormal behavioral related network data analytics.

- Proposal

It is proposed to add the TAC as the input data and output analytics for the mobility analytics.

**** First Change ****

6.10	UE mobility analytics

6.10.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from 5GC, OAM and/or AFs, and to perform data analytics to provide UE mobility statistics or predictions.

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE.

-	An Observation period indicates the time when the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics (low/high).

6.10.2	Input Data

The NWDAF supporting data analytics on UE mobility shall be able to collect UE mobility information from OAM, 5GC and AFs. The detailed information collected by the NWDAF could be MDT data from OAM, network data from 5GC and/or service data from AFs:

-	UE mobility information from OAM is UE location carried in MDT data;

-	Network data related to UE mobility from 5GC is UE location information as defined in the Table 6.10.2-1;

Table 6.10.2-1: UE information collected from 5GC

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | TA or cells that the UE enters |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar mobility behavior. The UE whose mobility behavior is unlike other UEs with the same TAC may be an abnormal one. |

-	Service data related to UE mobility provided by AFs is defined in the Table 6.10.2-2;

Table 6.10.2-2: Service Data from AF related to UE mobility

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Application ID | Identifying the application providing this information |
| UE trajectory (1..max) | Timestamped UE positions |
| >UE location | Geographical area that the UE enters |
| >Timestamp | A time stamp when UE enters this area |

**** Next Change ****

6.11	UE Communication Analytics

6.11.1	General

In order to support some optimized operations, e.g. customized mobility management or QoS improvement, in 5GS, an NWDAF may perform data analytics on UE communication pattern and user plane traffic, and provide the analytics results (i.e. UE communication statistics or prediction) to NFs in the 5GC.

An NWDAF supporting UE Communication Analytics collects per-application communication description from AFs. If consumer NF provides an Application ID, the NWDAF only considers the data from AF and SMF that corresponds to this application ID.

6.11.2	Input Data

The NWDAF supporting data analytic on UE communication shall be able to collect communication information for the UE from 5GC. The detailed information collected by the NWDAF includes service data related to UE communication as defined in the Table 6.11.2-1.

Table 6.11.2-1: Service Data from 5GC related to UE communication

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AF | Could be GPSI or external UE ID |
| Correlation information | AF | Identifying the application generating the traffic |
| Application ID | AF | Identifying the application providing this information |
| Communication pattern parameters | AF | Same as communication pattern specified in TS 23.502 |
| UE communication (1..max) | AF | Communication description per application |
| >Communication start |  | The time stamp that this communication starts |
| >Communication stop |  | The time stamp that this communication stops |
| >UL data rate |  | UL data rate of this communication expected by the application |
| >DL data rate |  | DL data rate of this communication expected by the application |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar communication behavior. The UE whose communication behavior is unlike other UEs with the same TAC may be an abnormal one. |

The correlation information may be used to correlate the communication information collected from an AF with the communication information collected from SMF(s). Other correlations can be made within the NWDAF using UE communication time pattern.

Editor's Note:	 the exact nature of correlation information (e.g. IP address 5-tuple) is FFS.

**** Next Change ****

6.8	Abnormal behaviour related network data analytics

6.8.1	General

This clause defines how NWDAF identifies a group of UEs or a specific UE being misused or hijacked based on the expected UE behavioural parameters.

The NWDAF performs data analytics on abnormal behaviour if there is a related subscription request from a 5GC NF (e.g. AMF, SMF, or PCF), OAM or an AF.

The consumer of this analytics shall indicate in the request:

-	AnalyticsId set to "Abnormal behaviour”

-	The Target of Analytics Reporting can be one UE, any UE or an Internal Group Identifier.

-	Optionally an Application ID, DNN or S-NSSAI

-	An Observation period indicates the time when the statistics or predictions are requested.

-	Analytics Filter Information: optional expected behaviour, list of risk IDs to monitor with associated thresholds

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

When the NWDAF detects those UEs that deviate from the expected UE behavioural information, e.g. unexpected UE location, abnormal traffic pattern, wrong destination address etc., the NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.8.3.

6.8.2	Input Data

The expected UE behavioural parameters that provided to the NWDAF are defined in Table 6.7.3-1.

The UE behavioural information collected from 5GC NFs is as specified in clause 6.7.2.

On request of the service consumer, the NWDAF shall collect and analyse the behaviour analytics from the 5GC NFs (SMF, AMF) or AF.

6.8.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are invoked to notify consumer NFs. A new Analytics ID named "Abnormal behaviour" is defined.

Corresponding to the Analytics ID, the analytics result provided by the NWDAF is defined in Table 6.8.3-1. The NWDAF shall notify the consumer of the exception(s) with appropriate exception ID if the exceptions level trespasses above or below the threshold.

Table 6.8.3-1: Abnormal behaviour Analytics

| Information | Description |
| --- | --- |
| UE ID list | Could be SUPI, Internal-Group-Identifier, external UE ID, TAC which indicates UE being subject to the risk(s) identified by the Exception ID(s) |
| Exceptions (1..max) |  |
| >Exception ID | The risk detected by NWDAF |
| >Exception Level | Measured level, compared to the threshold |
| >Exception trend | Measured trend (up/down/unknown/stable) |
| >Additional measurement | Specific information for each risk |

If PCF subscribes notifications on “Abnormal behaviour”, the NWDAF shall send the PCF notifications about the risk, which may trigger the PCF to update the AM/SM policies.

The NWDAF also sends the notification directly to the AMF or SMF, if the AMF or SMF subscribes the notification, so that the AMF or SMF may, based on operator local policies defined on a per S-NSSAI or per (DNN,S-NSSAI), take actions for risk solving. The following Table 6.8.3-2 gives examples of AM/SM policies and corresponding actions for solving each risk.

Table 6.8.3-2: Examples of policies and actions for risk solving

| Exception ID and description | AM/SM policy | Actions of NFs |
| --- | --- | --- |
| Unexpected UE location | Add the area of current UE location into mobility restriction | PCF may extend the Service Area Restrictions. AMF may extend the mobility restriction |
| Unexpected long-live/large rate flows | Decrease the MBR for the related QoS flow | SMF updates the QoS rule. PCF, if dynamic PCC applies for corresponding DNN, S-NSSAI, updates PCC Rules that triggers SMF updates the QoS rule. |
| Unexpected wakeup | Apply MM back-off timer to the UE | AMF applies MM back-off timer to the UE |
| Suspicion of DDoS attack | Release the PDU session and Apply SM back-off timer | PCF may request SMF to release the PDU session. SMF may release the PDU session and applies SM back-off timer |
| Wrong destination address | Update the packet filter of the related QoS flow to block the wrong SDF | PCF updates the packet filter in the PCC Rules that triggers the SMF to update the related QoS flow and configures the UPF |
| Ping-pong stationary UE | NWDAF notifies the AMF or AF (Service Provider) | AMF may adjust UE registration area. |
| Too frequent Service Access/Abnormal traffic volume | NWDAF notifies AF (Service Provider) |  |

*************** End of changes ***************