3GPP TSG-SA WG2 Meeting #133	S2-1906117

# Reno, Nevada, USA, 13 - 17 May 2019													(was 6039, 5047)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0258 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DNN replacement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2019-05-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed in S2-1905044 (/// DP) there could be a need to replace a UE requested but unsupported DNN (similarily as agreed for EPS in CR on 23.401 in S2-182371) as well as a UE requested but not wanted DNN, in accordance with operator policies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The SMF Selection related policy is added to let PCF control handling of UE requested unsupported DNN as well as DNN replacement of UE requested DNN(s). A new PCRT condition “SMF selection rules” is defined in Table 6.1.2.5-1 as a trigger for the AMF to request DNN replacement. New AMF SMF selection related policy information in the form of SMF selection rules is defined in Table 6.x-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handling of non-supported or unwanted DNN is not available for 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x (new), 5.3.3, 6.1.2.x (new), 6.1.2.5, 6.2.1.2, 6.2.1.4, 6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

************** Start of changes ***************

### 4.2.x	SMF Selection related policy (CN policy) control requirements

SMF selection control refers to the PCF capability to activate/deactivate SMF selection policies in the AMF for the purpose of:

-	the AMF rejecting an unsupported DNN or triggering a DNN replacement request to the PCF as described in TS 23.501 [2], clause 5.6.1.

-	the AMF triggering a DNN replacement request to the PCF for a DNN that is indicated as requiring DNN replacement, as described in TS 23.501 [2], clause 5.6.1.

************** Next change ***************

### 5.3.3	Interactions between PCF and AMF

Npcf and Namf enable the PCF to provide Access and Mobility Management related policies to the AMF and support the following functionality:

-	Creation of an AM Policy Association as defined in clause 4.16 of TS 23.502 [3];

-	Request for access and mobility management related policies from the AMF to the PCF when a Policy Control Request Trigger related to Access and Mobility Management has been met;

-	Provision of access and mobility management decision from the PCF to the AMF;

-	Deletion of an AM Policy Association as defined in clause 4.16 of TS 23.502 [3];

-	Creation of an UE Policy Association as defined in clause 4.16 of TS 23.502 [3];

-	Notification of changes to the PCF when a Policy Control Request Trigger related to UE access selection and PDU Session selection has been met;

-	Deletion of an UE Policy Association as defined in clause 4.16 of TS 23.502 [3];

-	Handling of transparent delivery UE access selection and PDU Session selection policy from PCF to the UE via the AMF.

-	Creation of an SMF Selection Policy Association as defined in clause 4.16 of TS 23.502 [3];

-	Request for DNN replacement from the AMF to the PCF when a Policy Control Request Trigger related to SMF selection has been met;

-	Provision of DNN replacement decision from the PCF to the AMF;

-	Deletion of an SMF Selection Policy Association as defined in clause 4.16 of TS 23.502 [3].

The N15 reference point is defined for the interactions between PCF and AMF in the reference point representation.

************** Next change ***************

#### 6.1.2.x	SMF selection policy control

The SMF selection policy control configures the AMF to request DNN replacement from the PCF.

The management of the SMF selection enables the PCF of the serving PLMN to provide the AMF with rules on; either rejection or request to the PCF for replacement of UE requested DNN(s) subject to replacement by request from the PCF, as described in TS 23.501 [2], clause 5.6.1.

If the DNN provided by the UE is not supported and AMF can not select an SMF by querying NRF, the AMF shall either reject the NAS Message containing PDU Session Establishment Request from the UE with a cause indicating that the DNN is not supported, or request PCF to perform a DNN replacement before continuing the PDU Session Establishment procedure.

If the DNN provided by the UE is indicated for replacement, the AMF shall request the PCF to perform a DNN replacement. During UE Registration procedure and at establishment of the SMF Selection policy association, the PCF may indicate the operator policies for the AMF to determine whether to reject or replace certain UE requested DNN. During PDU Session Establishment procedure and in case of DNN replacement, the PCF provides the selected DNN to the AMF. 
The AMF uses the selected DNN in the query towards the NRF for the SMF selection and provides both UErequested DNN and selected DNN to the selected SMF.

************** Next change ***************

#### 6.1.2.5	Policy Control Request Triggers relevant for AMF

The Policy Control Request Triggers relevant for AMF and 3GPP access type are listed in table 6.1.2.5-1 and define the conditions when the AMF shall interact again with PCF after the AM Policy Association Establishment, UE Policy Association Establishment or SMF Selection Policy Association Establishment.

The PCF provides Policy Control Request Triggers to the AMF indicating a specific UE (i.e. SUPI or PEI) in the Policy Association establishment and modification procedures defined in the TS 23.502 [3]. The Policy Control Request Triggers are transferred from the old AMF to the new AMF when the AMF changes.

The PCR triggers are not applicable any longer at termination of the AM Policy Association or termination of UE Policy Association.

Table 6.1.2.5-1: Policy Control Request Triggers relevant for AMF and 3GPP access type

| Policy Control Request Trigger | Description | Condition for reporting |
| --- | --- | --- |
| Location change (tracking area) | The tracking area of the UE has changed. | PCF (AM Policy, UE Policy) |
| Change of UE presence in Presence Reporting Area | The UE is entering/leaving a Presence Reporting Area | PCF (AM Policy, UE Policy) |
| Service Area restriction change | The subscribed service area restriction information has changed. | PCF (AM Policy) |
| RFSP index change | The subscribed RFSP index has changed | PCF (AM Policy) |
| Change of the Allowed NSSAI | The Allowed NSSAI has changed | PCF (AM Policy) |
| SMF selection rules | UE request for an unsupported DNN or UE request for a DNN indicated for replacement | PCF (SMF Selection Policy) |

NOTE:	In the following description of the Policy Control Request Triggers relevant for AMF and 3GPP access type, the term trigger is used instead of Policy Control Request Trigger where appropriate.

If the Location change trigger are armed, the AMF shall activate the relevant procedure which reports any changes in location as explained in TS 23.501 [2] clause 5.6.11 by subscribing with the Npcf_AMPolicyAssociation service or Npcf_UEPolicyAssociation service. The reporting is requested to the level indicated by the trigger (i.e. Tracking Area). The AMF reports that the Location change trigger was met and the Tracking Area identifier.

If the Change of UE presence in Presence Reporting Area trigger is armed, i.e. the PCF subscribed to reporting change of UE presence in a Presence Reporting Area, including a list of PRA ids. In addition, for "UE-dedicated Presence Reporting Area" a short list of TAs and/or NG-RAN nodes and/or cells identifiers is included. Then, the AMF shall activate the relevant procedure which reports any Change of UE presence in Area of Interest as explained in TS 23.501 [2], clause 5.6.11. The reporting is requested for the specific condition when target UE moved into a specified PRA. The AMF reports the PRA Identifier(s) and indication(s) whether the UE is inside or outside the Presence Reporting Area(s) to the PCF.

The Service Area restriction change trigger and the RFSP index change trigger shall trigger the AMF to interact with the PCF for all changes in the Service Area restriction or RFSP index data received in AMF from UDM. The reporting includes that the trigger is met and the subscribed Service Area restriction or the subscribed RFSP index provided to AMF by UDM, as described in clause 6.1.2.1.

The Change of the Allowed NSSAI trigger shall trigger the AMF to interact with the PCF if the Allowed NSSAI has been changed. The reporting includes that the trigger is met and the new Allowed NSSAI. The PCF may update RFSP index based on the Allowed NSSAI.

If the SMF selection rules triggers a DNN replacement, the AMF shall request DNN replacement from the PCF by providing the UE requested DNN. The request indicates that the DNN replacement trigger is met in the AMF and that a new DNN is requested. The PCF shall select DNN and provide the selected DNN to AMF.

************** Next change ***************

#### 6.2.1.2	Input for PCC decisions

The PCF shall accept input for PCC decision-making from the SMF, the AMF, the CHF if present, the UDR and if the AF is involved, from the AF, as well as the PCF may use its own predefined information. These different nodes should provide as much information as possible to the PCF. At the same time, the information below describes examples of the information provided. Depending on the particular scenario all the information may not be available or is already provided to the PCF.

The AMF may provide the following information:

-	SUPI;

-	The PEI of the UE;

-	Location of the subscriber;

-	Service Area Restrictions;

-	RFSP Index;

-	RAT Type;

-	GPSI;

-	Access Type;

-	Serving PLMN identifier;

-	Allowed NSSAI;

-	UE time zone;

-	Requested DNN.

NOTE 1:	The Access Type and RAT Type parameters should allow extension to include new types of accesses.

The UE may provide the following information:

-	OSId;

-	List of PSIs;

-	Indication of UE support for ANDSP.

The SMF may provide the following information:

-	SUPI;

-	The PEI of the UE;

-	IPv4 address of the UE;

-	IPv6 network prefix assigned to the UE;

-	Default 5QI and default ARP;

-	Request type (initial, modification, etc.);

-	Type of PDU Session (IPv4, IPv6, IPv4v6, Ethernet, Unstructured);

-	Access Type;

-	RAT Type;

-	GPSI;

-	Internal-Group Identifier

-	Location of the subscriber;

-	S-NSSAI;

-	NSI-ID (if available);

-	DNN;

-	PLMN identifier;

-	Application identifier;

-	Allocated application instance identifier;

-	Detected service data flow descriptions;

-	UE support of reflective QoS (as defined in clause 5.7.5.1 of TS 23.501 [2]);

-	Number of supported packet filters for signalled QoS rules (indicated by the UE as defined in clause 5.7.5.1 of TS 23.501 [2]);

-	3GPP PS Data Off status;

-	DN Authorization Profile Index (see clause 5.6.6 of TS 23.501 [2]);

-	Session AMBR (see clause 5.6.6 of TS 23.501 [2]).

The UDR may provide the information for a subscriber connecting to a specific DNN and S-NSSAI, as described in the sub clause 6.2.1.3.

The UDR may provide the following policy information related to an ASP:

-	The ASP identifier;

-	A transfer policy together with a reference ID, the volume of data to be transferred per UE, the expected amount of UEs and the network area information.

NOTE 2:	The information related with AF influence on traffic routing may be provided by UDR when the UDR serving the NEF is deployed and stores the application request.

The AF, if involved, may provide the following application session related information, e.g. based on SIP and SDP:

-	Subscriber Identifier;

-	IP address of the UE;

-	Media Type;

-	Media Format, e.g. media format sub-field of the media announcement and all other parameter information (a= lines) associated with the media format;

-	Bandwidth;

-	Sponsored data connectivity information;

-	Flow description, e.g. source and destination IP address and port numbers and the protocol;

-	AF application identifier;

-	AF-Service-Identifier, or alternatively, DNN and possibly S-NSSAI

-	AF Communication Service Identifier (e.g. IMS Communication Service Identifier), UE provided via AF;

-	AF Application Event Identifier;

-	AF Record Information;

-	Flow status (for gating decision);

-	Priority indicator, which may be used by the PCF to guarantee service for an application session of a higher relative priority;

NOTE 3:	The AF Priority information represents session/application priority and is separate from the MPS 5GS Priority indicator.

-	Emergency indicator;

-	Application service provider.

-	DNAI

-	Information about the N6 traffic routing requirements

-	GPSI

-	Internal-Group Identifier

-	Temporal validity condition

-	Spatial validity condition

-	AF subscription for early and/or late notifications about UP management events

-	AF transaction identifier;

The CHF, if involved, may provide the following information for a subscriber:

-	Policy counter status for each relevant policy counter.

The NWDAF, if involved, may provide the following slice specific network status analytic information:

-	Identifier of network slice instance.

-	Load level information of network slice instance.

In addition, the predefined information in the PCF may contain additional rules based on charging policies in the network, whether the subscriber is in its home network or roaming, depending on the QoS Flow attributes.

The 5QIs (see clause 5.7.4 of TS 23.501 [2]) in the PCC rule is derived by the PCF from AF or UDR interaction if available. The input can be SDP information or other available application information, in line with operator policy.

The Allocation and Retention Priority in the PCC Rule is derived by the PCF from AF or UDR interaction if available, in line with operator policy.

************** Next change ***************

## 6.x	SMF selection related policy information

To enable the enforcement in the 5GC system of the SMF selection related policy decisions made by the PCF for the DNN replacement, the 5GC system may provide the SMF selection related policy control information from the PCF to the AMF.

Table 6.x-1 lists the PCF SMF selection related policy information.

Table 6.x-1: SMF Selection related policy control information

| Information name | Description | Category | PCF permitted to modify in a UE context in the AMF | Scope |
| --- | --- | --- | --- | --- |
| SMF selection rules | This part defiens the SMF selection rules |  |  |  |
| Reject unsupported UE requested DNN | Defines if a UE requested unsupported DNN shall be rejected | Conditional | No |  |
| List of DNNs | Defines UE requested DNNs applicable for replacement by PCF | Conditional | No | UE context |

The Reject unsupported UE requested DNN defines the rules in AMF for rejection or replacement of unsupported DNN.

The List of DNNs defines the rules for UE requested DNN(s) to be replaced, as described in TS 23.501 [2], clause 5.6.1.

************** End of changes ***************